3GPP TSG-RAN WG2 Meeting #127bis		R2-2408705

Hefei, China, Oct 14th – 18th, 2024

Agenda Item: 	8.1.2.2

Source:		Sony

Title:	Some aspects for model inference and model monitoring

Document for:	Discussion & Decision

# Introduction

In this contribution we provide our views on inference contents for beam management. We also discuss the quality of input data for inference and the resource optimization for model monitoring.

# Discussions

## 2.1 Beam management inference contents

For the content in the reporting of inference results, following agreement was made in RAN1#116:

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

Considering the beam information, it is essential to include at least the predicted beam index or beam ID along with measurement results. For the UE-side model, when reporting the predicted beam index/ID, it is also crucial to investigate how to report the predicted beams that are not in Set B. Regarding option 3 and option 4, providing the probability information of predicted Top K beams among a set of beams can assist the NW in selecting the optimal beam(s) based on the UE reporting. For option 4 a clear definition of the confidence information associated with the RSRP is needed.

Option 2 seems to be aligned with the measurement model as specified in 38.300 and figure 1 below whereby both number of good beams along with measurement results and overall evaluation of reporting criteria is provided. This model includes neighbour cell measurements whereas beam measurements are limited to serving cell only:

Figure 1: measurement model from 38.300

Observation 1: Option 2 seems to have a good overlap with the measurement framework for mobility; except for the difference that mobility involves multiple cells including serving cell whereas beam management concerns the serving cell only. The main difference between options 3 and 4 is the definition of probability and confidence, which RAN1 should address.

Proposal 1: RAN2 to assume that option 2 is the baseline and wait for further progress from RAN1.

Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

## 2.2 Quality of collected data

gNB will configure a set of beams for UE to measure to collect input data. Hence, Set C [1] has been introduced. Set B as the model input can be chosen from Set C. We understand that model input data can be filtered based on specific rules, such as ensuring that the RSRP obtained through measurement is higher than a predefined threshold.

However, a challenge arises when there may not be enough high-quality input data available for the model. This can lead to two possible outcomes: firstly, inputting inadequate data into the model, resulting in a decline in predicted accuracy. If the predicted accuracy decreases significantly, the predicted beams are unsuitable for link communication. Alternatively, even if there is a decline in predicted accuracy, the results may still satisfy the link quality requirements. No matter whichever is the outcome, the gNB needs to understand the reason of the decline in model performance.

Hence, when the UE does not collect enough high-quality data for Set B from Set C, it should report to the gNB. Subsequently, the gNB can configure a new set for collecting input data. If the UE still cannot obtain high-quality data in the new set, it indicates that the scenario has changed and is no longer applicable for the current serving model. Model switching or fallback can be considered afterwards. We think this should in general apply to all use cases.

Proposal 2: At the inference stage, the UE report that the quality of input data has gone bad, to the gNB when the measurement results fail to meet the model's input requirements (applies to both beam management and positioning).

## 2.3 model monitoring resource optimization

For BM-Case 2, performing model monitoring needs to measure the full resource set configured by NW which results in significant measurement overhead and latency. Especially, in the case where the performance metric is calculated for multiple time instances to avoid the "ping-pong" effect. To reduce this overhead, we propose reusing part of the input data from the previous inference instance for each new inference instance. This approach can be illustrated as following picture: for each inference instance, the model's input is based on measurements from five consecutive slots and predict for future four consecutive slots. In the first inference instance, the input data is derived from the measurements of slots one through five. In the second inference instance, the input data is collected from two parts: the new measurement from slot six and the reused measurement data from slots two through five, which were measured in the first inference instance.

This approach doesn’t need NW to configure the full set of resources according to the model's input requirements. Instead, only a portion of the resources needs to be configured, while the other part of the input data is derived from the measurements obtained in the previous instance. This approach can significantly reduce the measurement overhead.

In this figure, the candidate beam in slot eight has been predicted in multiple inference instances. If the predicted results in each instance are the same or similar, this indicates that the model's performance is stable and suitable for the current scenario. Conversely, if the predicted results vary significantly, it may suggest that the model is failing to infer accurately.

Proposal 3: For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

# Summary

In this contribution, we have the following proposal and observations for RAN2 to agree:

Observation 1: Option 2 seems to have a good overlap with the measurement framework for mobility; except for the difference that mobility involves multiple cells including serving cell whereas beam management concerns the serving cell only. The main difference between options 3 and 4 is the definition of probability and confidence, which RAN1 should address.

Proposal 1: RAN2 to assume that option 2 is the baseline and wait for further progress from RAN1.

Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams.

Proposal 2: At the inference stage, the UE report that the quality of input data has gone bad, to the gNB when the measurement results fail to meet the model's input requirements (applies to both beam management and positioning).

Proposal 3: For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

# References

- RP-240774: AI/ML for NR air interface

- R2-240xxxx: Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)