3GPP TSG-RAN WG2 Meeting #127bis	R2-2408706

Hefei, China, October 14th – 18th , 2024

Agenda Item: 	8.2.2

Source:		Sony

Title:			Considerations on functionality aspects for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version of the SID for Ambient IoT in NR was approved including the following objectives [1].

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies. -

At RAN2#126 [2], the following agreements were made related to functionality aspects:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

At RAN2#127 [4], the following agreements were made;

Segmentation:

Agreements

-	Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size.

Message Size/Status Indication

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

Visibility of AIoT Information

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIoT interface as upper layer data.

In this contribution, we discuss our view on some functionality related aspects such as status indication, segmentation and visibility of inventory/command information.

# Discussions on Functionality aspects

What functionality that is required for A-IoT devices are being discussed, and some agreements have been made on functionality that is not going to be supported, but some aspects need to be further discussed

At RAN2#127 it was agreed to study energy status report and to study the use of simple message size report to the reader.

It would be beneficial for the A-IoT device to be able to indicate that the currently stored energy is not sufficient to transmit the data available for transmission.

Based on this report a number of different methods or alternatives can be foreseen.

- a) indicate low energy,

- b) indicate low energy + more data/message size,

- c) indicate low energy/charge rate and suggest a segmentation and transmission scheme for remaining data.

Proposal 1: Further discuss the alternatives for energy indication, message size indication and segmentation and transmission scheme.

## Energy indication

The simplest way forward is to let the A-IoT device be able to just indicate that it is low on energy. This could be done by using only one bit, and it would be up to the reader to interpret this indication, but one example would be that the A-IoT may not be able to complete its transmission or that it may have to charge before commencing a subsequent transmission.

Proposal 2: Let the A-IoT device be able to just indicate that it is low on energy.

## Message size

If the A-IoT device gets low on energy, it may be beneficial for the reader to understand not only that the device is low on energy, but also how much remaining data is available. At least the UE could indicate that more data is available for transmission, but not possible with current energy level and not until charging has been done.

Proposal 3: The UE can indicate that there is a remining data and its amount.

## Segmentation and transmission scheme

As a further improvement or additional information, the device could be able to not only indicate remaining data, but also suggest a segmentation and/or transmission scheme.

The reason for segmentation could also be relevant in case the data to be transmitted is too large to fit in e.g., a TBS, transport Block size defined for msg 1 or msg 3 transmission, and hence a subsequent transmission may be needed.

RAN2 has sent an LS to RAN1 and SA2 [5] asking about the message size but irrespectively, segmentation should be possible for dividing the data into suitable packet sizes for transmission.

Proposal 4: Support segmentation of buffered data.

Segmentation could be used in case

- the device is low on energy, but still has enough energy to transmit at least a part of the data, or

- the device has limited energy storage, and the entire data information cannot be transmitted in one message.

Then the device could indicate that it is transmitting a smaller packet size, and some sort of transmission or energy charging scheme for when the remaining part of the data can be included.

Proposal 5: Support segmentation and energy charging scheme for remaining data.

## Visibility of inventory/command information at the AS layer

Some agreements were made related to visibility, such as RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIoT interface as upper layer data. The question is what upper layer means and whether it means that the information is visible for the reader or not.

As per SA2 solution 3 [3], the proposal is “Whether the Reader needs to perform an Inventory before sending the command to the AIoT devices is a RAN2 decision”. Even if this has been left to RAN2 decision, it is not clear if the information is needed on the AS layer between the Reader and the A-IoT device.

Some alignment with SA2 and maybe RAN3 may be needed to clarify on what protocol level and endpoints the inventory/command signals are transmitted and which protocol should be aware of certain information.

Proposal 6: Further discuss protocol and message structure for visibility.

The reader could benefit from knowing the number of devices that are intended to respond. This information could be used for e.g. subgrouping, in order to reduce the number of devices to respond at the same time, to minimize collisions, or to understand whether e-g., the responses from all target devices need to be accumulated before responding to the Core Network or upper layer application.

Proposal 7: The reader may benefit from having information on type of trigger i.e., command or inventory, but also e.g., the number of devices that are intended to receive trigger, for subgrouping purpose.-

# Summary

In this contribution, we have discussed our views on aspects related to, random access, procedures.

Proposal 1: Further discuss the alternatives for Energy indication, Message size indication and segmentation and transmission scheme.

Proposal 2: Let the A-IoT device be able to just indicate that it is low on energy.

Proposal 3: The UE can indicate remining amount of data.

Proposal 4: Support segmentation of buffered data.

Proposal 5: Support segmentation and energy charging scheme for remaining data.

Proposal 6: Further discuss protocol and message structure for visibility.

Proposal 7: The reader may benefit from having information on type of trigger i.e., command or inventory, but also e.g., the number of devices that are intended to receive trigger, for subgrouping purpose.

# References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- Minutes RAN2#126

- 23.700-13-v040

- RAN2#127

- LS to SA2 and RAN1