3GPP TSG-RAN WG2 Meeting #127bis	R2-2408707

Hefei, China, October 14th – 18th , 2024

Agenda Item: 	8.2.3

Source:		Sony

Title:			Considerations on paging for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version of the SID for Ambient IoT in NR was approved including the following objectives [1].

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies. -

At RAN2#125bis [5], the following agreements were made related to paging:

:

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

At RAN2#126 meeting [6], the following agreements were made related to paging:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

At RAN2#127 [7], the following agreements were made related to paging;

Agreements:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In this contribution, we discuss our view on some RAN2 related aspects related to paging procedure.

# Discussions on paging/trigger for Ambient IoT

In legacy cellular systems, for the Network to reach the UE normally paging is used, for which a UE-id is needed. The UE-id is not only used for identification, but also to group the UEs in different wake-up groups for the system to page the UE without having to wake-up all UEs and for the UEs to save power, not having to listen for paging continuously.

In the case of Ambient IoT, the traffic assumptions have been defined as Device Terminated (DT), and Device Originated (DO). DO traffic only considers DO-DTT, Device Originated, Device Terminated Trigger, which means that the network wakes up the device but for expected or required uplink traffic. This could e.g., be sensor data that should be reported.

It may be relevant to also discuss the naming for the so-called paging or trigger message. In order to avoid misunderstanding.

Proposal 1: Use the term “Trigger” instead of the legacy term “Paging”.

Type of trigger message:

Except for the initial registration procedure, two types of functionalities should be supported [3],

- Inventory

- Command

For example, when a service request is sent from e.g., Application Function to the Core Network (CN), the CN can trigger the reader(s) to make an inventory of a group of Ambient IoT devices. The reader then broadcasts a trigger message containing some sort of identification triggering all, a subset or group of devices or even a single device to respond.

Observation 1: The trigger message needs to contain information of what type of trigger message it is, (inventory or command).

Identification:

What devices that should be triggered should be contained in the trigger message, though the size of the trigger message may need further discussion, e.g., whether the list of devices should be similar to a paging record, whether the device id can be shorter, and whether a type of device or group id be used as an identifier.

In RAN TR 38.848 [2], the RAN design targets are defined, and for A-IoT devices, the maximum message size to be received or to be transmitted is approximately 1000 bits: This is the upper limit for number of bits to be used and can be a baseline for defining how many bits are preferred by RAN2 to be used for subgrouping.

At latest RAN2 meeting [6], it was agreed to further study multiple alternatives on how many devices that can be addressed in the trigger message.

•	a message containing an ID of a single A-IoT device.

•	a message containing a group ID that maps to multiple A-IoT devices.

•	a message that does not contain an ID, i.e., addressed for all devices that can receive the A-IoT message.

•	a message containing multiple IDs of A-IoT devices.  Need to confirm the need for this use case based on SA2 discussion.

For the case of addressing only a single A-IoT device, the size of the ID could be larger, since only a single ID is included in the trigger message.

For the case of addressing a group of A-IoT devices, the size of the ID could be the same as if addressing only a single device, though it is intended for a group.

For the case of addressing multiple individual devices, the size of the ID may have to be shorter to fit several different Ids in one trigger message.

The simplest case is addressing all devices, in which case no Id would be needed.

Observation 2: The message needs to contain identification, and the size of the ID depends on the number of IDs that is to be included in a single trigger message.

Proposal 2: Further discuss the size and maximum number of Device IDs in a single trigger message.

Configuration for UL transmission:

Furthermore, unless the Ambient IoT devices have received some previous configuration, or have been pre-configured, some information about resources to be used for responding to the trigger needs to be conveyed. This could also include the type of random access procedure to be used  e.g., 4-step or 2-step like random access.

Configuration of resources could be general and explicit in the case of Contention Based Random Access (CBRA), i.e., common resources will be given to all devices, e.g., via system information.

For any UE specific configuration, the configuration could be part of the first trigger message, or as a subsequent message/transmission, e.g., multiple command massages could be transmitted.

Proposal 3: Further discuss how to convey uplink configuration for performing Random Access like procedure.

For the case of Contention Free Random Access (CFRA), the device may have to be configured with resources to use. This could be done either by explicitly appending resource configuration in the trigger message, or by implicitly indicating some preconfigured resources to be used.

Proposal 4: Further study the alternatives of explicit and implicit resource configuration.

DRX and Subgrouping:

The devices with limited energy storage would have to carefully save energy and may benefit from having the possibility to not constantly have to be able to react to a trigger message. E.g., when the device is out of energy or is performing harvesting, the device is either not able to respond, or occupied with energy harvesting.

It was discussed and agreed in the earlier RAN2#125bis meeting to not support legacy paging, but not concluded on monitoring occasions for trigger message.

Some sort of subgrouping based on the identity of the device, or potentially some other grouping mechanism, may  be relevant to avoid triggering all or any device as discussed in the section above.

At least device types with energy storage labeled as “ii” under bullet A in the SID, (or device 2a/2b [4]) above can be considered for supporting duty-cycled channel monitoring similar to DRX functionality.

Proposal 5: For triggering, further discuss the need for device identification, subgrouping and duty-cycled channel monitoring.

Multiple Readers scenario for Paging messages:

It has been agreed that RRC connection management and periodical system information (SIB and MIB) are not supported by A-IoT devices. This may mean that a device is not connected to a specific cell. Hence, as discussed in [8], there are issues on the multiple readers scenario for paging messages where a device is between multiple close-by Readers as shown on Figure 1. If we assume that Core Network (CN) initiates the paging message via gNB/Reader for the Device or a group of Devices, it is likely that paging messages may be sent to multiple close-by readers as CN does not know the exact location of a specific device. Note that there are cases where Device IDs may not be included, for example for the case of “Inventory” which is intended for everyone.

Therefore, RAN2 should discuss definition of the device behavior when the device receives more than one triggering message in a period from different readers. For example, paging/reader ID included in the triggering message can indicate that the triggering message is coming from a different Reader (i.e., paging ID is reader specific).

Figure 1. A-IoT device locating in between Readers.

Proposal 6: For paging, RAN2 to discuss definition of the device behaviour when the device receives more than one triggering paging message in a period from different readers.

At RAN2#127, it was agreed:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

Based on the above agreement, if a device receives initial paging message from a specific reader (i.e., request from the CN), the device should be able to receive subsequent paging message after a period (let’s say after one access period) from the same reader (including if the initial response from device was not successful), noting that there are a number of access occasions following paging message. But there may be a case where there is a paging cycle or a radio frame which comprises a number of access occasions. Hence, a trigger message can be repeated after each radio frame from the same reader or a different reader, where the trigger message is for the same or different request from the CN. In order to reduce the number of trigger messages from different readers, it can be assumed that a device can receive one trigger message in a period of a radio frame from one or more readers.

Proposal 7: RAN2 should assume a device can receive one trigger message in a period (e.g., a period of a radio frame), from one or more readers, including triggering multiple/subsequent paging messages.

# Summary

In this contribution, we have discussed our views on aspects related to paging.

Proposal 1: Use the term “Trigger” instead of the legacy term “Paging”.

Observation 1: The trigger message needs to contain information of what type of trigger message it is, (inventory or command).

Observation 2: The message needs to contain identification, and the size of the ID depends on the number of ID´s that is to be included in a single trigger message.

Proposal 2: Further discuss the size and maximum number of Device IDs in a single trigger message,

Proposal 3: Further discuss how to convey uplink configuration for performing Random Access like procedure.

Proposal 4: Further study the alternatives of explicit and implicit resource configuration.

Proposal 5: For triggering, further discuss the need for device identification, subgrouping and duty-cycled channel monitoring.

Proposal 6: For paging, RAN2 to discuss definition of the device behaviour when the device receives more than one triggering paging message in a period from different readers.

Proposal 7: RAN2 should assume a device can receive one trigger message in a period (e.g., a period of radio frame), from one or more readers, including triggering multiple/subsequent paging messages.

# References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- TR 38.848, Study on Ambient IoT (Internet of Things) in RAN (Release 18)

- TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)

- RAN1#116, Chair's notes RAN1#116 9.4 R19 Ambient IoT v07

- Minutes RAN2#125bis

- Minutes RAN2#126

- Minutes RAN2#127

- R2-2405603, “Discussion on initial trigger message (paging-like message) on Ambient IoT”, RAN2#126