3GPP TSG-RAN WG2 Meeting #127bis	R2-2408709

Hefei, China, October 14th – 18th , 2024

Agenda Item: 	8.4.2

Source:		Sony

Title:			RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version for the WID for Low Power Wake-up Signal and Receiver was approved including the following objectives [1].

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline Specify necessary RRM requirements-

At RAN2#125bis [3], the following agreements were made related to RRC Idle/Inactive mode.

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

Working assumption: the LP-WUS configuration in SIB at least includes the following information:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	FFS on Entry/exit condition for LP-WUS monitoring

The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

At RAN2#126 [4], the following agreements were made related to RRC Idle/Inactive mode;

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

    - LP-SS configuration

    - LP-WUS configuration

    - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8

From RAN2 perspective, no new procedure is introduced for SI reception/updates.

At RAN2#127 [5], the following agreements were made;

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

No agreements related to subgrouping was made.

In this contribution, we discuss our view on some RAN2 aspects related to configuration of LP-WUS triggered paging monitoring, including subgrouping, entry/exit criteria and latency aspects.

# Discussions on RAN2 aspects for LP-WUS/WUR in RRC Idle/Inactive

In RRC Idle/Inactive UE power saving gain comes from that the UE can operate with DRX operation, i.e., only periodically monitoring for paging, System update and performing cell measurements. Still the monitoring using the Main Radio, (MR), consumes a non-negligible power consumption, especially for small devices, that requires rather short wake-up times. For this purpose, the use of a low power wake-up radio has been solution to reduce power consumption even further.

Introducing such a low power wake-up radio (LP-WUR), requires that it is reachable from the network, i.e. it can react to paging or some wake-up signal. Furthermore, in order to not create false wake-up, some sort of subgrouping is needed, as well as a concept of whether the UE is in capacity to operate using LP-WUR or not.

## Entry/exit conditions

Baseline agreements for entry condition is when serving cell quality e.g., RSRP, RSRQ based on Main Radio measurements are above a certain threshold, the UE may start to monitor LP-WUS, and may stop monitor Paging Occasions (PO).

Baseline agreements for exit condition is when serving cell quality based on Low Power Wake-up Radio is below a certain threshold, the UE stops monitoring LP-WUS, and starts to monitor legacy PO.

At RAN1#117 [6], RAN1 made the following working assumptions related to Entry and Exit conditions.

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

RAN1 leaves some open question that would be relevant for RAN2 to discuss

- Are there any other conditions to be met for the UE to start LP-WUS monitoring?

- Are there any other conditions to be met for the UE to stop LP-WUS monitoring?

- Are there any serving cell measurement metrics still pending?

- Are there alternatives on how to configure entry/exit condition thresholds?

Proposal 1: Discuss and conclude on the following open issues from RAN1:

- Are there any other conditions to be met for the UE to start LP-WUS monitoring?

- Are there any other conditions to be met for the UE to stop LP-WUS monitoring?

- Are there any serving cell measurement metrics still pending?

- Are there alternatives on how to configure entry/exit condition thresholds?

For the partial coverage case, UE’s MR could stay in ultra-deep sleep power state only when UE is in the coverage of LP-WUS. When UE moves out of the coverage of LP-WUS, the UE should start up its MR.

But there may be different situations. Both the case where the LP-WUS coverage is partial already by configuration and cell planning, but also the case where the intention is to have full LP-WUS coverage in the cell, but due to fading the LP-WUR cannot detect the LP-WUS, and hence it should be considered being out of coverage.

Observation 1: There could be a case where the LP-WUS is defined to have full cell coverage, still the LP-WUR (LR) may not be able to detect the LP-WUS.

In order to evaluate whether the LR is within LP-WUS coverage or not, additional measurements may be needed. E.g. there could be a situation where the LR suddenly becomes out of coverage, even if being in the cell-center due to fading or channel conditions.

The MR could be used temporarily just to verify whether also the MR is out of coverage or its only the LP-WUR, that is not able to suddenly measure the serving cell.

If the MR experience good coverage, it could verify that also LR has good coverage before MR then as soon as possible resumes sleep mode and let LR continue measurements and monitor LP-WUS.

Proposal 2: It should be possible to use the MR to verify whether LR is actually out of coverage or was just temporarily not able to satisfactory monitor for LP-WUS due to fast variations in channel condition, to avoid ping-pong behavior between MR and LR.

## Subgrouping

In earlier release starting form Rel-15, subgrouping have been defined for both LTE, NB-IoT, and NR (Rel-17). The concept of subgrouping may also be applicable for Rel-18/19 WUS/LP-WUR.

After waking up from LP-WUS, in case UE needs to monitor PO, the time offset between LP-WUS and the PO should be long enough to include the transition time to start up the main receiver to be ready for PDCCH monitoring, either all UEs could be triggered to wake up to monitor paging in associated PO(s), or just a subset of the UEs sharing the PO, so called LP-WUS UE subgrouping. The purpose of sub-grouping is to reduce the number of false wake-up of UEs that the paging was not intended for. [2]

Furthermore, the study clarifies that the LP-WUS UE subgrouping information corresponding to 2 bits or more. How to indicate the subgrouping information, e.g., included in the LP-WUS payload or from the use of multiple LP-WUS signals corresponding to different subgroups, etc., depends on the design of LP-WUS. This is up to RAN1 to define.

What RAN2 can discuss and propose is what type of subgrouping to be defined and how many bits that is reasonably for different scenarios, e.g., coverage area, max number of UEs per cell, paging rate etc.

At earlierRAN2 meeting [3] it was agreed that PEI subgrouping is to be taken as baseline for LP-WUS subgrouping. It is, however, still FFS what should be the maximum number of subgroups.

At last RAN2 meeting [4] it was agreed that at least 8 subgroups should be supported. Though we think the number of subgroups should be configurable and depending on scenario and considering the size of the wake-up message. And maybe 16 subgroups is a reasonable number.

In some scenarios the devices may not be addressed by specific ID, but only with group id. E.g. if all devices related to a certain category should be woken up, like sensors of different types, or even more if tracking of different products stored in a warehouse. For these cases, maybe hundreds or thousands of different groups may be needed.

Proposal 2: Discuss the number of subgroups that is preferred to be supported and number of bits needed to support it based on UE-Identity, device type, or type of functionality.

Proposal 3: Discuss how coverage area, max number of UEs per cell, paging rate etc. could affect number of bits to be carried in LP-WUS for subgrouping.

## Latency (duty-cycled vs always on)

When operating based on a duty-cycled scheme, the average communication delay from when data is available at the gNB until paging/data is correctly received by the UE depends on the sleep duration in the duty-cycle length, contributing error events such as false-alarm or miss-detection, and the transition time before the main receiver is ready for PDCCH reception.

The average communication delay in the LP-WUS/WUR scheme would be longer than the baseline scheme, i.e., Rel. 15/Rel. 16 scheme, if both are configured with the same duty-cycle length to fulfill a certain delay requirement. This is since the main radio goes to ultra-deep sleep state to further save power and therefore a longer transition time is needed to wake-up the main radio from ultra-deep sleep. In certain scenarios with extremely tight delay requirements, the communication delay time can become very significant and even the shortest DRX cycle can prevent the UE from meeting the tight delay requirement. This can happen, for instance, if the communication delay is required to be shorter than the length of “existing shortest possible” DRX configuration (320 msec) + the main radio transition and synchronization time. In these types of scenarios, the LP-WUR can instead be configured to monitor the channel continuously.

To address this, the operation of the LP-WUR can be configured as an adaptive operation where the LP-WUR can be adaptively operated according to a duty-cycled or an always-on scheme based on the communication delay requirement.

Observation 4: The long transition time to wake-up the main radio from ultra-deep sleep state together with the sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement.

In the study [2], it was agreed that at least duty-cycled monitoring of LP-WUS is supported, but depending on use case and capabilities it should be possible to configure either always on or duty cycled monitoring.

Proposal 6: Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on either always-on or a duty-cycle scheme.

To give more freedom for configuring the LP-WUS monitoring, and for subsequent action after detecting the LP-WUS, the LP-WUR could be configured with a DRX cycle that is totally independent from the legacy Paging DRX cycle that the Main Radion (MR) is configured with. This should also make it possible to trigger other activities after detection the LP-WUS, e.g., to data transmission/reception or initiating RACH procedure directly.

Proposal 7: Support a configurable operation of duty-cycled LP-WUS monitoring to be done with respect to the existing DRX configuration and/or a stand-alone DRX configuration for LP-WUR.

# Summary

In this contribution, we have discussed our views on aspects related to.

Proposal 1: Discuss and conclude on the following open issues from RAN1:

- Are there any other conditions to be met for the UE to start LP-WUS monitoring?

- Are there any other conditions to be met for the UE to stop LP-WUS monitoring?

- Are there any serving cell measurement metrics still pending?

- Are there alternatives on how to configure entry/exit condition thresholds?

Observation 1: There could be a case where the LP-WUS is defined to have full cell coverage, still the LP-WUR (LR) may not be able to detect the LP-WUS.

Proposal 2: It should be possible to use the MR to verify whether LR is actually out of coverage or was just temporarily not able to satisfactory monitor for LP-WUS due to fast variations in channel condition, to avoid ping-pong behavior between MR and LR.

Proposal 2: Discuss the number of subgroups that is preferred to be supported and number of bits needed to support it based on UE-Identity, device type, or type of functionality.

Proposal 3: Discuss how coverage area, max number of UEs per cell, paging rate etc. could affect number of bits to be carried in LP-WUS for subgrouping.

Observation 4: The long transition time to wake-up the main radio from ultra-deep sleep state together with the sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement.

Proposal 6: Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on either always-on or a duty-cycle scheme.

Proposal 7: Support a configurable operation of duty-cycled LP-WUS monitoring to be done with respect to the existing DRX configuration and/or a stand-alone DRX configuration for LP-WUR.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TR 38.869, Study on low-power wake up signal and receiver for NR (Release 18

- Minutes RAN2#125bis, Changsha

- Minutes RAN2#126, Fukuoka

- Minutes RAN2#127

- Minutes RA1#116, Fukuoka