3GPP TSG-RAN WG2 Meeting #127bis  	                            R2-2408712

Hefei, China, Oct 14th – 18th, 2024							                (Revision of R2-2406620)

Agenda Item:	8.5.2

Source: 	Sony

Title:	On-demand SSB Scell operation discussion

Document for:	Discussion

# Introduction

In RAN2#127 meeting, on-demand SSB Scell operation was discussed, and the agreements are as follows.

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this contribution, we discuss some open issues regarding the UE triggered OD-SSB, MAC CE for OD-SSB transmission indication,

# Discussion

2.1 Uplink wake-up-signal from UE

This wake-up-signal is sent from UE to network, once certain conditions are met, in order to trigger the network to activate/deactivate on-demand SSB operations in SCell. It is up to network to decide whether to activate/deactivate the on-demand SSB transmission in SCell, based on the received wake-up-signal from UEs within the cell.

UE triggered on-demand SSB has potential benefits as following:

- The uplink wake-up-signal from UEs can give network a real-time overview of upcoming short burst traffic demand, if the wake-up-signal is triggered when UE has additional (sometimes unexpected) UL traffic to transmit. Therefore, the network can activate the Scell on time just before the traffic arrives which potentially maximizes the energy saving gain without sacrificing the UL traffic transmission performance of the UE.

- The uplink wake-up-signal from UE can indicate to the network its demand to perform connected mode measurement, so that network can activate the cell to facilitate such requirement on demand.

- Uplink wake-up-signal may have added benefit to allow the network to be aware of the uplink channel quality of the UE based on the wake-up-signal.

Observation 1: Uplink wake-up-signal from UE to trigger the on-demand SSB in SCell is beneficial from RAN2 point of view. But we wait for RAN1 input.

The discussions regarding the uplink wake-up-signal design based on the existing channel/signal, trigger conditions as well as the reception node of the uplink wake-up-signal should be led by RAN1.

2.2 Cell on/off indication via backhaul

RAN1#116 agreed to support on-demand SSB SCell operation triggered by gNB and cell on/off indication via backhaul is one of the candidate solutions. With the existing CU-DU architecture, it is the CU who is responsible for the cell on/off decision. In gNB-CU configuration update message sent from gNB-CU to gNB-DU, it contains cells to be activated/deactivated list which gNB-DU shall activate/deactivate the corresponding cells as indicated by the cell group and physical cell identity.

With the existing signalling, it is not possible to support dynamic cell on/off use cases where the cell on/off decision is made based on the current traffic and energy consumption status of the cell. It is the gNB-DU who is aware of the up-to-date status of the cells but the cell on/off indication comes from gNB-CU.

Cell on/off indication via backhaul will be a promising solution if it can be enhanced to support:

- Assistance information from gNB-DU to gNB-CU to help gNB-CU making cell on/off decision. We think this information is needed because gNB-DU has more awareness and gNB-DU should be involved in cell on/off decision as explained below.

- Certain level of autonomy is given to gNB-DU to allow it make its own cell on/off decision according to the cell status. The gNB-DU has the up-to-date information e.g., number of active UEs, radio resource utilization ratio, traffic status of the cell. Therefore, if certain level of autonomy is given to gNB-DU, it can make proper cell on/off decision based on the above cell status. But gNB-CU should always control this procedure and being informed of any gNB-DU’s decision.

RAN3 should lead the work and it may have some impacts on the assistance information which can be potentially supported by the UEs in the cell.

Proposal 1: Enhancements to support dynamic cell on/off indication via backhaul should be supported and RAN3 should be informed about it.

2.3 SCell activation/deactivation signalling

RAN2 agreed that a new MAC-CE for OD-SSB transmission indication will be introduced and the legacy SCell activation/deactivation MAC CE won’t be changed. There was discussion that whether we consider to further optimise the MAC-CE for scenario 2A where OD-SSB can be activated with the Scell activation, so the new MAC CE can indicate joint OD-SSB transmission and Scell activation. We think the joint indication is not necessary as follows.

- Joint OD-SSB transmission and Scell activation/deactivation will make the MAC-CE design more complicated, as the MAC-CE size will be variable to cover both the case of OD-SSB transmission indication and joint OD-SSB, Scell activation/deactivation.

- Without the joint OD-SSB and Scell activation/deactivation, in scenario 2A, network needs to transmit both OD-SSB transmission indication and Scell activation/deactivation MAC-CE. But from the signalling overhead point of view, there is no difference whether we have a joint MAC-CE or not.

- RAN2 has agreed to introduce a new MAC-CE for OD-SSB transmission indication, there is no strong motivation to introduce separate MAC-CE for different scenarios.

Proposal 2: A new MAC-CE to indicate only OD-SSB transmission is introduced.

# Conclusion

We have observation as following:

Observation 1: Uplink wake-up-signal from UE to trigger the on-demand SSB in SCell is beneficial from RAN2 point of view. But we wait for RAN1 input.

We propose RAN2 to discuss following proposals:

Proposal 1: Enhancements to support dynamic cell on/off indication via backhaul should be supported and RAN3 should be informed about it.

Proposal 2: A new MAC-CE to indicate only OD-SSB transmission is introduced.