3GPP TSG-RAN WG2 Meeting #127bis  	                            R2-2408719

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.8.2

Source: 	Sony

Title:	SMTC impacts due to NTN downlink coverage enhancements

Document for:	Discussion

# Introduction

In RAN2#127 meeting, the impacts of beam hopping/extended SSB periodicity to RAN2 has been discussed and the corresponding agreements are as follows.

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In this contribution, we mainly discuss the SMTC impacts due to beam hopping/extended SSB periodicity.

# Discussion

According to RAN1#118 meeting, RAN1 considers the extended SSB periodicity by NTN UE for initial access can be one or more values from the list {40ms, 80ms, 160ms, 320ms, 640ms} in the system level and link level evaluations. In NTN, the network can configure up to 4 SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities. But considering the typical cell/beam deployment around the serving cell to guarantee the coverage, together with the extended SSB periodicity up to 640ms, the number of parallel SMTCs may need to be further extended due to NTN downlink coverage enhancement i.e. beam hopping/extended SSB periodicity.

In NTN, for RRC_CONNECTED UEs, network controlled adjustment of SMTCs can be based on the reported UE assistance information. The UE assistance information includes the service link propagation delay difference between the serving cell and the neighbour cells. For RRC_IDLE/RRC_INACTIVE UEs, the UE can adjust SMTCs based on its location and assistance information in SIB19 and the assistance information is based on the broadcasted reference location which is typically e.g, serving cell’s reference location.

With only the serving cell’s reference location and a threshold to that reference location, is not enough for UE to perform the proper the SMTC measurement selection, considering the potentially extended number of SMTCs configured to the UE. Furthermore, for the RRC_CONNECTED UEs, the propagation delay difference is not accurate enough either for network to configure the proper SMTCs for the UEs. Therefore, we propose to enhance the SMTC measurement adjustment, for both UEs in RRC_CONNECTED and RRC_IDLE/INACTIVE. The signalling details for additional assistance information e.g. additional reference location information from network, finer UE location information from UE if available and requested should be further discussed.

Proposal 1: UE autonomous SMTC measurement adjustment for UEs in RRC_IDLE/INACTIVE, and network controlled SMTC measurement adjustment for UEs in RRC_CONNECTED should be further enhanced, due to NTN downlink coverage enhancement.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: UE autonomous SMTC measurement adjustment for UEs in RRC_IDLE/INACTIVE, and network controlled SMTC measurement adjustment for UEs in RRC_CONNECTED should be further enhanced, due to NTN downlink coverage enhancement.