3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408727

Hefei, China, 14th – 18th October, 2024

Agenda item:	8.7.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on LCP enhancements

Document for:	Discussion and Decision

# 1. Introduction

In RAN#105, the study of scheduling enhancements using delay/deadline information was formalized into the WI objective as follow [1]:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

For the additional priority handling in the WI scope, RAN2 discussed the solution and had the following agreements:

- RAN2#126: For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). - RAN2#127: FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In this contribution, we will further discuss the additional priority handling as updated in the WI scope.

# 2.	Discussion

## 2.1	Additional priority handling

In existing LCP procedure during resource allocation for new transmission, resources are allocated based on logical channel priority of the selected logical channels in two phases: first one when resources are initially allocated (which we will refer to as Initial resource allocation) and another when there are remaining resources to be allocated (which we will refer to as Remaining resource allocation).

##### 5.4.3.1.3 Allocation of resources 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); [Initial resource allocation] 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. [Remaining resource allocation]

UE performs resource allocation at an LCH granularity. For resource allocation, RAN2#126 agreed that “LCP prioritization within a logical channel will not be considered in RAN2 discussions” which means that the data within the logical channel is not reshuffled or priority is not changed based on whether it is delay-critical or not, while performing resource allocation during Initial resource allocation or Remaining resource allocation However, we think that LCH priority can change before Remaining resource allocation during LCP resource allocation (i.e. it can be different than the LCH priority at Initial resource allocation). The reason is that if the additional priority is still used when there is no delay-critical data, it would negatively impact other LCHs without delay-critical data whose priority level is normally higher than default priority of the LCH for which the priority is adjusted. For example, default priority of LCH1 is 5, default priority of LCH2 is 3, UE adjusts the priority of LCH1 which has delay-critical data to the additional priority, e.g., 2. If additional priority “2” is still used after all the delay-critical data are assembled, e.g., during the remaining resource stage when the non-delay-critical data of LCH1 still uses the additional priority “2” even though all the delay-critical data is already assembled, LCH2 (which has higher priority than default priority of LCH1) would be deprioritized unnecessarily.

Observation 1: If the additional priority is still used for an LCH when there is no delay-critical data on the LCH at Remaining resource stage, it would have negative impact on other LCHs without delay-critical data whose priority level is higher than the default priority of the LCH for which the priority is adjusted.

To generalize, the MAC entity should determine the LCH priority for an LCH before the resource allocation procedure based on whether the LCH contains “delay critical data” at Initial resource allocation and Remaining resource allocation. If there is, the additional priority is used, otherwise, the default priority is used. In addition, considering that the additional priority is used only when there is delay-critical data, it is obvious to fall back to the default priority if there is no delay-critical data in the LCH at deciding on remaining resource.

Proposal 1: The MAC entity should determine the LCH priority based on whether the LCH contains “delay critical data” before each of Initial resource allocation and Remaining resource allocation phases.

With respect to impacts on traffic from SRBs, NW can configure a higher priority for the LCH associated with SRBs. For example, by NW implementation, NW can ensure that the priority for LCH associated with SRBs is higher than that of delay-critical data, even when the priority of the LCH with delay-critical data has been adapted.

Proposal 2: For LCP prioritisation enhancement, if there is a concern on traffic from SRBs delayed due to LCHs with delay-critical data, it can be left to network implementation to ensure LCH of SRBs are always higher than LCHs of delay critical data.

As mentioned in Proposal 1, another question is how to define the “Delay Critical data” in LCP (i.e. how LCP decides to apply the additional priority or default priority for an LCH during LCP procedure), RAN2#126 discussed and have the following for remaining time threshold of LCP enhancement:

FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In existing DSR procedure, the legacy DSR threshold is used for UE to trigger and report the delay status for delay-critical data, based on which gNB can have a better knowledge of the delay information of UE’s pending data and determine its scheduling strategy. However, the delay-aware LCP enhancement is for UE to consider the delay information during data assembly, which is essentially different procedure. Therefore, it is better not to couple the two procedure, and a separate threshold should be used.

Proposal 3: DSR and delay-aware LCP enhancement should not be coupled. A separate remaining time threshold should be used for additional priority handling (i.e. an LCH containing data with remaining time less than the remaining time threshold for LCP should apply additional priority; otherwise default priority is applied for the LCH).

## 2.2	Impact of additional priority handling on Intra-UE prioritization

With respect of additional LCH priority, RAN2#127 discussed the issues of intra-UE prioritization, and have the following FFS:

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

In legacy intra-UE prioritization procedure, for the MAC entity configured with lch-basedPrioritization, the following mechanism is used to determine the priority of an UL grant as defined in 38.321. There is also similar mechanism on deciding on the priority of a SR based on the priority of the logical channel triggering the SR.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

Considering that an LCH may be adjusted to use the additional priority when there is delay-critical data on the LCH, it is more reasonable to use the LCH’s additional priority when deciding the priority of its associated uplink grant when the LCH contains delay critical data. The overlapping cases involve not only the UL-SCH grant, but also the SR resources. Therefore, it is obvious that additional LCH priority should be considered when deciding the priority of SR which is triggered by LCH with delay-critical data, as well as when deciding the priority of an UL-SCH grant which is associated with LCH(s) with delay-critical data. Otherwise, the LCH priority actually used for resource scheduling is different from the LCH priority used when deciding the UL grant and SR priority, which would result in delay critical data being delayed further if its UL grant is not selected.

Moreover, we think that this should be considered for new transmission and retransmission case. And in the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become delay critical data. This should be considered also when determining the priority of the UL grant.

Observation 2: In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become delay critical data.  This should be considered also when determining the priority of UL grant.

Proposal 4: The additional LCH priority should be considered for intra-UE prioritization while determining the priority of UL grant for new transmission, UL grant for retransmission and the priority of SR.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for LCP enhancements. The following observations and proposals were made:

Additional priority handling

Observation 1: If the additional priority is still used for an LCH when there is no delay-critical data on the LCH at Remaining resource stage, it would have negative impact on other LCHs without delay-critical data whose priority level is higher than the default priority of the LCH for which the priority is adjusted.

Proposal 1: The MAC entity should determine the LCH priority based on whether the LCH contains “delay critical data” before each of Initial resource allocation and Remaining resource allocation phases.

Proposal 2: For LCP prioritisation enhancement, if there is a concern on traffic from SRBs delayed due to LCHs with delay-critical data, it can be left to network implementation to ensure LCH of SRBs are always higher than LCHs of delay critical data.

Proposal 3: DSR and delay-aware LCP enhancement should not be coupled. A separate remaining time threshold should be used for additional priority handling (i.e. an LCH containing data with remaining time less than the remaining time threshold for LCP should apply additional priority; otherwise default priority is applied for the LCH).

Impact of additional priority handling on Intra-UE prioritization

Observation 2: In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become delay critical data.  This should be considered also when determining the priority of UL grant.

Proposal 4: The additional LCH priority should be considered for intra-UE prioritization while determining the priority of UL grant for new transmission, UL grant for retransmission and the priority of SR.

# 4. Reference

[1]. RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3