3GPP TSG-RAN WG2 Meeting #127bis	R2-2408728

Hefei, China, 14th – 18th October, 2024

Agenda item:	8.7.4.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on DSR enhancements

Document for:	Discussion and Decision

# 1. Introduction

In RAN#105, the study of scheduling enhancements using delay/deadline information was formalized into the WI objective as follow [1]:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

For DSR enhancements, RAN2 made the following agreements in RAN2#127:

- RAN2#126: - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included - RAN2#127: Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this contribution, we will further discuss DSR enhancement as updated in the WI scope.

# 2.	Discussion

## 2.1	Multiple reporting thresholds for DSR enhancements

In RAN2#126, it is agreed to enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG (i.e. there will be multiple reporting thresholds).

In RAN2#127, it is agreed that there will be a single threshold for triggering the DSR, similar to R18. The following highlighted FFSes have been captured to be further discussed:

For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

In R18 XR, the delay-critical data is defined in 38.323 as below:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

It can be seen that the delay-critical data is determined based on the remaining time of the discardTimer and the remainingTimeThreshold.  remainingTimeThreshold is also the threshold which is used for MAC layer to trigger DSR. In our understanding, the triggering and reporting thresholds are independent to each other. There is no need to change the definition of delay-critical data in R19 because of multiple reporting thresholds.

Proposal 1: There is no need to change the definition of delay-critical data, i.e. as per legacy where it is determined based on the discard timer and remaining time threshold used for DSR triggering.

We further analyze the relation between the DSR triggering and reporting thresholds, under the following cases:

Figure1. Different cases for DSR triggering threshold and reporting thresholds

- Case1 (where the triggering threshold is less than or equal to reporting thresholds)

The triggering threshold is the smallest one among all the configured reporting thresholds.  In this case, the information reported in DSR MAC CE includes the information of delay-critical data same as in legacy, and the information of non-delay-critical data.

For example, as shown in the Figure1 above, in case 1, the triggering threshold is set to reporting threshold 1, which is the smallest one in the multiple thresholds. When DSR is triggered by data#1, in DSR MAC CE, UE reports the information of delay-critical data#1 and the information of non-delay-critical data from data#2 to data#5. To report the non-delay-critical information in DSR MAC CE, it can avoid the case where most of the data becomes delay-critical before UE transmits it, and leaving less time for the gNB to schedule it and causing more DSRs to be triggered.

Observation 1: The DSR triggering threshold less than or equal to the DSR reporting thresholds can provide more information (e.g. non-delay critical data that may become delay critical soon etc.) to the gNB.

- Case2 (where the triggering threshold is always greater or equal to reporting threshold)

The triggering threshold is the largest one among all the configured reporting thresholds. In this case, the information reported in DSR MAC CE includes the information of delay-critical data with finer granularity.

For example, as shown in the Figure1 above, in case 2, the triggering threshold is set to reporting threshold 5, which is the largest one in the multiple thresholds. When DSR is triggered by data#5, in DSR MAC CE, UE reports the information of all the delay-critical data from data#1 to data#5, where the information of data#1 to data#4 may have been reported before. Although this can provide a finer information of the delay-critical data, gNB can estimate the remaining time range of the previous delay-critical data#1 to data#4. Therefore, it may cause signalling overhead if the triggering threshold is the largest one among all the reporting thresholds. However, if there are no network overhead issues, it is still valuable to configure a triggering threshold larger than the multiple thresholds to obtain the finer granularity information about delay-critical data.

Observation 2: Triggering threshold higher than reporting threshold leads to signalling overhead as UE unnecessarily reports information of the previous delay-critical data again whenever DSR is triggered for the LCG. However, it can be left to network implementation to decide whether such overhead is acceptable.

Based on the above analysis, it is more reasonable to leave it up to gNB implementation on how to configure the multiple DSR reporting thresholds and the DSR triggering threshold.

Proposal 2: There is no constraints on how the NW configures DSR triggering and reporting thresholds, the configuration can be left to gNB implementation.

In conclusion, based on the above analysis on the relationship between the multiple reporting thresholds and the triggering threshold, it is obvious that UE should report the data according to the configured reporting thresholds, and for the case where reporting threshold is larger than triggering threshold, the non-delay critical data information should also be reported.

Proposal 3: UE should report the data according to the configured reporting thresholds even when this data is non-delay critical, i.e. in the case reporting threshold is larger than triggering threshold.

## 2.2	Non-delay critical data in enhanced DSR

RAN2#127 discussed the non-delay data related issues for enhanced DSR and agreed the following.

For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

The non-delay critical data ahead of delay critical data in transmission buffer size may consume the resources scheduled for delay-critical data, making the delay-critical data to be discarded. If the transmission of non-delay-critical data has impact on transmission of delay-critical data, but the information is not indicated through any other signaling, e.g., BSR, the gNB may not know the existence of the non-delay-critical data and will not schedule sufficient grant to the UE. The cases where non-delay-critical data has impact on transmission of delay-critical data include the following:

- Case (A): non-delay-critical data from other LCHs configured with higher priority impacts the transmission of delay-critical data. Currently, UE reports the delay-critical data information per LCG if DSR is triggered by an LCH, based on which gNB can schedule UL grant for the delay-critical data. However, if there are other non-delay-critical data on LCH(s) with higher priority, and the priority is higher than that of the LCH which triggered the DSR, the delay-critical data transmission may be impacted by these non-delay-critical data. In this case, the non-delay-critical data information from LCH(s) with higher priority can be reported to gNB through enhanced DSR MAC CE, so that gNB can allocate more UL grant and avoid the delay-critical data being discarded. Furthermore, considering that the DSR enhancement and LCP enhancement are two de-coupled mechanism, i.e., the remaining time threshold for DSR enhancement and LCP enhancement are configured separately, it may not be necessary for UE to support both DSR enhancement and LCP enhancement (i.e. independent UE capability). Therefore, the non-delay-critical data information in Case (1.1) can be reported.

Observation 3: If the information of non-delay-critical data from other LCHs with higher LCH priority (higher than that of the LCH which triggers the DSR) is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data, particularly in the case UE supports DSR enhancement but not LCP additional priority handling.

- Case (B): when PSI-based discard is activated, low importance data which is in front of the delay-critical data impacts the transmission of delay-critical data. Currently, when DSR is triggered and PSI-based discard is activated, the non-delay-critical data information in front of the delay-critical data is not included in DSR MAC CE. However, RAN2 agreed in the previous meeting that intra-LCH scheduling is not in RAN2 scope, this means that data within an LCH is assembled in a first-in and first-out manner. Therefore, there is no doubt that non-delay-critical data in front of the delay-critical data will be assembled firstly and consume some UL grant, under which case the delay-critical data transmission may be impacted and even be discarded when there is not enough UL grant. Therefore, the non-delay-critical data information which is in front of the delay-critical data should be reported to gNB through enhanced DSR MAC CE, so that UE can request more UL grant from gNB and avoid the delay-critical data being discarded.

Observation 4: When PSI-based discard is activated, if the information of non-delay-critical data which is in front of the delay-critical data is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data.

Proposal 4: The non-delay-critical data information which has impact on delay-critical data transmission (i.e. in front of the transmission queue) should also be included for each LCG and this includes:

- Non-delay-critical data information from other LCHs with LCH priority higher than that of the LCH which triggers the DSR;

- When PSI-based discard is activated, the data volume of the low importance PDU sets which is ahead of delay-critical data in the UE buffer.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for DSR enhancements. The following observations and proposals were made:

Multiple reporting thresholds for DSR enhancements:

Proposal 1: There is no need to change the definition of delay-critical data, i.e. as per legacy where it is determined based on the discard timer and remaining time threshold used for DSR triggering.

Observation 1: The DSR triggering threshold less than or equal to the DSR reporting thresholds can provide more information (e.g. non-delay critical data that may become delay critical soon etc.) to the gNB.

Observation 2: Triggering threshold higher than reporting threshold leads to signalling overhead as UE unnecessarily reports information of the previous delay-critical data again whenever DSR is triggered for the LCG. However, it can be left to network implementation to decide whether such overhead is acceptable.

Proposal 2: There is no constraints on how the NW configures DSR triggering and reporting thresholds, the configuration can be left to gNB implementation.

Proposal 3: UE should report the data according to the configured reporting thresholds even when this data is non-delay critical, i.e. in the case reporting threshold is larger than triggering threshold.

Non-delay critical data in enhanced DSR:

Observation 3: If the information of non-delay-critical data from other LCHs with higher LCH priority (higher than that of the LCH which triggers the DSR) is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data, particularly in the case UE supports DSR enhancement but not LCP additional priority handling.

Observation 4: When PSI-based discard is activated, if the information of non-delay-critical data which is in front of the delay-critical data is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data.

Proposal 4: The non-delay-critical data information which has impact on delay-critical data transmission (i.e. in front of the transmission queue) should also be included for each LCG and this includes:

- Non-delay-critical data information from other LCHs with LCH priority higher than that of the LCH which triggers the DSR;

- When PSI-based discard is activated, the data volume of the low importance PDU sets which is ahead of delay-critical data in the UE buffer.

# 4. Reference

[1]. RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3