3GPP TSG-RAN WG2 Meeting #127bis	R2-2408739

Hefei, CN, Oct. 14th-18th, 2024

Agenda Item:	8.8.2

Source: 	CSCN

Title: 	Discussion on Downlink Coverage Enhancements

Document for: 	Discussion and Decision

Introduction

Regarding downlink coverage enhancements in NR NTN, RAN1 has replied to the LS R2-2405767 from RAN2, and the content is excerpted below.

////////////////////

Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?

Answer to Q1:

According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope.

The extension of the SSB periodicity is still under RAN1’s discussion.

Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.

Answer to Q4:

Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

////////////////////

Furthermore, the following agreements were achieved in RAN2#127 [2].

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In this contribution, we discuss system level downlink coverage enhancements in NR NTN based on the reply LS [1] and the report of RAN2#127 meeting [2].

Discussion

2.1 Access restriction in Rel-19 NR NTN

In the last meeting, how to handle legacy UEs was discussed. The companies think beam-hopping mechanisms would increase UE power consumption since the beam state is unknown to legacy UEs and legacy UEs may keep on cell search with no cell found. Based on the discussion in the last meeting, companies tend to bar legacy UEs (i.e., pre-Rel19 NTN UE) from accessing a cell operating with DL coverage enhancement. Moreover, RAN2 agreed the existing NTN bar bit can be used if there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement. In our understanding, barring the legacy UE from accessing a cell operating with DL coverage enhancement is necessary. The network can achieve this access limitation by setting the cellBarredNTN to “barred”. Naturally, a new NTN bar bit should be introduced for Rel-19 NTN UE to determine whether can access the NTN cell.

Proposal 1: A new NTN bar bit should be introduced for R19 NR NTN.

Furthermore, according to the agreement in the last meeting, RAN2 should further discuss the behavior of Rel-19 UEs not supporting DL coverage enhancement. From our perspective, there are three options regarding such UE behavior, which are listed as follows.

- Option 1: Rel-19 UEs not supporting DL coverage enhancement can access a cell operating with DL coverage enhancement for normal service;

- Option 2: Rel-19 UEs not supporting DL coverage enhancement cannot access a cell operating with DL coverage enhancement for any service;

- Option 3: Rel-19 UEs not supporting DL coverage enhancement can access a cell operating with DL coverage enhancement at least for limited service.

For option 1, since such UEs don’t support DL coverage enhancement and can't acquire beam hopping configuration, UEs may perform unnecessary operations in beam “off” state (e.g., monitor paging, measurement and perform RACH), and this will result in much unnecessary power consume. For option 2, considering the following scenario, if all the cells that the UE can detect over a period of time operate with DL coverage enhancement, then the UE can't access any service during that time. We think this is a bad experience for the UE especially in situations where the UE needs to make an emergency call. Therefore, considering that both option 1 and option 2 have shortcomings, we think option 3 is more reasonable, i.e., the cell operating with DL coverage enhancement allows Rel-19 UEs not supporting DL coverage enhancement access for limited service (e.g., initiate emergency calls, receive ETWS and CMAS).

Proposal 2: Rel-19 UEs not supporting DL coverage enhancement are allowed to access a cell operating with DL coverage enhancement at least for limited service.

2.2 Analyses of cell/beam level DTX

Considering that satellite service area may consist of thousands of satellite beams, only a portion of these beams can be activated at any given time due to limited power and the nominal EIRP density per satellite beam. In order to enhance downlink coverage, beam hopping is an efficient technology that allows dynamic and flexible power sharing between satellite beams. Therefore, it is possible that some satellite beams within a cell are active while others remain inactive at the same time based on UE traffic requirements and network implementation. If cell level DTX is adopted, all satellite beams within a cell would be activated or deactivated simultaneously. Therefore, beam level DTX is more suitable.

Proposal 3: RAN2 assumes beam level DTX is preferred for downlink coverage enhancements.

Based on the beam level DTX configuration, UE can know the deactivation/activation time of beams, i.e., the time of beam “off”/“on”. Next, we will analyze the UE’s behaviors based on beam level DTX.

Since beam “off” means there is no Tx signaling for this beam, in order to save UE’s power, the UE should not do any operations in this state, which is applied to RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs. More specifically, for UEs in RRC_IDLE/RRC_INACTIVE, UEs should not initial RRC connection establishment/RRC connection resume procedure, monitor paging, or perform serving cell measurements when the beam is “off”. For UEs in RRC_CONNECTED, UEs should not perform serving cell measurements or detect radio link failure when the beam is “off”.

Proposal 4: RAN2 to discuss the behavior of RRC_IDLE/RRC_INACTIVE mode UEs and RRC_CONNECTED mode UEs in case of beam “off”/“on”.

2.3 How to provide beam-level DTX configuration

In R18 NES, the cell level DTX/DRX configuration is configured only through dedicated signaling, that is because even for the DTX/DRX off state, the common procedure like RACH/paging/SI is not affected. But for Rel-19 NR NTN, the situation is different. Based on our analyses of UE behaviors above, beam hopping mechanical may influence the RRC_IDLE/RRC_INACTIVE UEs in the NTN cell, so we think the DTX configuration should be provided through system information.

As for RRC_CONNECTED UEs, although UEs can use the DTX configuration provided in system information, we think a more flexible approach is necessary. For example, the network may configure UE-specific DTX configuration for UE according to the density of the UEs or the amount of data to be received/sent by the UE. Therefore, providing DTX configuration through dedicated signaling should also be considered.

Proposal 5: RAN2 to discuss how to provide beam level DTX configuration, the following options can be considered:

- Option 1: through system information;

- Option 2: through system information and dedicated signaling.

Furthermore, Rel-18 NES supports DCI-based activation and deactivation of the DTX pattern. We think this dynamic activation/deactivation mechanism can also be considered for RRC_CONNECTED UEs to achieve a more flexible configuration.

Proposal 6: RAN2 to discuss whether dynamic activation/deactivation mechanism for beam level DTX configuration can be considered.

Conclusions

Based on above analyses, proposals are summarized as follows.

Proposal 1: A new NTN bar bit should be introduced for R19 NR NTN.

Proposal 2: Rel-19 UEs not supporting DL coverage enhancement are allowed to access a cell operating with DL coverage enhancement at least for limited service.

Proposal 3: RAN2 assumes beam level DTX is preferred for downlink coverage enhancements.

Proposal 4: RAN2 to discuss the behavior of RRC_IDLE/RRC_INACTIVE mode UEs and RRC_CONNECTED mode UEs in case of beam “off”/“on”.

Proposal 5: RAN2 to discuss how to provide beam level DTX configuration, the following options can be considered:

- Option 1: through system information;

- Option 2: through system information and dedicated signaling.

Proposal 6: RAN2 to discuss whether dynamic activation/deactivation mechanism for beam level DTX configuration can be considered.

Reference

[1] R1-2407538, Reply LS on DL coverage enhancements

[2] Report of 3GPP TSG RAN WG2 meeting #127