3GPP TSG-RAN WG2 Meeting #127bis	R2-2408753

Hefei, China, 14 – 18 October 2024

Agenda item:	8.2.5

Source:	Nokia

Title:	Topology 2 Contribution

WID/SID:	SI_AIoT - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following are the RAN2 agreements related to Topology 2 considerations in RA2-127 meeting. In this discussion paper we analyse remaining issues related to Uu-Interface impacts for handling the AIoT procedure in topology 2.

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface  in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

# 2	Discussion

## 2.1	Topology 2 Procedures

For the reader functionality to be operation with suitable resource allocation for AIoT interface and also communication mechanism with gNB multiple procedures needs to be completed including reader authentication and registration. In this section we discuss about reader registration and selection procedure and impacts to Uu Interface for these procedures

GNB Awareness of Reader Registration

For Topology 2, the radio resource in the Uu Interface needs to be allocated inline with the type of AIoT procedure /traffic being carried out in the Uu Interface. Moreover, for AIoT interface operating in the same band as Uu Interface the GNB needs to be involved in resource allocation for the AIoT radio interface. This coordination is needed for co-existence of AIoT radio interface and Uu-Interface operation. Hence the gNB involved in topology 2 need to know the reader functionality of UE and also the capabilities of the reader related to AIoT-interface that impacts the Uu Interface operation. The reader functionality activation can be informed from CN at the end of registration. However the capability related to AIoT-functions can be either provided from CN or directly from UE via the UE capability information.

Proposal  1 :  The reader registration procedure supports indication of registration status to gNB  . FFS How the Reader capability on AIoT-Interface is indicated to GNB.

## 2.2	User Plane Architecture for T2

The AIoT messages can be carried transparently as NAS payload or UP traffic over the interface between reader and GNB. However, if there could be some cases where GNB may need to know some additional information about the AIoT related message transfers that can be used by GNB for reader selection and discovery later. For better radio resource control for downlink RRC based AIoT Information is preferred. For uplink transmission either NAS based on transperant option can be considered

Proposal 2: For downlink AoT RRC based option is supported. For uplink either NAS based or Transperant architecture can be considered.

The signalling procedures for Inventory and command procedures are under discussion in RAN2.  In case of topology 2 there can be two options to handle the complete AIoT transaction via reader. Either the reader can complete the whole procedure and provide the outcome towards AIoT-Function via NR Air-interface. Alternatively, the reader may forward each transaction transparently.  Each approach have different impacts and advantages. We propose that RAN2 to conclude on the AIoT signalling procedure for above use-cases and analyse the changes needed in data forwarding based on the same.

Proposal 3: RAN2 to study the data forwarding schemes and modifications needed in data forwarding over Uu-radio-interface once the AIoT interface signalling procedure is finalised.

## 2.3	Control Plane Aspects

### 2.3.1 RRC States for Reader

In AIoT, although the amount of data per message is low (~1000 bits) the data rate is also within the area of few, or even fractional kbps. Given a larger set of devices, and multiple messages being transferred within a single procedure, the duration of an AIoT session i.e. from the initial paging being sent till all devices has replied, may comprise several seconds. In this case, having a UE being in RRC_CONNECTED for the entire duration of the AIoT session seems not practical. Additionally, there is no need to require that for communication with an AIoT device the reader UE shall be in RRC_CONNECTED state since this communication is possible for the AIoT device in a stateless manner. Furthermore, Rel-18 brings SDT to both UL and DL, and thus it is possible to communicate with the reader even when it is in RRC_INACTIVE.

Observation 1: Maintaining the Reader UE in RRC_CONNECTED state during the entire AIoT session is not resource efficient.

Observation 2: SDT transmission may be sufficient to start/stop an AIoT session within a reader and provide reader data back to the AIoT access function.

Proposal 4: For topology 2, RAN2 to study AIoT operation with reader UE in RRC_CONNECTED and RRC_INACTIVE for delivery of AIoT data transmission over Uu Interface. FFS on RRC_IDLE.

The UEs in RRC_INACTIVE state can connect to the network by performing Random Access procedure and move to RRC_CONNECTED state, or if the data to be transmitted is small, the UE can perform RA-SDT procedure and transfer the data via RRC Resume request message. The RA-SDT procedure can use 2 step/4 step RA procedure based on the UE configuration. To engage a reader UE for AIoT procedure, it needs to be configured with the inventory or command information. It would be beneficial if the UE can stay in RRC_INACTIVE state and receive the AIoT configuration and perform AIoT communication with AIoT devices while being in RRC_INACTIVE. This would help to save the transition of the UE to RRC_CONNECTED state to receive the AIoT configuration. This maybe an even more significant consideration for applications that require periodic paging of AIoT devices.

Proposal 5: RAN2 to discuss whether a Reader UE should be paged and moved to RRC_CONNECTED to perform an AIoT session or the Reader UE can be configured for AIoT and enabled to perform AIoT communication while being in RRC_INACTIVE state.

### 2.3.2 Reader Operation Support for AIoT Operation for different coverage scenarios

In RAN2-127 following agreement is made related to the radio-condition and procedures of Uu Interface on the reader operation.

Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

Observation 3: Support of out of coverage operation depends on the reader capability to store the AIoT related packets and also the timers associated with the E2E operation.

Proposal 6: Rel-19 study for T2 operation need not consider out of coverage operation that requires store-forward kind of functionality.

## 2.4 Reader Selection

When multiple readers are registered with AIoT access function, a specific reader needs to be selected for triggering AIoT services for the AIoT devices at specific location. In case of two different readers involved, i.e., one for transmitting on R2D link and another on D2R link, a reader pair needs to be selected by the AIoT access function. This selection of readers for devices may help avoiding a device responding to multiple readers and occupying network resources. The reader registration may include additional information that assist the AIoT access function to select specific reader(s) towards AIoT devices in certain location or towards certain group of AIoT devices.  AIoT procedures to trigger the AIoT access function need to identify the anchor BS that servers the relevant reader devices to trigger the reader discovery procedure. There could be some RAN2 aspects in reader discovery  and providing additional information for either the BS or AIOT-function to select the suitable reader to start the AIoT procedure.

It is possible in AIoT deployment scenario, different readers may be within range of different devices, but likewise some readers may be in range of the same devices. In a simply inventory command, where the AIoT access function wants to discover the device IDs and potentially associate upper layer data of nearby devices, controlling a devices’ access to specific readers will mainly reduce resource usage as;

- CN/Readers may configure sufficient resources to the amount of expected UEs

- Devices may refrain from accessing multiple readers in case of multiple parallel pagings in time.

Observation 4: Multiple readers may be able to receive from the same device(s).

Observation 5: Selecting suitable reader(s) for inventory helps with reducing resource usage due to devices being aware to which reader is responded.

Another aspect of the architecture is to discuss whether the AIoT design should include diversity like behaviour, in the sense that multiple readers could be allowed to receive from the same device, on the same paging request. RAN1 already has defined scenarios of having different readers associated to a particular device, i.e. one reader for activating the device and another reader for reading the signal from the device. Along with this, the scenario of independent CW provider is also considered. The different scenario can be seen in the table below.

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b | CW spectrum | D2R spectrum | R2D spectrum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | CW inside topology |  | - CW node inside topology 1 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different | Device 1, 2a | Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL) | Same as CW |  |
| D1T1-A2 |  |  | - CW node inside topology 1 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D |  | Same as D1T1-A1 | Same as CW |  |
| D1T1-B | CW outside topology |  | - CW node outside topology 1 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same |  | Case 1-4 (outside topology, UL) | Same as CW |  |
| D1T1-C | No CW |  | - No CW Node. | Device 2b | N/A | UL |  |
| D2T2-A1 | CW inside topology |  | - CW node inside topology 2 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different - BS communicates with R1 and R2 | Device 1, 2a | Case 2-2 (inside topology, UL) | Same as CW |  |
| D2T2-A2 |  |  | - CW node inside topology 2 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D - BS communicates with R |  | Same as D2T2-A1 | Same as CW |  |
| D2T2-B | CW outside topology |  | - CW node outside topology 2 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - BS communicates with R |  | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW |  |
| D2T2-C | No CW |  | - No CW Node. - BS communicates with R | Device 2b | N/A | FFS |  |
| Note: this table is for the case where D2R is in the same spectrum as CW2D. |  |  |  |  |  |  |  |

This means that it is already expected that reader operation can be dedicated, and thus RAN2 should not preclude a AIoT access function configuring multiple readers for i.e. D2R communication, or even R2D.

Observation 6: Engaging multiple readers in serving the same paging request may provide sufficient diversity to combat limited / interrupted coverage as well as mobility issues.

Proposal 7: Multiple readers can engage in receiving and forwarding responses from multiple AIoT devices to a given paging request (including multiple readers receiving a response from the same AIoT device).

Observation 6: Reader selection will be an important consideration in the AIoT study, as it allows for the devices to be read without significant overhead.

Observation 7: A single reader may not have the full picture of the nearby devices at registration due to change in physical environment.

Proposal 8: RAN2 to agree that readers are selected based on actual radio conditions. FFS whether this includes reader assistance information based on previous device status, keep alive messaging, or discovery procedure.

# 3	Conclusion

We analysed the radio-interface impacts for the reader UE related to topology 2 operation and make the following proposals.

Proposal  1 :  The reader registration procedure supports indication of registration status to gNB  . FFS How the Reader capability on AIoT-Interface is indicated to GNB.

Proposal 2: For downlink AoT RRC based option is supported. For uplink either NAS based or Transperant architecture can be considered.

Proposal 3: RAN2 to study the data forwarding schemes and modifications needed in data forwarding over Uu-radio-interface once the AIoT interface signalling procedure is finalised.

Observation 1: Maintaining the Reader UE in RRC_CONNECTED state during the entire AIoT session is not resource efficient.

Observation 2: SDT transmission may be sufficient to start/stop an AIoT session within a reader and provide reader data back to the AIoT access function.

Proposal 4: For topology 2, RAN2 to study AIoT operation with reader UE in RRC_CONNECTED and RRC_INACTIVE for delivery of AIoT data transmission over Uu Interface. FFS on RRC_IDLE.

Proposal 5: RAN2 to discuss whether a Reader UE should be paged and moved to RRC_CONNECTED to perform an AIoT session or the Reader UE can be configured for AIoT and enabled to perform AIoT communication while being in RRC_INACTIVE state.

Observation 3: Support of out of coverage operation depends on the reader capability to store the AIoT related packets and also the timers associated with the E2E operation.

Proposal 6: Rel-19 study for T2 operation does not consider out of coverage operation that requires store-forward kind of functionality.

Observation 4: Multiple readers may be able to receive from the same device(s).

Observation 5: Selecting suitable reader(s) for inventory helps with reducing resource usage due to devices being aware to which reader is responded.

Observation 6: Engaging multiple readers in serving the same paging request may provide sufficient diversity to combat limited / interrupted coverage as well as mobility issues.

Proposal 7: Multiple readers can engage in receiving and forwarding responses from multiple AIoT devices to a given paging request (including multiple readers receiving a response from the same AIoT device).

Proposal 8: RAN2 to agree that readers are selected based on actual radio conditions. FFS whether this includes reader assistance information based on previous device status, keep alive messaging, or discovery procedure.