3GPP TSG-WG2 Meeting #127bis	R2-2408755

# Hefei, China, 14th  - 18th OCT 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1947 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to 38.321 on LTM Cell Switch UP Aspect |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current 38.321, the UE operation on receiving LTM Cell switch MAC CE is captured in the subclause 5.18.35, in that subclause, only RACH-less LTM Cell switch is indicated. There is no any clear guidance for UE how to continue LTM Cell switch if the RACH-less Cell switch cannot be executed , in other words, if there is no valid TA obtained no matter from either LTM Cell switch MAC CE or UE based TA measurement. For RACH less based LTM, the MAC entity to indicate upper layer about the successfully terminating of the LTM cell switch procedure, but for the RACH based LTM, the similar procedure is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: clarify that in the subcaluse 5.18.35, the MAC entity shall initiate the RACH procedrue if theTA value are received in the LTM cell swtich MAC CE is FFF and UE cannot get TA by using UE based TA measurement. 2: clarify that in the subclause 5.1.6, the MAC entity of the UE shall indicate the success of the RACH procedure for the LTM Cell switch to the upper layer in order for upper layer to stop the regulate timer. Impact analysis Impacted 5G architecture options: NR SA, MR-DC Impacted functionality: LTM Cell Switch Interoperability: If the NW implements this CR but not the UE, the MAC entity of the UE may have no idea how to execute the LTM cell switch if there is no valid TA of the target cell can be obtained. The RRC entity may not obtain the successful indication from lower layer, then the T304 cannot be stopped, which may cause the RRC reestablishment or fast recovery. If the UE implements this CR but not the NW, there is no any interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may have no idea how to execute the LTM cell switch if there is no valid TA of the target cell can be obtained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6, 5.18.35 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

The First Change

5.1.6	Completion of the Random Access procedure

Upon completion of the Random Access procedure, the MAC entity shall:

1>	discard any explicitly signalled contention-free Random Access Resources for 2-step RA type and 4-step RA type except the 4-step RA type contention-free Random Access Resources for beam failure recovery request, if any;

1>	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer and the MSGA buffer.

Upon successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall:

- indicate the successful completion of the Random Access procedure to the upper layers.

Upon successful completion of the Random Access procedure initiated for LTM Cell switch, the MAC entity shall:

1 > inidicate the successful completion of the Random Access procedure to the upper layer.

The Second Change

18.35	LTM Cell Switch Command

The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75.

The MAC entity shall:

1>	if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell:

2>	indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE;

2>	if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

3>	if Timing Advance Command value (hexa-decimal) is not set as FFF; or

3> if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the indicated LTM target configuration:

4>	process the received Timing Advance Command (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing;

4>	if the MAC entity is associated with SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

4> consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2);

4> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

3> else:

4> initiat the RACH for LTM cell switch (as in clause 5.1).