3GPP TSG-RAN WG2 #127-bis	R2-2408758

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.6.2

Source:	Ericsson

Title:	DC aspects for inter-CU LTM

Document for:	Discussion

# 1	Introduction

The recent advancement in this work item for inter-CU LTM brings the total readiness to a good point. With that said there are some discussions necessary still which is brought up in this contribution. The SA3 input is expected to be ready for RAN2#128 and thus the security related aspects for vertical key derivation are not included in this contribution.

# 2	Discussion

## 2.1	Handling of SCG

In the last RAN2 meeting, the following agreement has been taken for how to handle the SCG when an inter-CU LTM cell switch is executed:

SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

Based on this agreement, if the UE has an SCG at the moment an inter-CU LTM cell switch is executed at the MN, the SCG can be released, configured, or kept as it is. Further, according to the agreement that RAN2 took, this also implies that when an inter-CU LTM cell switch is executed, there is also the possibility to add an SCG. Therefore, we propose:

RAN2 to confirm that when an inter-CU LTM cell switch is executed, the SCG addition is supported.

Given that now the SCG can be either added, released, or reconfigured, it is evident that this behaviour is basically the same that we have for the legacy DC procedures that have been specified since Rel-15. However, in Rel-18 there is a restriction on how the SCG is handled in case an intra-CU LTM cell switch is triggered. In fact, when this happen the SCG can either be kept of released.

In Rel-18, in case an intra-CU LTM cell switch is triggered. In fact, when this happen the SCG can either be kept or released.

Since this restriction has been lifted in Rel-19, it would be good to clarify that also when an intra-CU LTM cell switch is executed, how to handle the SCG part is up to the network implementation (e.g., it is either released or reconfigured).

The SCG configuration can be changed when an intra-CU LTM cell switch is executed and how to handle the SCG is up to the network implementation.

One aspect that needs to be considered, however, is whether the SCG configuration in case of reconfiguration (or addition, if supported) can be a delta configuration or always a full configuration. In case we allow the SCG configuration to be a delta of the current UE configuration on when LTM is configured, the following scenario can happen:

- Step 1: UE has an SCG and MN LTM candidate configuration ID 1 is added which has an SCG configuration that is a delta of the current SCG configuration of the UE, and MN LTM candidate configuration ID 2 is added which releases the current SCG configuration at the UE.

- Step 2: UE performs an MN LTM cell switch to LTM candidate configuration ID 2 which then releases the SCG.

- Step 3: UE performs an MN LTM cell switch to LTM candidate configuration ID 1 which potentially can add the SCG, but since this was a delta of the previous SCG configuration of the UE (which has been released by LTM candidate configuration 2) the UE considers the SCG configuration is invalid and triggers the SCG failure configuration procedure.

According to the example, for a MN LTM candidate configuration a simple way to avoid issues in the handling of the SCG would be to only allow the SCG configuration to be always a full configuration. Thus, we propose:

If the MN LTM candidate configuration includes an SCG configuration (to reconfigure or add the SCG during an LTM cell switch procedure at the MN), this SCG configuration is always a full configuration.

## 2.2	How to provide sk-counter(s) for the SCG

The security at the SCG is handled according to the reception of the sk-counter from the MN. In this case, in the last RAN2 meeting the following agreement has been reached:

RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

With this agreement, RAN2 has agreed to consider what has been agreed for subsequent CPAC as a baseline. If we look at the signalling in Rel-18, for subsequent CPAC the MN provides a list of sk-counters which are linked to a particular SN and that are used only when execution of a subsequent CPAC configuration leads to that an inter-SN change or SCG addition is performed:

-- ASN1START

-- TAG-CONDITIONALRECONFIGURATION-START

ConditionalReconfiguration-r16 ::=   SEQUENCE {

attemptCondReconfig-r16              ENUMERATED {true}              OPTIONAL,   -- Cond CHO

condReconfigToRemoveList-r16         CondReconfigToRemoveList-r16   OPTIONAL,   -- Need N

condReconfigToAddModList-r16         CondReconfigToAddModList-r16   OPTIONAL,   -- Need N

...,

[[

scpac-ReferenceConfiguration-r18     SetupRelease {ReferenceConfiguration-r18}          OPTIONAL,   -- Need M

servingSecurityCellSetId-r18         SecurityCellSetId-r18                              OPTIONAL,   -- Need M

sk-CounterConfiguration-r18          SK-CounterConfiguration-r18                        OPTIONAL    -- Need M

]]

}

CondReconfigToRemoveList-r16 ::=     SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

SK-CounterConfiguration-r18      ::= SEQUENCE {

sk-CounterConfigToReleaseList-r18    SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SecurityCellSetId-r18    OPTIONAL,    -- Need N

sk-CounterConfigToAddModList-r18     SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfig-r18     OPTIONAL     -- Need N

}

SK-CounterConfig-r18 ::=             SEQUENCE {

securityCellSetId-r18                SecurityCellSetId-r18,

sk-CounterList-r18                   SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter

}

SecurityCellSetId-r18 ::= INTEGER (1.. maxSecurityCellSet-r18)

-- TAG-CONDITIONALRECONFIGURATION-STOP

-- ASN1STOP

For subsequent CPAC, each list of sk-counters is associated to one or more candidate configurations.

Given that the assumption of CPAC is that the MN does not change, for LTM the approach is different as the list of sk-counter is for the case when an inter-CU LTM cell switch is executed, and the security needs to be refreshed at the SCG.

For MCG inter-CU LTM the approach is different as the list of sk-counters is for the case when an MCG inter-CU LTM cell switch is executed, and the security needs to be refreshed at the SCG.

Since in LTM we also need to handle the case when the UE performs multiple LTM cell switches back and forth between the two same MNs, in this case it would make sense that each MN provide its own list of sk-counters, and that this list is included in the MN RRC reconfiguration message, thus inside of the LTM-Config IE.

Each MN LTM candidate configuration includes a list of sk-counters.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In Rel-18, in case an intra-CU LTM cell switch is triggered. In fact, when this happen the SCG can either be kept or released.

Observation 2	For subsequent CPAC, each list of sk-counters is associated to one or more candidate configurations.

Observation 3	For MCG inter-CU LTM the approach is different as the list of sk-counters is for the case when an MCG inter-CU LTM cell switch is executed, and the security needs to be refreshed at the SCG.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that when an inter-CU LTM cell switch is executed, the SCG addition is supported.

Proposal 2	The SCG configuration can be changed when an intra-CU LTM cell switch is executed and how to handle the SCG is up to the network implementation.

Proposal 3	If the MN LTM candidate configuration includes an SCG configuration (to reconfigure or add the SCG during an LTM cell switch procedure at the MN), this SCG configuration is always a full configuration.

Proposal 4	Each MN LTM candidate configuration includes a list of sk-counters.

# 4	References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.