3GPP TSG-RAN WG2 Meeting #127bis	R2-2408763

# Hefei, China, Oct 14th – 18th, 2024

Source:                      Qualcomm Incorporated

Title:                          LP-WUS in IDLE and Inactive state

Agenda Item:	8.4.2

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# Discussion

## 2.1 LP-WUS mode control

In the last RAN2 meeting, RAN2 made working assumption that LP-WUS monitoring entry condition can also include LR-based threshold. It seems RAN1 and RAN4 has no concerns on it, then it is proposed

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

- RAN2 confirms the working assumption

	Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

There were some discussions in the last RAN2 meeting that whether dedicated LP-WUS configuration can be provided to the UE. It is understood that the LP-WUS configuration here means entry/exit condition, it is unclear what is the motivation for dedicated entry/exit condition to be introduced. And if the dedicated condition is introduced, the UE needs to maintain both the dedicated configuration and common configuration and RAN2 also needs to discuss in what condition the dedicated configuration is invalid.

- Do not support dedicated LP-WUS monitoring entry/exit condition configuration.

There are some proposals that network can activate or deactivate LP-WUS monitoring using dedicated RRC signalling. If the UE is deactivated LP-WUS monitoring, then the UE will never use LP-WUS to save power consumption as long as the UE stays in IDLE state in this cell. And the motivation is not clear. Then it is proposed not to introduce LP-WUS activation/deactivation in dedicated RRC message.

- Do not introduce LP-WUS activation/deactivation in dedicated RRC message.

Some companies propose to indicate in SIB whether the UE should skip PEI monitoring when the UE is monitoring LP-WUS. The motivation is to replace PEI with LP-WUS totally, and the network does not need to send indication signalling in both LP-WUS and PEI. But the issue is, when the UE moves outside of LP-WUS coverage, the UE will not indicate to the network as the baseline RAN2 made in the last meeting and the network cannot enable PEI, so the UE cannot use PEI. Another issue is, LP-WUS and PEI can be used together to reduce more false paging. Also, RAN1 has discussed this issue, and the agreement is that it is left to UE implementation to monitor PEI or not.

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

- Network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB.

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

In the last RAN2 meeting, there were discussion on whether entry/exit condition of LP-WUS monitoring should be mandatory or can be optionally configured.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Since gNB does not know whether the UE enters or leaves LP-WUS mode, then gNB needs to send paging indication in both of LP-WUS and PEI or legacy paging if PEI is not supported. Then it is not critical that the condition is absent. UE can determine whether to enter or exit LP-WUS based on the MR/LR measurement.

- The configuration of the entry/exit condition of LP-WUS monitoring can be optionally configured to the UE. If absent, it is left to UE implementation to determine whether to enter or exit LP-WUS mode.

In the last RAN2 meeting, it has been agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

One issue is, if both of the OFDM-based and OOK-based WUS are configured for the entry/exit threshold. which one of should be used by the UE. It assumes that LP-WUS UE may have different capabilities on OFDM based WUS measurement. In this case, it should allow the UE to select the measurement metrics the UE can support.

- It is up to UE implementation to choose which one is used if both of OFDM-based and OOK-based WUS entry/exit thresholds are configured.

## 2.2 Whether to support dynamic PO

In RAN1 there is some discussion on dynamic PO, and has following agreement. Dynamic PO has also impact on higher layer.

Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO

Dynamic paging occasion means UE monitors paging after deterring LP-WUS indicating its subgroup ID and UE does not need to monitor paging on legacy paging occasion.

Currently, network and UE calculate paging occasions and transmit/receive paging message on paging occasion. When DL data arrives to CN, CN will calculate paging occasion, and send paging request to gNB before the paging occasion to reduce gNB burden of saving paging for a long time.

If dynamic PO is introduced, CN will send paging request to gNB when DL data arrives, which will change existing CN behavior.

- Currently, CN will calculate paging occasion, and send paging request to gNB before the paging occasion. If dynamic PO is introduced, CN will send paging request to gNB when DL data arrives, which will change existing CN behaviour.

UE in IDLE and Inactive state will not indicate to gNB when the UE enters or leaves LP-WUS mode, then when gNB does not know whether legacy paging on LP-WUS paging will be used. gNB has to send two paging message on the dynamic PO and legacy PO, then paging message load will be doubled.

- gNB needs to send two paging message on both of dynamic PO and legacy PO, which will waste paging resource.

Based on the above observations, it is proposed dynamic PO is not introduced.

- Dynamic PO mechanism is not introduced, and legacy PO mechanism is used in LP-WUS mode.

## 2.3 LP-WUS subgrouping

LP-WUS has similar functionality with existing PEI to save UE power consumption for paging monitoring. It is assumed LP-WUS will indicate the intended subgrouping and UE will determine whether to wake up MR to further monitor paging.

One issue is whether UE needs to continue to monitor PEI or UE directly monitor paging message. There are two possible options.

Option 1: LP-WUS replaces PEI

In this option, UE capable LP-WUS only monitors LP-WUS, and if UE is indicated to trigger MR for monitor paging, UE will skip PEI and monitor paging message directly.

Option 2: LP-WUS is used together with PEI

In this option, UE capable LP-WUS firstly monitors LP-WUS, and if the UE is indicated to trigger MR for paging monitoring, UE will continue to monitor PEI and determine whether to further monitor paging message. It is assumed that two levels of subgroup will be indicated in LP-WUS and PEI. The UEs mapped to the same LP-WUS subgroup can be further distributed into different PEI subgroups. This option can save LP-WUS overhead compared to option 1. But it brings more complexity for UE implementation. As shown in the figure, the joint subgrouping between LP-WUS and PEI improves the subgrouping granularity by a factor of the number of PEI subgroups with no additional signaling cost.

|  |  | PEI grouping |  |  |  | P1 = {UE1, UE4, UE7} |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | P1 | P2 | P3 |  | P2 = {UE2, UE5, UE8} |
| LP-WUS grouping | L1 | UE1 | UE2 | UE3 |  | P3 = {UE3, UE6, UE9} |
|  | L2 | UE4 | UE5 | UE6 |  | L1 = {UE1, UE2, UE3} |
|  | L3 | UE7 | UE8 | UE9 |  | L2 = {UE4, UE5, UE6} |
|  |  |  |  |  |  | L3 = {UE7, UE8, UE9} |

Figure: Different subgroup splitting for LP-WUS and PEI.

- LP-WUS can be used together with PEI, in which the UE is indicated by LP-WUS can be additionally sub-grouped by PEI.

In existing PEI mechanism, if the UE detects PEI and the PEI indicates the subgroup the UE belongs to monitor its associated PO, the UE monitors the associated PO.

There are CN assigned subgrouping and UE_ID based subgrouping defined. For CN assigned subgrouping, a UE can be assigned a subgroup ID (between 0 to 7) by AMF through NAS signalling.

For UE_ID based subgrouping, the following equation is used to determine the subgroup ID.

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID)

For CN based mechanism, RAN2 agreed that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

For UE_ID based subgrouping, UE needs to determine LP-WUS subgroup ID and PEI subgroup ID. It needs to be further discussed how the UE derives the LP-WUS subgroup ID. then it is possible to design the hashing function for UE_ID such that each subgroup in LP-WUS can fully contain several subgroups in PEI. Then again, no additional UEs are waked up by PEI than the set of UEs waken up by LP-WUS.

- For UE-based subgrouping, the subgroup ID in LP-WUS can be independent with the one in PEI.

- It is FFS whether and how UE based subgrouping is enhanced for LP-WUS subgroup ID and PEI subgroup ID.

Now suppose subgroups in LP-WUS and PEI are of different types, e.g. subgroups in LP-WUs are assigned by CN and those in PEI are generated based on UE_ID. Then it is not difficult to see that the set of UEs in a subgroup in LP-WUS can be very different from those in a subgroup in PEI. What this difference means is that it can increase false paging alerts. That is not desirable.

- If LP-WUS and PEI are used together but they have different types of subgroups, the set of UEs in a LP-WUS subgroup can be different from those in a PEI subgroup. This difference can increase false paging alerts.

For this reason, we think it is better for network to ensure that the same type of subgroups are used in LP-WUS and PEI. This may require some coordination in CN and RAN. But the details can be studied in the WI phase.

- If LP-WUS and PEI are used together, network should configure them with the same type of subgroups. FFS how network ensures that.

## 2.4 LP-WUS DRX cycle and occasion

LP-WUS DRX cycle

In RAN1 #116, two options were discussed for the periodicity of LO for a UE in iDRX mode:

- Option 1: it is the same as the PO iDRX cycle

- Option 2: it can be the same as or smaller than the PO iDRX cycle

We think option 1 is sufficient for the idle/inactive mode LP-WUS design. For option 2, maybe the idea is to use different LOs for different sets of LP-WUSs that are associated with different sets of UE subgroups, or to enable repetitions of LP-WUS in the same LO for coverage enhancements. The only difference between option 2 and option 2 enables even distribution of chunks of LP-WUS MOs associated with different sets of UE subgroups or repetitions. However, it is unclear what benefit the even distribution can provide. Hence, we support option 1 for the periodicity of LO.

- LO DRX cycle for a UE in idle/inactive mode is the same as the PO iDRX cycle.

When the LO DRX cycle is the same as the iDRX cycle, for the association of LO and PO, the following options were discussed in RAN #116:

- Option 1: One LO is associated with one PO

- Option 2: One LO can be associated with multiple POs

- Option 3: Multiple LOs can be associated with one PO

Option 1 should be supported as baseline for the idle/inactive LP-WUS design. Option 3 is equivalent to multiple LP-WUS MOs in the LO that are associated with different sets of UE subgroups. It can be combined with option 1. For option 3, the LP-WUS carries more information than option 1, and hence occupies more OFDM symbols. However, this may pose design challenges due to the limited data rate of LP-WUS. For example, if network wants to align the LP-WUS beams with SSB beams, the maximum LP-WUS transmission duration is confined within the inter-SSB interval which can be as small as half slot. This restricts the maximum number of ON/OFF symbols of LP-WUS to be a relatively small value even for the agreed maximum OOK-4 M value 4.

- Support one LO is associated with one paging cycle.

## 2.5 UE LP-WUS capability reporting

For CN-assigned subgroup ID, UE should report capability to CN, and then CN can assign CN-assigned subgroup ID to UE.

For UE-based subgroup ID, it is also beneficial to indicate CN UE capability. In existing paging framework, CN can also calculate POs and sends Paging Request to gNB just earlier than the PO. The motivation is to avoid gNB storing paging for a long time. If the UE does not indicate UE-based subgrouping capability to CN, then CN will send Paging Request to gNB as legacy, i.e. just earlier than legacy PO. However, the time of sending Paging Request could be late that LP-WUS occasion, and gNB has to send LP-WUS indication in the next cycle. This will delay paging one paging DRX cycle. This can be shown in the following figure.

- Without UE-based subgrouping capability in CN, the paging could be delayed one paging cycle.

- UE should indicate supporting CN-assigned and/or UE-based subgrouping with LP-WUS in the REGISTRATION REQUEST message

# Conclusion

- If LP-WUS and PEI are used together but they have different types of subgroups, the set of UEs in a LP-WUS subgroup can be different from those in a PEI subgroup. This difference can increase false paging alerts.

- Currently, CN will calculate paging occasion, and send paging request to gNB before the paging occasion. If dynamic PO is introduced, CN will send paging request to gNB when DL data arrives, which will change existing CN behaviour.

- gNB needs to send two paging messages on both of dynamic PO and legacy PO, which will waste paging resource.

- Without UE-based subgrouping capability in CN, the paging could be delayed one paging cycle.

- LP-WUS mode control

- RAN2 confirms the working assumption

	Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

- Do not support dedicated LP-WUS monitoring entry/exit condition configuration.

- Do not introduce LP-WUS activation/deactivation in dedicated RRC message.

- Network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB.

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

- The configuration of the entry/exit condition of LP-WUS monitoring can be optionally configured to the UE. If absent, it is left to UE implementation to determine whether to enter or exit LP-WUS mode.

- It is up to UE implementation to choose which one is used if both of OFDM-based and OOK-based WUS entry/exit thresholds are configured.

- Dynamic PO mechanism is not introduced, and legacy PO mechanism is used in LP-WUS mode.

- LP-WUS subgrouping

- LP-WUS can be used together with PEI, in which the UE is indicated by LP-WUS can be additionally sub-grouped by PEI.

- For UE-based subgrouping, the subgroup ID in LP-WUS can be independent with the one in PEI.

- It is FFS whether and how UE based subgrouping is enhanced for LP-WUS subgroup ID and PEI subgroup ID.

- If LP-WUS and PEI are used together, network should configure them with the same type of subgroups. FFS how network ensures that.

- LP-WUS DRX cycle and occasion

- LO DRX cycle for a UE in idle/inactive mode is the same as the PO iDRX cycle.

- Support one LO is associated with one paging cycle.

UE LP-WUS capability reporting

- UE should indicate supporting CN-assigned and/or UE-based subgrouping with LP-WUS in the REGISTRATION REQUEST message.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)