3GPP TSG-RAN WG2 Meeting #127bis	R2-2408764

Hefei, China, Oct 14th – 18th, 2024

Source:                Qualcomm Incorporated

Title:	LP-WUS operation in CONNECTED state

Agenda Item:	8.4.4

Document for:	Discussion and Decision

# Introduction & Background

• Initiate the discussion on the configuration, activation/deactivation.

• Initiate the discussion on the procedure of using LP-WUS with/without C-DRX.

# Discussion

## 2.1 RAN2 impact on LP-WUS options

In RAN1, there are several options about how UE performs LP-WUS in CONNECTED state. Since these options impacts RAN2 specification, it is worthy to evaluate these options from RAN2 perspective.

RAN1 achieved following agreement and working assumption at the RAN1#118 meeting:

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

### 2.1.1    RAN2 aspects for Option 1-1

In the last RAN2 meeting, it is agreed to take the existing DCP mechanism as baseline. Based on this agreement.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

In existing DCP configuration, besides legacy long C-DRX configuration, DCP configurations are also provided to the UE. Similarly, option 1-1 configuration should also include LP-WUS related configurations. Additionally, the indications similar with the following DCP indications can also be configured to the UE.

o	LPWUS-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

o	LPWUS-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

o	LPWUS-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

- For Option 1-1, the following configurations can be provided to the UE.

- Legacy long DRX configuration.

- LPWUS-Offset: The start of LPWUS monitoring relative to the start of drx-onDurationTimer of Long DRX.

- the indications similar with the following DCP indications can also be configured to the UE.

o	LPWUS-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

o	LPWUS-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

o	LPWUS-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

RAN1 agreed that for option 1-2, UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected ( drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL ). The similar agreements can also be applied to option 1-1.

- For Option 1-1, no impact on RRM/RLM/BFD measurement requirements is assumed.

- For Option 1-1,  UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected ( drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL ).

In existing DCP mechanism, how to monitor PDCCH after receiving DCP indication is specified in TS 38.331 as follows. Since Option 1-1 LP-WUS just replaces existing DCP using LR, then the procedure can be reused for how to use LP-WUS to trigger PDCCH monitoring, i.e. all

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

- For Option 1-1, all the existing conditions to start drx-onDurationTimer defined in clause 5.7 of TS38.321 for DCP can be reused to Option 1-1 LP-WUS.

### 2.1.2    RAN2 aspects for Option 1-2

Option 1-2 introduces additional enhancements that UE can monitor PDCCH after successfully detecting LP-WUS, irrespective of the PDCCH occasion being outside or inside of the legacy PDCCH active time window, as shown in Figure 2. There are two options  on whether UE needs to monitor legacy PDCCH occasions, as shown in the figure below.

While it could be argued that this approach could reduce the DL transmission latency due to legacy C-DRX cycle, there could be a significant specification impact and  the benefits have not been evaluated.

The newly introduced PDCCH occasions do not follow existing C-DRX configurations such as drx-LongCycleStartOffset, drx-SlotOffset, shortDRX. This would require new mechanisms and a new RRC configurations. Also, a new MAC procedure and PDCCH monitoring procedure (e.g. UE behavior on starting or stopping monitor PDCCH occasions) need to be introduced.

The relationship of the new PDCCH occasions and legacy PDCCH occasions needs to be further discussed, e.g. whether UE additionally monitors the legacy PDCCH occasions, as Option 1-2-1 and Option 1-2-2. The legacy C-DRX normally is configured according to DL QoS requirements and traffic characteristics, and if UE does not monitor legacy PDCCH occasion, whether the QoS requirement may be deviated. Also, interaction with HARQ operation has to be specified.

For Option 1-2-1, gNB has to send PDCCH in both of the new PDCCH occasions and legacy PDCCH occasions, which will consume PDCCH signalling overhead, and UE also needs to monitor both of the newly introduced PDCCH active time window and legacy active time window defined by drx-onDurationTimer. Then from RAN2 perspective, Option 1-2-1 is not preferred.

- From RAN2 perspective, Option 1-2-1 is not preferred.

For Option 1-2-2, RAN1 agreed that  LP-WUS related configuration needs to be provided to the UE. A drx-onDurationTimer-LPWUS which is similar to drx-OnDurationTimer in the legacy C-DRX operation. The UE can stop PDCCH monitoring when the timer expires. Besides, the LP-WUS monitoring occasions need to be configured to the UE using LP-WUS cycle and offset.

RAN1 agreements:

- UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

  - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

- For Option 1-2-2, the following LP-WUS configuration needs to be provided to the UE

-  drx-onDurationTimer-LPWUS: newly introduced PDCCH monitoring on-duration timer for LP-WUS, which is similar with drx-onDurationTimer.

- LPWUSCycleStartOffset: LP-WUS DRX cycle and drx-StartOffset which defines the subframe where LP-WUS DRX cycle starts;

-  SlotOffset-LPWUS: the delay before starting the drx-onDurationTimer-LPWUS in the subframe

- LPWUS-offset: The start of drx-onDurationTimer-LPWUS relative to the start of LPWUS monitoring.

### 2.1.3    RAN2 aspects for Option 1-3

Option 1-3 is further enhanced of the Option 1-1, where LP-WUS is monitored inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring, as shown in Figure 3.

This option has potential benefit on UE power consumption as UE does not need to monitor all PDCCH occasions during the active time duration. Only when the UE detects LP-WUS successfully, it will monitor PDCCH occasion during the active time duration. This option enables more flexible triggering for PDCCH monitoring that is not restricted by C-DRX configuration. However, unlike Option 1-2, the start of PDCCH monitoring in Option 1-3 is not fully flexible – it is anytime but within ON-duration identified by C-DRX configuration. In other words, PDCCH monitoring windows still follows existing C-DRX configurations such as drx-LongCycleStartOffset, drx-SlotOffset, shortDRX. From MAC layer point of view, a PDCCH monitoring, start at a middle of a DRX active time, is not different from the PDCCH monitoring in the DRX active time that starts at the beginning of the DRX active time. Therefore, there is no reason not to re-use all the other legacy MAC timers, drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, for Option 1-3.

- For Option 1-3, all the legacy RRC parameters for C-DRX configuration can be re-used

  - drx-OnDurationTimer, drx-LongCycleStartOffset, drx-SlotOffset provides the understanding of legacy C-DRX active time

  - drx-OnDurationTimer, drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, enables PDCCH monitoring window extension same as in the legacy C-DRX configuration

## 2.3 Switching between LP-WUS and legacy PDCCH monitoring

As discussed in IDLE/Inactive state, it is possible the LP-WUS coverage is smaller than MR coverage. In this case, UE should be able to fall back to PDCCH monitoring on MR using the configured C-DRX configuration.

Considering different LP-WUS options and different legacy PDCCH there are two fallback cases to be considered.

Case 1: UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured

In this case, the UE is using LP-WUS option 1-1, when the UE detects moving outside of LP-WUS coverage, UE can automatically switch to legacy C-DRX PDCCH monitoring. There is no essential need for the network to know the exact moment that the UE autonomously fallback to PDCCH monitoring using C-DRX configuration. In option 1-1, the network anyway transmits both LP-WUS and PDCCH in the corresponding C-DRX active time. The UE monitors either LP-WUS or PDCCH in the corresponding C-DRX active time, depending on fallback status. Therefore, the network does not need to know the particular moment that the UE switches back to legacy C-DRX operation. However, it would be beneficial for network to know whether the UE is able to detect the LP-WUS or not, then gNB can determine whether to send LP-WUS any more.

If the UE is configured DCP already, then gNB should send DCP enable command to the UE to activate DCP functionality.

Observation 1: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Case 2: UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured

For Option 1-2, if UE autonomous fallback to legacy C-DRX is enabled for Option 1-2, network does not know whether/when the UE leaves LP-WUS mode. There can be misalignment between the UE and the gNB when the UE monitors legacy PDCCH and gNB will continue to send LP-WUS indication and PDCCH in the new active time window, and then PDCCH could be missed. So in this case, UE should report to the gNB before switching to legacy PDCCH monitoring. And gNB should send command to the UE to switch to legacy PDCCH monitoring.

If DCP is configured already, gNB can also indicate whether DCP functionality is enabled.

Observation 2: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Case 3: UE is using legacy PDCCH monitoring with/without DCP configured and enters LP-WUS mode

In this case, the UE is monitoring PDCCH as legacy, and UE detects entering LP-WUS coverage. Since for LP-WUS, gNB should firstly indicate to UE in LP-WUS and then send PDCCH on active time window. Then UE should report to the gNB before switching LP-WUS. And gNB should send switching command to the UE to enable LP-WUS. Then there is no mis-alignment between the gNB and the UE, and there is no potential PDCCH monitoring missed.

Observation 3: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

Considering the above cases, UE should indicate to gNB when enters or leave LP-WUS mode. The condition for entering or leaving LP-WUS mode could be based on MR measurement, it can be FFS for other conditions, and it can be FFS on whether the entry/exist condition need to be configured to the UE or can be left UE implementation.

- For both of Option 1-1 and Option 1-2,  UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- The LP-WUS entry/exist condition can be based on MR measurement. FFS for other conditions.

- It is FFS whether the entry/exist conditions are configured by the network or leave to UE implementation.

- For option 1-1, when the UE switches from LP-WUS mode to legacy PDCCH monitoring, RAN2 discusses whether gNB needs to send a switching command to UE.

- For option 1-2, when the UE switches from LP-WUS mode to legacy PDCCH monitoring, gNB needs to send a switching command to UE.

## 2.4 LP-WUS activation/deactivation

RAN1 decided to study activation/deactivation of LP-WUS monitoring as highlighted below in the RAN1#116 agreement.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

It could be possible that a network configures one or multiple LP-WUS configurations, and one of the LP-WUS configuration is activated by the MAC-CE. The MAC-CE can also deactivate LP-WUS monitoring (i.e., it activates none of the LP-WUS configurations), in which case the UE should switch to legacy C-DRX configuration.

In each of the LP-WUS configurations, various RRC parameters can be provided, then the MAC-CE activation can control the overhead/efficiency, associated beam, etc, in dynamic manner.

- gNB can configure multiple LP-WUS configurations to the UE, and activate one of the configurations to the UE via MAC CE.

## 2.3 LP-WUS in DC/CA

First of all, it must be common understanding that for a UE configured with dual connectivity (DC), support LP-WUS monitoring on a serving cell per cell-group. It does not make sense to enable LP-WUS that triggers PDCCH monitoring for serving cells in different cell-groups, since different cell-groups are supposed to be associated with different gNB schedulers.

For CA, from Rel-16, dual DRX groups has been supported. A UE can be configured with different values of drx-onDurationTimer and drx-InactivityTimer for one DRX group and for the other DRX group. Long DRX cycle is common for both DRX groups.

Dual DRX groups

Rel-16 DCP was enabled only on PCell or PSCell and indicates to start drx-onDurationTimer of all the DRX groups. In other words, it was not possible to trigger PDCCH monitoring of different DRX groups independently. This is not beneficial for UE power saving. For Rel-19 LP-WUS in connected mode, per DRX group PDCCH monitoring triggering should be considered. There are two options to enable this:

- Opt.1: LP-WUS is configured to be monitored per DRX group

- Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

Option 1 is enabled if LP-WUS is monitored in two different serving cells in different DRX groups. The LP-WUS in each serving cell is nothing to do with PDCCH monitoring for the cell(s) in the other DRX group. Option 2 is enabled without LP-WUS monitoring on multiple serving cells. However, it has to reserve a couple of bits in the LP-WUS to indicate DRX group. Further study is necessary on which option to support.

- For a UE configured with DC, support LP-WUS monitoring on a serving cell per cell-group

- For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group

  - Opt.1: LP-WUS is configured to be monitored per DRX group

    - For dual DRX groups, LP-WUS monitoring is configured per DRX group

    - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

  - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

# Conclusion

RAN2 impact on LP-WUS options,

- For Option 1-1, the following configurations can be provided to the UE.

- Legacy long DRX configuration.

- LPWUS-Offset: The start of LPWUS monitoring relative to the start of drx-onDurationTimer of Long DRX.

- the indications similar with the following DCP indications can also be configured to the UE.

o	LPWUS-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

o	LPWUS-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

o	LPWUS-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

- For Option 1-1, no impact on RRM/RLM/BFD measurement requirements is assumed.

- For Option 1-1,  UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected ( drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL ).

- For Option 1-1, all the existing conditions to start drx-onDurationTimer defined in clause 5.7 of TS38.321 for DCP can be reused to Option 1-1 LP-WUS.

- From RAN2 perspective, Option 1-2-1 is not preferred.

- For Option 1-2-2, the following LP-WUS configuration needs to be provided to the UE

-  drx-onDurationTimer-LPWUS: newly introduced PDCCH monitoring on-duration timer for LP-WUS, which is similar with drx-onDurationTimer.

- LPWUSCycleStartOffset: LP-WUS DRX cycle and drx-StartOffset which defines the subframe where LP-WUS DRX cycle starts;

-  SlotOffset-LPWUS: the delay before starting the drx-onDurationTimer-LPWUS in the subframe

- LPWUS-offset: The start of drx-onDurationTimer-LPWUS relative to the start of LPWUS monitoring.

- For Option 1-3, all the legacy RRC parameters for C-DRX configuration can be re-used

  - drx-OnDurationTimer, drx-LongCycleStartOffset, drx-SlotOffset provides the understanding of legacy C-DRX active time

  - drx-OnDurationTimer, drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, enables PDCCH monitoring window extension same as in the legacy C-DRX configuration

Switching between LP-WUS and legacy PDCCH monitoring

Observation 1: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Observation 2: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Observation 3: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

- For both of Option 1-1 and Option 1-2,  UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- The LP-WUS entry/exist condition can be based on MR measurement. FFS for other conditions.

- It is FFS whether the entry/exist conditions are configured by the network or leave to UE implementation.

- For option 1-1, when the UE switches from LP-WUS mode to legacy PDCCH monitoring, RAN2 discusses whether gNB needs to send a switching command to UE.

- For option 1-2, when the UE switches from LP-WUS mode to legacy PDCCH monitoring, gNB needs to send a switching command to UE.

LP-WUS activation/deactivation

- gNB can configure multiple LP-WUS configurations to the UE, and activate one or more of the configurations to the UE via MAC CE.

LP-WUS in DC/CA

- For a UE configured with DC, support LP-WUS monitoring on a serving cell per cell-group.

- For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells.

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group.

  - Opt.1: LP-WUS is configured to be monitored per DRX group.

    - For dual DRX groups, LP-WUS monitoring is configured per DRX group.

    - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group.

  - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)