3GPP TSG-RAN WG2 Meeting #127bis	R2-2408766

Hefei, China, Oct 14th – 18th, 2024

Source:                Qualcomm Incorporated

Title:	MRO enhancement for SON and MDT

Agenda Item:	8.10.2

Document for:	Discussion and Decision

# Introduction & Background

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## 2.1 MRO for LTM

### Too early LTM/too late LTM/LTM to wrong

In last RAN2 meeting, there were some agreements on the scenarios to be supported for LTM SON/MDT.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

For too late LTM, the main issue is that an RLF occurs after the UE has stayed for a long period of time in the source cell; the UE attempts to re-establish the radio link connection in a different cell. Then for RLF report, UE should report the following information corresponding to different cases.

For Case 1a, UE should include source cell ID in which RLF is detected and cell ID for re-established cell. Then the network should have the information about the failure situation, e.g. on which cell the RLF is detected, and on which cell the RLF is recovered.

For Case 1b, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which HOF is detected.

For Case 1c, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

- For too late LTM, UE should report the following information in RLF report.

    - For Case 1a, UE should include source cell ID in which RLF is detected and cell ID for re-established cell.

    - For Case 1b, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which HOF is detected.

    - For Case 1c, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

For too early LTM or LTM to wrong cell, the main issue is RLF or HOF may happen shortly after a successful LTM, or a failure occurs during the LTM procedure.

- For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

For LTM to wrong cell, based on the following agreed cases:

- LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

For Case 2a/3a, UE should include target cell ID in which HOF or RLF is detected and cell ID for re-established cell. Then the network should have the information about the failure situation, e.g. whether the HOF or RLF is detected in the target LTM cell.

For Case 2b/3b, UE should include target cell ID in which HOF or RLF is detected and HOF in the selected LTM candidate cell and reestablished cell ID.

For Case 2c/3c, UE should include target cell ID in which HOF or RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

Additionally, for case 2b/3b and 2c/3c, it also makes sense to indicate the time between UE receives LTM command and HOF/RLF detection, as shown T2 in following figures, so that network will have the knowledge on how shortly the UE detects RLF after receiving LTM command.

- For too late LTM and LTM to wrong cell, UE should report the following information in RLF report.

    - For Case 2a/3a, UE should include target cell ID in which HOF or RLF is detected and cell ID for re-established cell. Then the network should have the information about the failure situation, e.g. whether the HOF or RLF is detected in the target LTM cell.

    - For Case 2b/3b, UE should include target cell ID in which HOF or RLF is detected and HOF in the selected LTM candidate cell and reestablished cell ID.

    - For Case 2c/3c, UE should include target cell ID in which HOF or RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

    - Additionally, UE should include the time between UE receives LTM command and HOF/RLF detection.

### RACH information in RLF Report

Currently UE includes RACH information (ra-InformationCommon) in RLF Report if connectionFailureType is hof and if the failed handover is an intra-RAT handover. But LTM can be RACH-based or RACH-less. Therefore, the RACH information should be only included in case of RACH-based LTM and in case of RACH-less HO, UE should omit RACH information in RLF Report during RACH-less LTM and further can add an explicit indicator that the LTM was RACH-less.

- UE can include existing RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-based LTM failure.

In the previous meeting, some companies proposed to discuss RACH-less LTM to RACH-based LTM “fall back” scenarios. There could be two cases that the RACH-less LTM is not successful:

Case 1: The UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure.

Case 2: The RACH-less LTM is initiated (because of UE having a valid TA either received from LTM CSC or via UE-based measurement) but the RACH-less LTM cell switch execution is not successful e.g., because UE couldn’t determine that the network has successfully received its first UL data. In such a case, T304 could expire.

Upon T304 expiry, UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed. Whatever the LTM recovery is configured or not, is executed successfully or not, it is beneficial the UE indicates to gNB which case results in RACH-less LTM failure.

- UE indicates to gNB in RLF Report which case results in RACH-less LTM failure

Case 1: The UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful.

Case 2: The RACH-less LTM is initiated but the RACH-less LTM cell switch execution is not successful.

### Successful LTM scenarios (SHR enhancements)

In the last RAN2 meeting , the following agreements was reached.

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

RA-based access will not be a new triggering condition for the SHR report.

Based on the above RAN2 agreement, the following scenarios are covered:

- RACH-less LTM fails  Indication in RLF Report that this LTM was RACH-less

- RACH-based LTM fails  Indication in RLF Report that this LTM was RACH-based

We can include a similar “access type” (RACH-less or RACH-based) indicator in successful LTM scenarios as well (i.e., enhance SHR to include access type):

- Successful RACH-less LTM  Indication in SHR that this LTM was RACH-less

- Successful RACH-based LTM  Indication in SHR that this LTM was RACH-based

- Include an indicator in SHR whether the successful LTM execution was RACH-less or RACH-based

### Optimizing early TA acquisition procedure

In the previous meetings, there were several proposals to optimize aspects related to early TA acquisition procedure.

There were few proposals for the UE to report whether the UE used UE-based TA measurement, or whether it used the network-provided TA value in LTM CSC.

During LTM preparation, source gNB provides the UE with Early UL Sync configuration (obtained from candidate cells) and whether UE-based TA measurement should be performed for each LTM candidate cell. Also, the source gNB-DU knows that it has sent a PDCCH order to trigger Early UL sync procedure. The network can store this Early UL sync configuration and whether it has provided a PDCCH order and there is no need for UE to report it in SON reports. But this needs to be discussed in RAN3 firstly. Then it is preferred to wait for RAN3 progress on whether whether network can store this Early UL sync configuration and whether it has provided a PDCCH order.

- It is proposed wait for RAN3 progress on whether network can store this Early UL sync configuration and whether it has provided a PDCCH order.

### Optimizing LTM candidate cells at gNB-CU

In last RAN2 meeting, RAN2 agreed to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

In Rel-17, we similarly discussed how to optimize the CHO candidate cell list and there were 2 options discussed - a network-based solution and a UE based solution. After a contentious discussion, both solutions were agreed, albeit for different sub cases:

- UE based solution (for the case of CHO execution failure): UE reports the CHO candidate cell list in RLF Report, in case of CHO execution failure.

- Network based solution (for the case of RLF shortly after a successful CHO execution): Source gNB provides the CHO configuration (which includes CHO candidate cell list and CHO execution conditions) to target gNB in SN STATUS TRANSFER. Target gNB stores it and provides it back to source gNB in HANDOVER REPORT

In Rel-17, the UE based solution was agreed first, but then we realized that the UE releases the CHO configuration after a successful CHO execution and therefore UE based solution can’t work in the case that there is a RLF shortly after the successful CHO execution. Therefore, the network-based solution was also agreed.

LTM is different from CHO in that we support subsequent LTM and therefore UE stores the LTM configuration even after a successful LTM execution. Also, Rel-18 only supports intra-CU LTM and therefore the gNB-CU can have the LTM candidate cells in its UE context even if there is inter-DU mobility within the gNB-CU.

In Rel-19 MRO for LTM, we should avoid having two solutions (i.e., both UE based and network-based solution) for optimizing LTM candidate cell list. Also existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

We propose to use a network-based solution as follows:

- There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

### UHI/MHR enhancements for LTM

A UE supporting LTM might undergo both LTM and normal L3 HO scenarios depending on the gNB implementation. A gNB might do a back-to-back L3 HO and LTM due to L3 execution conditions being met in gNB-CU and LTM execution condition met in gNB-DU.

Therefore, it useful for the gNB to know whether the UE’s visited cells were due to LTM or L3 HO. Without such knowledge, the gNB might unnecessarily be trying to optimize ping-pong handovers even though they happened due to different reasons (i.e., L3 HO and LTM). It is beneficial to optimize the UHI to include a HO-type (i.e., LTM or L3 HO) similar to how it has been agreed by RAN2 for RLF Report.

- Enhance UHI and/or MHR to include a “HO-type” (i.e., LTM or L3 HO).

## 2.2 MRO for Subsequent CPAC (S-CPAC)

A subsequent CPAC is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA.

The UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the execution conditions of candidate PSCells after completion of a PSCell addition, a PSCell change, a PCell change or an SCG release.

In last RAN2 meeting, RAN2 agreed that RAN2 will look into failure and near failure scenarios.

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

The S-CPAC configuration contains

- candidate SCG configuration(s) of candidate PSCell(s)

- execution conditions

- MCG configuration (to be applied when subsequent CPAC execution is triggered)

- reference configuration

- security update configuration

In case a UE configured with S-CPAC configuration encounters an SCG failure, it sends a SCGFailureInformation to MN (including the measurement results) and waits for MN’s subsequent action e.g., whether it wants UE to add a new PSCell or keep it in standalone mode

After the last CPC/CPA execution, SCG Failure occurs as the execution condition for subsequent CPA/CPC does not meet; measurements shows that appropriate cell was not configured for subsequent CPA/CPC. For example (in Figure 5), successful S-CPAC is executed from A0 to A1. While UE is connected to A1, SCG Failure occurs as none of the execution condition for S-CPAC is met; measurements shows that appropriate candidate cell was not configured for subsequent CPA/CPC. measurement report shows PSCell B3 (not configured for subsequent CPA/CPC for PSCell A1) was the strongest cell.

Figure 5

Then it is worthy to discuss the optimization of the S-CPAC configuration (e.g. wrong sequence of candidate cells).

- Existing measurement information in SCGFailureInformation can be used to identify the best cell, so that it can be added into the candidate subsequent PSCell.

## 2.3 MRO for CHO with candidate SCG

On MRO for CHO with candidate SCG, there could be some scenarios to be considered as following.

Scenario 1: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and execution conditions of no candidate PCell and no candidate PSCells were met before the RLF or SCG failure

Scenario 2: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and one execution condition was “blocking” the whole execution e.g.,

- A candidate PCell (A1 in the example below) satisfies the execution condition but none of its associated candidate PSCells (i.e., B1 or B2) satisfy the corresponding execution condition.

- A candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

In scenarios 1 and 2, it might be useful for the network to know why the CHO with candidate SCG execution e.g., it didn’t happen because of one “blocking” execution condition (even though the other execution condition was met) or because none of the execution conditions were met

Scenario 3: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and non-associated PCells and PSCells met the execution condition

In the example below, Execution condition of candidate PCell A1 and candidate PSCell B3 is met but the CHO with candidate SCG execution didn’t happen because there is no (A1, B3) associated configuration.

It would be beneficial if the network can know what execution conditions were met (i.e., A1 and B3) so that the network can fine-tune the CHO with candidate SCG configuration eg., by including a (A1, B3) configuration or (A2, B3) confguration to avoid RLF or SCG failures in the future.

RAN2 had the following agreements in the last meeting.

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

Considering the above example scenarios, it is proposed further the following:

- Enhance RLF Report and SCGFailureInformation with:

    - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

    - Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

    - Time elapsed between the fulfilled execution and RLF for CHO and/or CPC.

    - Time elapsed between the two fulfilled execution conditions (CHO and CPAC)

# Conclusion

MRO for LTM

- For too late LTM, UE should report the following information in RLF report.

    - For Case 1a, UE should include source cell ID in which RLF is detected and cell ID for re-established cell.

    - For Case 1b, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which HOF is detected.

    - For Case 1c, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

- For too late LTM and LTM to wrong cell, UE should report the following information in RLF report.

    - For Case 2a/3a, UE should include target cell ID in which HOF or RLF is detected and cell ID for re-established cell. Then the network should have the information about the failure situation, e.g. whether the HOF or RLF is detected in the target LTM cell.

    - For Case 2b/3b, UE should include target cell ID in which HOF or RLF is detected and HOF in the selected LTM candidate cell and reestablished cell ID.

    - For Case 2c/3c, UE should include target cell ID in which HOF or RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

    - Additionally, UE should include the time between UE receives LTM command and HOF/RLF detection.

- UE can include existing RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-based LTM failure.

- UE indicates to gNB in RLF Report which case results in RACH-less LTM failure

Case 1: The UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful.

Case 2: The RACH-less LTM is initiated but the RACH-less LTM cell switch execution is not successful.

- Include an indicator in SHR whether the successful LTM execution was RACH-less or RACH-based

- It is proposed wait for RAN3 progress on whether network can store this Early UL sync configuration and whether it has provided a PDCCH order.

- There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

- Enhance UHI and/or MHR to include a “HO-type” (i.e., LTM or L3 HO).

MRO for Subsequent CPAC (S-CPAC)

- Existing measurement information in SCGFailureInformation can be used to identify the best cell, so that it can be added into the candidate subsequent PSCell.

MRO for CHO with candidate SCG

- Enhance RLF Report and SCGFailureInformation with:

    - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

    - Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

    - Time elapsed between the fulfilled execution and RLF for CHO and/or CPC.

    - Time elapsed between the two fulfilled execution conditions (CHO and CPAC)

# Reference