3GPP TSG-RAN WG2 Meeting #127bis	R2-2408767

Hefei, China, Oct 14th – 18th, 2024

Source:                Qualcomm Incorporated

Title:	SON and MDT for SDT

Agenda Item:	8.10.5

Document for:	Discussion and Decision

# Introduction & Background

Rel-19 SON/MDT agreed to continue Rel-18 leftovers as following, and this contribution discuss the related issues.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]:

•	RACH optimization for SDT

•	MHI Enhancement for SCG Deactivation/Activation

•	MRO for MR-DC SCG failure

# Discussion

## 2.1 RACH optimization for SDT

In the last RAN2 meeting, RAN2 had some discussion on RACH optimization for SDT and achieved the following agreement.

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

For the failure cause, currently there are some failure causes defined for the SDT failure, including e.g. the maximum number of retransmissions has been; if random access problem indication is received from MCG MAC while SDT procedure is ongoing; or cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired; integrity check failure indication, or T319a expires. It would be beneficial to indicate the failure case to the gNB and gNB can optimize SDT configuration.

- RAN2 discusses the following cause failure included in RACH report for SDT.

•	Maximum number of retransmissions has been attempted;

•	If random access problem indication is received from MCG MAC while SDT procedure is ongoing; or

•	Integrity check failure indication, or

•	T319a expires.

The failure cause of cg-SDT-TimeAlignmentTimer or configuredGrantTimer is used for CG based SDT, it is also beneficial to report to network when the timers expire. But this is not related to RACH optimization for SDT.

- RAN2 discusses whether to optimize CG based SDT via e.g. reporting the timer cg-SDT-TimeAlignmentTimer or configuredGrantTimer expires.

# Conclusion

- RAN2 discusses the following cause failure included in RACH report for SDT.

•	Maximum number of retransmissions has been attempted;

•	If random access problem indication is received from MCG MAC while SDT procedure is ongoing; or

•	Integrity check failure indication, or

•	T319a expires.

- RAN2 discusses whether to optimize CG based SDT via e.g. reporting the timer cg-SDT-TimeAlignmentTimer or configuredGrantTimer expires.

# Reference