3GPP TSG-RAN WG2 Meeting #127bis			         R2-2408770

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.2

Source: 	LG Electronics Inc.

Title:         	On-demand SSB SCell operation

Document for: 	Discussion and Decision

Introduction

RAN1 has made some progress on the on-demand SSB operation and shared it with RAN2 through the LS [1]. In this contribution, we will analyse the impact of the on-demand SSB on L3 measurement reporting and propose MAC CE signalling aligned with the RAN1 progress.

Discussion

According to TS38.331, whenever the UE has a measurement configuration, the UE shall perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured, and include the measurement results of the serving cell in every measurement report message.

In the previous meeting, RAN2 agreed for Case #1 (no always-on SSB on the cell) that UE does not expect to measure SSB when on-demand SSB transmission is deactivated. If servingCellMO is configured for the OD-SSB SCell, the RAN2 agreement conflicts with the current UE behaviour. To resolve the contradiction between the current UE behaviour and the RAN2 agreement, two solutions can be considered as follows:

- Option 1. Do not configure servingCellMO for OD-SSB SCell

- Option 2. Change the procedural text to meet the RAN2’s agreement

If servingCellMO is not configured for OD-SSB SCell, the measurement results of the OD-SSB SCell will not be reported even when the OD-SSB transmission is activated, unless the measurement reporting condition associated with OD-SSB SCell is met. Since the SCell requires tight management though the measurement results frequently reported by UE, option 1 does not seem acceptable. Therefore, it seems more desirable to modify the procedural text related to the servingCellMO.

UE is not required to include the SSB based measurement results of the SCell configured with servingCellMO in the measurement report, if the SSB based measurement results of the SCell are not available. Therefore, even if the SSB transmission is deactivated, it will not impact the L3 measurement reporting.

Proposal 1	Even if the servingCellMO is configured for the OD-SSB SCell, UE does not derive the SSB based measurement results of the OD-SSB SCell when the on-demand SSB transmission is deactivated.

Based on the progress of RAN1, RAN2 made following agreement in the last meeting:

RAN2#127bis:

New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

At least for scenario 2, a new MAC CE needs to be defined for OD-SSB transmission indication, as RAN2 already agreed. The same MAC CE also can be used in scenario 2A. From MAC CE signalling perspective, there is no reason to distinguish scenario 2 and 2A. The only different is that the network needs to simultaneously transmit two MAC CEs, i.e. one for SCell activation indication and another for SSB transmission indication, in scenario 2A. Therefore, no optimization is needed for scenario 2A and we propose to introduce a single new MAC CE for OD-SSB transmission indication, which can be used in all scenarios.

Proposal 2	Introduce a single new MAC CE which indicates OD-SSB transmission and relevant parameters, which can be applied to both scenario #2 and #2A.

Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

According to [1], the new MAC should be able to indicate at least the activation of the on-demand SSB transmission and the periodicity of the on-demand SSB transmission. RAN1 also noted that any other relevant parameters are FFS and the deactivation of the on-demand SSB transmission can be separately discussed. Therefore, it is not possible to design a complete MAC CE at this point, and we need to wait for further inputs from RAN1. We propose just to confirm that the new MAC CE can indicate at least the activation of the on-demand SSB transmission and the periodicity of the on-demand SSB transmission as requested by RAN1.

Proposal 3	RAN2 confirms that the new MAC CE can indicate at least the activation of the on-demand SSB transmission and the periodicity of the on-demand SSB transmission as requested by RAN1.

Conclusion

Proposal 1	Even if the servingCellMO is configured for the OD-SSB SCell, UE does not derive the SSB based measurement results of the OD-SSB SCell when the on-demand SSB transmission is deactivated.

Proposal 2	Introduce a single new MAC CE which indicates OD-SSB transmission and relevant parameters, which can be applied to both scenario #2 and #2A.

Proposal 3	RAN2 confirms that the new MAC CE can indicate at least the activation of the on-demand SSB transmission and the periodicity of the on-demand SSB transmission as requested by RAN1.

Reference

[1] R1-2407438	LS to RAN2 for on-demand SSB SCell operation