3GPP TSG-RAN WG2 Meeting #127bis	R2-2408772

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.3

Source:	OPPO

Title:	Consideration on on-demand SIB1

Document for:	Discussion, Decision

Introduction

The revised Rel-19 NES WID has been approved with the following objective.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

This paper will outline our opinions on on-demand SIB1 for UEs in RRC idle/inactive mode.

Discussion

The WUS configuration

RAN2 has agreed that Cell A is obliged to provide the UL WUS configuration for multiple NES Cells through its SIB. When the Rel-19 UE camps on the NES cell, it can receive the UL WUS configuration update from that camping NES cell. This question is whether the camping NES cell only provides its own WUS configuration or if it can also provide the WUS configuration of neighbouring cells.

Further clarification 6. Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells: - More than one Cell A may provide configuration for the same NES cell. - The same Cell A may assist more than one NES Cells. Transmission of WUS configuration - Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB. WUS configuration of NES cell from NES cell: 4. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

In order to prevent the NES UE from reselecting Cell A again to re-acquire the UL WUS configuration of the NES cell, the NES cell should provide the UL WUS configuration for neighbouring cells as well. If this is not done, the UE may fail to acquire the SIB1 of the neighbouring NES cell and eventually go back to Cell A if the stored UL WUS configuration of this neighbouring NES cell is outdated when the UE is camped on the current NES cell.

The NES cell provides the UL WUS configuration of its own as well as that of its neighbour NES cells.

Another question is whether the UL WUS configuration is provided via the new SIB or the existing SIB. In RAN1#117, a unified configuration format is recommended for the WUS configuration design. RAN1 assumes that the WUS configuration applies either to multiple NES cells or a single NES cell.

Agreement From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it. At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

In the existing RRC spec, SIB1 contains the essential access information for the current cell, while SIB2/3/4 contains the cell re-selection parameters for frequency as well as cell-specific re-selection parameters. When considering the UL WUS configuration, it should include the information for neighbouring cells and/or its own, and the design of the configuration format needs to accommodate both frequency/cell-specific and frequency/cell-agnostic cases. It may not be a good choice to extend the existing SIB to encompass this. On the other hand, fitting the entire UL WUS configuration into one existing SIB is difficult as the maximum payload size of the existing SIB is less than 3k bits. Therefore, it is recommended to introduce a new SIB to contain the UL WUS configuration, regardless of the cell being Cell A or NES Cell.

NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

Introduce a new SIB to contain the UL WUS configuration for NES Cell.

In RAN2#126, it has been concluded that the RRC release message can be an option for WUS configuration provision.

7. RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

In legacy, the RRC release message is used to provide the resource configuration used for the idle or inactive UEs for the cell re-selection procedure. The cell reselection-related information remains valid until a specific timer expires. After that, the UE applies the cell reselection-related information that has been broadcasted. This principle can also be applied to the UL WUS configuration provided via the RRC release message, which helps reduce the signalling overhead caused by periodic SIB transmission. In this case, the UE applies the UL WUS configuration provided via the RRC release message before a proper timer expires and applies the UL WUS configuration provided via SIB later.

The UL WUS configuration can also be provided via the RRCRelease message.

In the previous discussion, RAN2 concluded that the UE triggers WUS transmission if it chooses the NES cell using the legacy intra-F/inter-F cell re-selection procedure. The undecided issue is whether to introduce a new RSRP triggering condition/threshold for the NES UE triggering WUS.

Triggering condition of WUS transmission 11. If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

We are unsure about introducing a separate condition/threshold for WUS triggering beyond the existing criteria of the cell reselection, as it could complicate the UE behaviour and cell reselection procedure. For example, if the threshold for WUS triggering is met but that for the cell reselection is not met eventually, early WUS triggering would waste the system energy. Conversely, if the threshold for WUS triggering is met after that for cell reselection criteria is met, the reselection to an NES cell would be delayed.

For an on-demand SIB1 cell, when fulfilling legacy cell re-selection criteria based on the current reselection parameters, the UE requests OD-SIB1 by sending WUS and then camps on that cell (if SIB1 is acquired and the cell is suitable).

Legacy UE impact

In the SI phase, RAN2 discussed the impact on the legacy UE and reached the following agreements. As Option 3 can only be used for cell reselection, Option 1/2 should be the starting point to prevent the legacy UE from selecting or reselecting the NES Cell.

Legacy UE impact 14. NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism) 15. How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that). Access restriction for legacy UEs: 6. Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Option 1 shares a similar principle/logic as Rel-18 NES barring bit. Compared to Option 1, Option 2 requires an always-invalid Kssb value to be broadcasted by the MIB of the NES Cell. Otherwise, the legacy UE would access the NES cell, encounter a temporarily transmitted SIB1, and then face an abnormal situation when the SIB1 disappears suddenly. The case occurs under the following assumption: The NES UE camping on Cell A knows the valid Kssb of an NES cell from the UL WUS configuration, rather than from the MIB of this NES cell since there is no SIB1 indication in the MIB of this NES cell for the purpose of preventing the legacy UE from accessing this NES cell. After the NES UE camps on this NES cell, the NES cell may notify the UE of the value change of Kssb via the legacy SI modification procedure. At this time, if the NES cell does not change invalid Kssb to valid Kssb in its MIB, the NES UE may fail to obtain the updated SI message. However, if the NES cell changes invalid Kssb to valid Kssb in its MIB, the legacy UE would be allowed to access unexpectedly. Thus, Option 1 is preferred from our perspective.

For cell (re)selection, use cellbarred in MIB to prevent the legacy UE from accessing the on-demand SIB1 cell.

For cell (re)selection, similar to Rel-18 NES, on-demand SIB1 capable UE uses 1-bit new indication in SIB1 for barring control.

Conclusion

We have the following proposals:

Proposal 1	The NES cell provides the UL WUS configuration of its own as well as that of its neighbour NES cells.

Proposal 2	Introduce a new SIB to contain the UL WUS configuration for NES Cell.

Proposal 3	The UL WUS configuration can also be provided via the RRCRelease message.

Proposal 4	For an on-demand SIB1 cell, when fulfilling legacy cell re-selection criteria based on the current reselection parameters, the UE requests OD-SIB1 by sending WUS and then camps on that cell (if SIB1 is acquired and the cell is suitable).

Proposal 5	For cell (re)selection, use cellbarred in MIB to prevent the legacy UE from accessing the on-demand SIB1 cell.

Proposal 6	For cell (re)selection, similar to Rel-18 NES, on-demand SIB1 capable UE uses 1-bit new indication in SIB1 for barring control.

Reference

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR