3GPP TSG-RAN WG2 Meeting #127bis	R2-2408782

Hefei, China, Oct 14-18, 2024

Agenda item:	8.7.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on reply LS to SA2 on multi-modality awareness

Document for:	Discussion and Decision

1. Introduction

During RAN2#126 meeting, RAN2 has sent an LS on multi-modality awareness to SA2 in R2-2405782 [1]. SA2 has provided the feedback in S2-2409444 [2]. In the reply LS, SA2 asked RAN2 to further clarify the usage of the multi-modality information. In this contribution, we discuss what feedback to provide to SA2 based on RAN2 agreements and RAN WID objectives.

2. Discussion

In S2-2409444, SA2 replied that it is technically feasible to provide MMSID to RAN. But before deciding whether to provide it, SA2 would like RAN to clarify the usage of the multi-modality information at RAN.

SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information.

Actually, RAN2 has already discussed about the potential usages during the previous meetings. Some agreements can be referred to provide the feedback to SA2.

For MMSID, it was agreed in RAN2#127 meeting that the it can be used for joint admission control and QoS flow to DRB mapping. The agreements can be found below.

RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

Observation 1: RAN2 has agreed MMSID can be used for joint admission control and QoS flow to DRB mapping.

Furthermore, RAN2 also consider whether it is possible to perform enhancements for synchronization or PDU set discard based on the multi-modality information, as shown by the following agreements from RAN2#127 meeting. For these usages, more information other than MMSID is required, e.g. the synchronization threshold and/or the inter-PDU set dependency across QoS flows.

For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

Whether the above synchronization threshold and inter-PDU set dependency information can be obtained needs to be further evaluated by SA2 and SA4. It is also noted in the updated WID objective for multi-modality, as shown below.

- Specify support for multimodality in RAN for UL and DL [RAN3]: NOTE: This is subject to alignment with SA2, e.g., if MMSID is not available from CN, then UE assistance information-based approach as an alternative. RAN#106 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE.

Observation 2: Multi-modality information is also considered to be used for traffic synchronization and PDU set discard if information such as synchronization threshold and inter-PDU set dependency can be provided. SA2/SA4 may further check whether it is possible.

From RAN2 perspective, the above agreements are enough to clarify the potential usage for multi-modality information. To provide the reply LS to SA2, RAN2 can simply include the agreements achieved at the previous RAN2 meeting.

Proposal 1: RAN2 to reply to SA2 about the previous agreements on the usages of multi-modality information, including:

- MMSID can be used for joint admission control and QoS flow to DRB mapping.

- Multi-modality information is considered to be used for traffic synchronization and PDU set discard if it is confirmed by SA2/SA4 that information such as synchronization threshold and inter-PDU set dependency can be available from CN and/or at the UE.

3. Conclusion

In this contribution, we discussed on how to reply SA2 on the usage of multi-modality information. The following proposal was made:

Proposal 1: RAN2 to reply to SA2 about the previous agreements on the usages of multi-modality information, including:

- MMSID can be used for joint admission control and QoS flow to DRB mapping.

- Multi-modality information is considered to be used for traffic synchronization and PDU set discard if it is confirmed by SA2/SA4 that information such as synchronization threshold and inter-PDU set dependency can be available from CN and/or at the UE.

4. Reference

[1]. R2-2405782, TSG RAN WG2, LS on multi-modality awareness at RAN.

[2]. S2-2409444, TSG SA WG2, Reply LS on multi-modality awareness at RAN.

Annex. [Draft] Reply to SA2 LS on multi-modality awareness

3GPP TSG-RAN WG2 Meeting #127bis	   R2-24xxxxx

Hefei, China, Oct 14-18, 2024

Title:	[Draft] Reply to SA2 LS on multi-modality awareness

Response to:	S2-2409444

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	Huawei, Hisilicon (to be RAN2)

To:	SA2, SA4, RAN3

Cc: 	RAN

# Contact Person:

# Name:	Yinghao Guo

# Email:		yinghaoguo AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN2 thanks SA2 for the feedback on multi-modality awareness.

For the usage of multi-modality information, RAN2 has discussed and reached the following agreements during the RAN2#127.

First, it was agreed that the multi-modality information, such as MMSID, can be used for joint admission control and QoS flow to DRB mapping.

RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

Furthermore, RAN2 also consider whether it is possible to perform enhancements for synchronization or PDU set discard based on the multi-modality information, such as the synchronization threshold and/or the inter-PDU set dependency across QoS flows.

For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

As shown above, RAN2 also thinks that SA2/SA4 may further evaluate whether the synchronization information can be available.

# 2. Actions:

# To SA2, SA4, RAN3:

ACTION:

1. RAN2 kindly asks SA2 to take the above agreements on multi-modality awareness into consideration and provide further feedback on the availability of the required information (MMSID, synchronization threshold) from CN.

2. RAN2 kindly asks SA4 to clarify whether synchronization information can be available at the UE.

3. RAN2 kindly ask RAN3 to and SA4 to provide any further feedback on the agreements from RAN2, as requested by SA2.

# 3. Date of Next RAN2 Meetings:

RAN2#128	18th Nov – 22nd Nov 2024				Orlando, US

RAN2#129	17th Feb – 21st Feb 2025				Athens, Greece