3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408783

Hefei, China, 14 - 28 October 2024

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon

Title:	 Discussion on event triggered measurement report

Document for:  Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18. The network receives L1 measurement report to determine the LTM target cell and target beam. Three types of report are defined, namely, periodic, aperiodic and semi-persistent L1 report.

For R19 mobility enhancements, one of the goals outlined in the WID is to define the required improvements for enabling event-triggered L1 measurements.

RP-242356, Revised Work Item: NR mobility enhancements Phase 4 - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

In this contribution, we present a detailed analysis of event-triggered L1 measurement reports. This includes the event triggering condition, the specification modelling of event-triggered L1 reporting, the configuration framework and the reporting after event triggering.

2	Discussion

2.1	Event triggering condition

2.1.1	TTT mechanism

Agreements on measurement enhancements for LTM: 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In RAN2 #126 meeting, TTT mechanism should be applied for the LTM L1 event evaluation procedure.

Agreements on L1 MR event evaluation 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation.

Beam in serving cell

At the last meeting, it was agreed that the beam of the serving cell is the beam corresponding to the indicated TCI state, referred to as the "current beam", which is used for event evaluation in L1 measurement reporting. The current beam may change due to the beam management processes so it is necessary to consider whether the TTT should be restarted when the current beam changes. We consider two options to address this:

- Option-1: If the current beam changes and the entering condition is still met with the new current beam, the TTT continues to run without being restarted.

- Option-2: If the current beam changes, the TTT is restarted.

The beam management process is expected to select the best beam, so the new current beam is better than the old current beam. The entering condition of all events for L1 event-triggered reporting is that the current beam is below some absolute or some relative level, so if such a condition is met with the new current beam, it is also met with the previous current beam. Option-1 is appropriate and there is no reason to restart the TTT like in Option-2.

Proposal 1: TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

Beam in the candidate cell

Similarly, we need to discuss whether the TTT should be restarted when the beam (any beam) used for LTM event evaluation changes. Two options are identified to address this:

Start the TTT

LTM resource set {SSB1, SSB2. SSB3}  √means the beam satisfies the event condition

SSB3√

SSB2√

SSB1√

SSB1√

- Option-1: If the beam (any beam) used for LTM event evaluation changes and the entering condition is still met with the new beam, the TTT continues to run without being restarted.

Restart the TTT

Start the TTT

LTM resource set {SSB1, SSB2. SSB3}  √means the beam satisfies the event condition

SSB2√

SSB2√

SSB1√

SSB1√

- Option-2: If the beam (any beam) used for LTM event evaluation changes, the TTT is restarted.

Unlike the previous case, it is possible that the entering condition is not met anymore with the previous beam of the candidate cell. However, if the event would be triggered based on cell results and the number of beams used to derive the cell results would be 1, the TTT would continue running, so we think that it should be the same here i.e., Option-1.

Proposal 2: TTT timer should not be restarted if the beam in the candidate cell used for event evaluation changes and the new beam still meets the entering condition.

In the R18 LTM framework, an LTM resource set is associated with an LTM report Config and an event is often associated with an LTM report Config as well. There are also two options for the application of the TTT.

- Option-1: TTT can be per cell: TTT continues to run if any RS/beam in one candidate cell meets the event condition.

- Option-2: TTT can be per LTM resource set (event/report Config): TTT continues to run if any RS/beam in one LTM resource set meets the event condition.

One advantage of option-2 is that for TTT per LTM resource set (event/report Config), the RS/beam can belong to either a single cell or multiple cells. This allowed for greater flexibility, as compared to applying TTT per cell. Additionally, in the legacy RRC event evaluation procedure, the TTT is configured per event/report Config. The TTT per LTM resource set (event/report Config) offers a more simplified design for specification.

Proposal 3: TTT timer is maintained per event/report Config.

2.2	Specification modelling for event-triggered L1 reporting

2.2.1	Unified event for evaluation procedure

In RAN1#117 Rel-19 MIMO discussions, the following has been agreed for the event evaluation: MIMO measurement report is triggered when the condition is satisfied more than M times during a time window.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

The MIMO agreement for triggering a report includes considerations of time windows and of a number of occurrences. This mechanism is currently not specified in L1 specifications yet.

Observation 1: The event triggering evaluation agreed by RAN1 for MIMO involves time windows and counters.

Agreements on measurement enhancements for LTM: 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In RAN2#126 meeting, LTM agreed that the TTT mechanism is used for the event evaluation procedure.

Technically, the TTT mechanism in LTM is similar to the mechanism of the time window and M times in MIMO. Thus, the mechanism in MIMO can be applied in LTM event evaluation procedure. In order to simplify the specification, it would be advantageous to unify the LTM and MIMO evaluation procedures.

Proposal 4: MIMO and LTM event evaluation can be unified under the same UE procedure in MAC spec.

2.2.2	Evaluation of the entering and leaving conditions

In the last meeting, it has been agreed that the MAC layer handles the event evaluation procedure.

Agreements on L1 MR event evaluation 6. MAC layer handles the event evaluation and measurement report triggering.

Considering about the event-triggering procedures for LTM, we see two possible specification approaches for evaluating the entry/leaving condition:

-	Option-1: L1 checks the entry/leaving condition for the events and indicates to the MAC when a condition is satisfied. The MAC determine whether to trigger a report:

-	Option-2: L1 report results to upper layer, the MAC evaluates the entering and leaving conditions for the events and determine whether to trigger a report.

For Option-1, if the comparison occurs in the PHY layer, it sends the instance to the higher layers. Alternatively, for Option-2, if conducted in the higher layers, the PHY layer sends the beam quality metrics, such as L1-RSRP, to the higher layers, allowing them to perform the comparison based on beam quality.

Between these two approaches, we consider it t more appropriate for the L1 layer to check whether the entering/leaving condition is satisfied/not satisfied, as the candidate beam or the currently used beam is known to the PHY, but transparent to the MAC layer. The primary reason to against the option-2 is that if the L1 sends the beam quality metrics to the MAC, it needs more layer interaction, thereby complicating the process.

Proposal 5: The entering/leaving conditions for events with beam-level measurement as triggering quantity are evaluated in L1. After evaluation, L1 indicates to higher layer whether the condition is satisfied/not satisfied.

2.3	Configuration framework

2.3.1	Configuration of event reporting

In R18 LTM, we use the following Fig.1 to illustrate the structure of configuration for measurement report

Figure 1: General framework for LTM resource and report configuration

For the LTM-CSI-ResourceConfig, each configuration is associated with a unique ltm-CSI-ResourceConfigId. Similarly, the LTM-CSI-ReportConfig also contains the ltm-CSI-ResourceConfigId, which works as the link between the LTM-CSI-ResourceConfig and the LTM-CSI-ReportConfig. This structure implies that a single LTM-CSI-ResourceConfig can only be associated with one LTM-CSI-ReportConfig.

If multiple LTM-CSI-ResourceConfig need to be connected to a single LTM-CSI-ReportConfig, the LTM-CSI-ReportConfig must be repeated for each individual LTM-CSI-ResourceConfig. Likewise, if multiple LTM-CSI-ReportConfig need to connect to the same LTM-CSI-ResourceConfig, the same duplication occurs, since multiple LTM-CSI-ResourceConfig must be created for each LTM-CSI-ReportConfig. This results in significant redundancy in the RRC configuration.

Observation 2: The legacy R18 LTM configuration framework leads to repetitive configurations.

If the legacy R18 LTM configuration framework is reused for event reporting, the number of LTM-CSI-ResourceConfig and LTM-CSI-ReportConfig entries will become massive. In order to achieve a signalling optimization, we can introduce the measID.

Another reason to support measID is outlined as following. In periodic, aperiodic, or semi-persistent reporting, the network is able to identify which the report is based by associating the report content with the uplink resources through which the measurement result is received.

In event reporting, the CSI/SSB resources of the report cannot be deduced based on uplink resources. A measID can be used to indicate this information. If the measID is not used, the UE needs to report the candidate cell Reference Signal and RS set ID.

It functions the same as the legacy R15 measurement report configuration, which is illustrated as the Fig.2.

measID 1

reportConfigID 1

reportConfigList

measObjectID 1

measObjectList

measIDList

reportConfigID 2

measID 2

measObjectID 2

reportConfigID 3

measID 3

measObjectID 3

reportConfigID 4

measID 4

measObjectID 4

Figure 2: General framework for R15 RRC Measurement Reporting

Proposal 6: Support a measID in configuration of event-triggered L1/2 reports to optimize signalling overhead.

2.3.2	Configuration of SR

Additionally, the SR mechanism for the event-triggered reporting procedure should be discussed. When no UL grant is available at the time of event reporting, the UE should send an SR to trigger the event reporting procedure. Under the current mechanism, SR for event-triggered reporting cannot be initiated in BSR-SR mode. Therefore, similar to the BFR MAC CE mechanism, the event-triggered reporting should also have MAC CE to trigger the SR.

Proposal 7: When there is no UL-SCH resource to transmit the triggered MAC CE for event-triggered measurement report, trigger SR.

Besides, we need to consider the configuration of the SR, it should either configured per LTM-CSI-ReportConfig or as a dedicated SR for the LTM event reporting procedure. For the SR per LTM-CSI-ReportConfig, it requires many PUCCH resources for SR corresponding to report Config, which will complicate the configuration design and have a significant impact on the specifications. Therefore, we propose to introduce a dedicated SR resource specifically configured for LTM event reporting procedure, to save the PUCCH-SR resources.

Proposal 8: Support dedicated SR configuration for the SR triggered by event-triggered measurement report.

2.4	Reporting after event triggering

In legacy L3 measurement report, the parameter reportAmount is used to indicate the number of reports that the UE sends to the network when the event is triggered. The network may instruct the UE perform a handover after receiving an event-triggered report with reportAmount set to 1. The network may also require multiple reports from the UE in order to make a more informed decision. After the network has received a first report, it may not make the handover decision immediately and wait for further reports over a period of time before initiating a handover, in order to avoid handover too early.

For event-triggered L1 reports for LTM cell switch, if the mechanism of one-shot reporting should be supported, the network could similarly decide to make the LTM cell switch decision immediately upon receiving a report.

Event triggered periodic report involves multiple/periodic event report after the event is triggered and the potential HO/cell switch decision is only made after the reception of multiple report in the gNB side. This is contradictory with the benefits of event-triggered measurement report or LTM since the intention of LTM is to enable fast cell switch or HO. Hence, we propose the following:

Proposal 9: Event-triggered periodic report is not supported.

In L3 measurements, when the event is triggered for a specific cell, this cell is added to the cellsTriggeredList defined within the VarMeasReportList and a report is sent. Then, no new report is triggered for the cells already in the cellsTriggeredList and the network assumes that the quality of the cell remains unchanged. If the cell satisfies the leaving condition, the UE removes it from the cellsTriggeredList and it could trigger another report.

For L1 event-triggered reports, RAN2 already agreed to have an entering condition and a leaving condition, Therefore, it also makes sense that when the L1 event is triggered for a certain cell i.e., entry condition satisfied during the TTT, no new report is triggered for the cell until the leaving condition is satisfied during TTT. This avoids unnecessary signaling, which is the motivation of introduction of event-triggered LTM.

Proposal 10: When event is triggered by a certain cell, it shouldn't be triggered again until the leaving condition is satisfied and the entering condition is met again.

Another question is whether to trigger a report when the leaving condition is satisfied during the TTT after a report was triggered for the entering condition i.e., report on leave.

In the L3 measurement, the parameter reportOnLeave indicates whether the UE should trigger the measurement reporting procedure when the leaving condition is satisfied for a candidate cell after meeting the entering condition. In the agreements of last meeting, the legacy mechanism for event triggering of entering and leaving condition is agreed to be reused. Without report set by reportOnLeave, the network remains unaware of the any change in cell quality, so it might instruct the UE to perform a handover to a low-quality cell, which is likely to fail. Therefore, when the UE meets the leaving condition, it is essential to send a report to inform the network timely.

Observation 3: If measurement report is not sent to the network when the leaving condition is met, the network may assume that the cell remains unchanged and make wrong handover decisions.

As can be seen above, in the legacy L3 measurement, the support of report on leave goes hand-in-hand with the above-mentioned UE variable cellTriggeredList: the UE will not trigger the measurement report for the cells in the cellsTriggeredList for satisfying entry condition to save power consumption.

In LTM, it is also crucial for the network to get the updated cell quality when the previously reported cell has met the leaving condition if we think that the event should not be repeatedly triggered by entering condition.

Proposal 11: Report on leave is supported for event-triggered L1 measurement reporting.

# 3	Conclusion

In this paper, we have addressed the objective regarding improvements in event-triggering reports for LTM. There are the following proposals:

Event triggering condition

Proposal 1: TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

Proposal 2: TTT timer should not be restarted if the beam in the candidate cell used for event evaluation changes and the new beam still meets the entering condition.

Proposal 3: TTT timer is maintained per event/report Config.

Specification modelling for event-triggered L1 reporting

Proposal 4: MIMO and LTM event evaluation can be unified under the same UE procedure in MAC spec.

Proposal 5: The entering/leaving conditions for events with beam-level measurement as triggering quantity are evaluated in L1. After evaluation, L1 indicates to higher layer whether the condition is satisfied/not satisfied.

Configuration framework

Proposal 6: Support a measID in configuration of event-triggered L1/2 reports to optimize signalling overhead.

Proposal 7: When there is no UL-SCH resource to transmit the triggered MAC CE for event-triggered measurement report, trigger SR.

Proposal 8: Support dedicated SR configuration for the SR triggered by event-triggered measurement report.

Reporting after event triggering

Proposal 9: Event-triggered periodic report is not supported.

Proposal 10: When event is triggered by a certain cell, it shouldn't be triggered again until the leaving condition is satisfied and the entering condition is met again.

Proposal 11: Report on leave is supported for event-triggered L1 measurement reporting.

# 4	Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4;

- RAN2 #126 chairman notes;

- TS 3GPP 38.331 V18.1.0

- TS 3GPP 38.212 V18.3.0