3GPP TSG-RAN WG2 Meeting #127bis	R2-2408791

Hefei, P. R. China, 14th– 18th October, 2024

Agenda item:	8.16

Source: 	Huawei, HiSilicon

Title: 	Discussion on the support of BDS B2b in A-GNSS

Document for:	Discussion and Decision

1. Introduction

During RANP 105, a new WI for the support of BDS B2b signal (RP-242413) has been agreed with the following objectives:

Specify A-BDS enhancements in Rel-19, including: - Supporting new signal of BDS B2b in A-GNSS for both E-UTRA and NR [RAN2]; - Specify corresponding assistant information in dedicated signaling [RAN2].

In this discussion, we provide our thinkings on how to support BDS B2b signal in 3GPP specs per objectives above.

2. Discussion

According to the ICD, within the nav message of BDS B2b, the following fields have been added for B2b. Basically, for the support of BDS B2b in A-GNSS, what we need to do is to carry these information elements defined in the nav message within the LPP message. We examine the spec impacts of each parameter within the nav message one by one

| Nav message elements | Description for the element | LPP spec impact |
| --- | --- | --- |
| PRN | identity for the satellite | Corresponds to the IE SV-ID in the current LPP spec. legacy IE can be reused but reference needs to be added |
| Message Types | MesType is used to identify the message types of the B-CNAV3 frames | No change needed. Message type is for the format delivered by the satellite link directly. While for GNSS assistance data delivery by LPP transport, it is not needed |
| SOW | Seconds of week | No impact |
| WN | Week number | Corresponds to weekNumber and weekNumber-ext in GNSS-almanac. No impacts |
| Clock Correction parameters | Includes toc, a0, a1, a2 | Cannot reuse the existing fields in BDS-ClockModel2 with the range of different parameters to be very different. Better to create a new IE to hold these parameters with new ranges |
| Group delay differential parameters | TGDB2bI | Can be added to the new clock model created to the B2b signal per above in the clock correction parameters |
| Ephemeris parameters |  | Can reuse the existing fields in NavModel-BDS-KeplerianSet2. But description needs to be updated |
| Ionospheric Delay Correction Model Parameters | The BeiDou Global Ionospheric delay correction Model (BDGIM) contains nine parameters which are used to correct the effect of ionospheric delay for the single frequency user | Can reuse klobucharModel2 in the current LPP |
| Midi Almanac Parameters | Contains 14 parameters | Can reuse the existing field AlmanacMidiAlmanacSet but description needs to be updated |
| Reduced Almanac parameters |  | Can reuse the existing field AlmanacReducedKeplerianSet but description needs to be updated Almanac reference time |
| Earth Orientation Parameters |  | Can reuse the IE GNSS-EarthOrientationParameters |
| BDT-UTC Time Offset Parameters |  | Can reuse the IE UTC-ModelSet2 |
| BDT-GNSS Time Offset Parameters |  | Can reuse the IE GNSS-TimeModelElement |
| Satellite Health Status | Satellite Health Status (HS) is an unsigned integer with a length of 2 bits, which indicates the health status of the transmitting satellite. | Can reuse the existing IE svHealth |
| Satellite Integrity Status Flag | The specific definitions of the signal integrity status flag parameters will be published in a future update of this ICD. | Since the definition has not been specified by the document, we can come back to this later when the definition is updated |
| Signal In Space Accuracy Index | The signal in space accuracy describes the predictive accuracy of the orbital parameters and clock correction parameters broadcast in the navigation message. It contains the along track and cross-track accuracy of the satellite orbit (SISAoe) and the satellite orbit radial and satellite clock correction accuracy (SISAoc). The specific definitions of the signal in space accuracy index parameters will be published in a future update of this ICD | Since the definition has not been specified by the document, we can come back to this later when the definition is updated |
| Signal In Space Monitoring Accuracy Index | The estimated error of the signal in space accuracy is described by the zero-mean Gaussian distribution model. The signal in space monitoring accuracy (SISMA) is the variance of the Gaussian distribution, which is indicated by the signal in space monitoring accuracy index (SISMAI). The specific definitions of the signal in space monitoring accuracy index parameters will be published in a future update of this ICD | Since the definition has not been specified by the document, we can come back to this later when the definition is updated |

Based on the above discussion, in order to support BDS B2b signal, we propose the following

Proposal1: For different parameters in the navigation message defined in clause 6 and 7 of [1], add them into the LPP message by the following changes:

- PRN: reuse the legacy IE SV-ID in LPP

- MessageType: No need for any change for message type

- Time model: Can reuse the legacy IE GNSS-TimeModel

- Clock correction parameters: Create a new IE for holding the parameters Toc, a0, a1, a2

- Group delay differential parameters: Can be added to the new IE created for the clock correction parameters

- Ephemeris parameters: Reuse the existing IE NavModel-BDS-KeplerianSet2

- Ionospheric model: can reuse the klobucharModel2 in the current LPP

- Midi Almanac Parameters: reuse the existing field AlmanacMidiAlmanacSet and add one bit indicating the B2b SV health

- Reduced Almanac parameters: reuse the existing field AlmanacReducedKeplerianSet and add one bit indicating the B2b SV health

- Earth Orientation Parameters: reuse the IE GNSS-EarthOrientationParameters

- BDT-UTC Time Offset Parameters: reuse the IE UTC-ModelSet2

- BDT-GNSS Time Offset Parameters: reuse the IE GNSS-TimeModelElement

- Satellite Health Status: reuse the existing IE svHealth

- No change is needed for all the parameters related to integrity in the nav message.

3. Conclusion

In this contribution, we analyzed the potential aspects for enhancements related to multi-modal XR. The following observations and proposals were made:

Assistance data in LPP

Proposal1: For different parameters in the navigation message defined in clause 6 and 7 of [1], add them into the LPP message by the following changes:

- PRN: reuse the legacy IE SV-ID in LPP

- MessageType: No need for any change for message type

- Time model: Can reuse the legacy IE GNSS-TimeModel

- Clock correction parameters: Create a new IE for holding the parameters Toc, a0, a1, a2

- Group delay differential parameters: Can be added to the new IE created for the clock correction parameters

- Ephemeris parameters: Reuse the existing IE NavModel-BDS-KeplerianSet2

- Ionospheric model: can reuse the klobucharModel2 in the current LPP

- Midi Almanac Parameters: reuse the existing field AlmanacMidiAlmanacSet and add one bit indicating the B2b SV health

- Reduced Almanac parameters: reuse the existing field AlmanacReducedKeplerianSet and add one bit indicating the B2b SV health

- Earth Orientation Parameters: reuse the IE GNSS-EarthOrientationParameters

- BDT-UTC Time Offset Parameters: reuse the IE UTC-ModelSet2

- BDT-GNSS Time Offset Parameters: reuse the IE GNSS-TimeModelElement

- Satellite Health Status: reuse the existing IE svHealth

- No change is needed for all the parameters related to integrity in the nav message.