3GPP TSG-RAN2 Meeting #127bis	R2-2408799

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.11.2

Source:	Qualcomm Incorporated

Title:	Views on random access for SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. One of the objectives is to specify SBFD operation to support random access in SBFD symbols for UE in RRC-connected UE while studying the benefit of extending the random-access operation in SBFD symbols to UE(s) in RRC idle and inactive state.

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] …

In RAN2 #127 meeting, the following agreements were made.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In this contribution, we would like to discuss the configurations of random access resource in SBFD symbols for UE in RRC_CONNECTED and RRC_IDLE/INACTIVE.

# Discussion

There were already several basic agreements regarding supporting SBFD random access operation in RAN1.

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

The working assumption to support both CBRA and CFRA in SBFD symbols were confirmed in the later RAN1 meetings. RAN1 was also agreed to support Type-1 random access procedure (4-step RACH) in SBFD symbols and further discuss Type-2 random access process (2-step RACH) after finalizing the design of 4-step RACH. RAN2 #127 meeting further agreed RAN2 will focus on the 4-step RACH for SBRA RA, FFS on 2-step if needed.

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols.

Observation 2: Both RAN2 and RAN1 has agreed to at least support 4-step RACH and FFS on 2-step RACH.

It could be observed that there are several benefits if the SBFD symbols is enabled for random access such as access latency reduction, PRACH collision probability reduction and improved coverage of PRACH. In our view, all the existing RACH triggering events can be supported by random access procedure in SBFD symbols. One may argue that SI request may be the exception. However, we don’t see a reason to exclude it, since it is the same procedure if network can configure the SBFD resource for SI request and it is up to network to configure this. Thus, the RAN2 working assumption can be confirmed.

Proposal 1: RAN2 confirms the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Regarding the RACH configuration for SBFD operation, there were two options under RAN1 discussion. One option is to use single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1) from legacy RACH configuration with modification, and another option is to have two separate RACH configurations (i.e., RACH configuration Option 2) including one legacy RACH configuration and one additional RACH configuration to configure ROs (RACH occasion) within UL subband in SBFD symbols which could be valid for SBFD-aware UE.

It can be observed that single RACH configuration would enable random access for SBFD operation for both SBFD-aware UE and legacy UE without extra signaling overhead and separate RACH configuration for SBFD operation may enable more flexible configuration on the PRACH format, PRACH time/frequency resource, and so on.

In last RAN1 meeting, RAN1 confirmed the working assumption on supporting both options by network configuration but UE is not required to support both options.

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

For a SBFD-aware UEs in RRC_CONNECTED state, it may indicate whether supporting single or separate RACH configurations via UE capabilities. It is up to network to determine single or separate RACH configurations and network provides one RACH configuration for all UEs in the serving cell. For example, network may configure separate RACH configuration. However, if UE only supports single RACH configuration, UE has to use the legacy RACH resource to perform RACH.

Proposal 2: For a SBFD-aware UE in RRC_CONNECTED state and RRC_IDLE/INACTIVE state, it is up to network to configure the UE with either a single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1) or two separate RACH configurations (i.e., RACH configuration Option 2) for SBFD operation.

Proposal 3: If SBFD-aware UE supports the RACH configuration for SBFD according to UE capability and network configures the RACH resource in SBFD symbols matching the UE capability, UE can use SBFD RACH resource.

In addition, for SBFD-aware UE in RRC_IDLE/INACTIVE state, it is straightforward to determine whether the RACH is applicable to the SBFD symbol if network configures separate RACH configurations via the system information. However, for the single RACH configuration, network should also indicate the legacy RACH configuration as only legacy (no valid SBFD-ROs in SBFD symbols) or as single RACH configuration including SBFD resource (with valid ROs in SBFD symbols).

Proposal 4: For single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1), whether the configured ROs in SBFD symbols are valid or not is indicated in system information.

The discussions on two options of SBFD RACH configuration in RAN1 are still ongoing. RAN2 should wait for RAN1 further inputs on the detailed RACH configuration parameters.

Proposal 5: RAN2 waits for RAN1 further inputs on the detailed RACH configuration parameters for SBFD operation.

For RO selection, SBFD-aware UE may be configured with separate SSB mapping to SBFD-ROs and legacy TDD-ROs. In RAN1, there are several options on the table for further discussion.

Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations. - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs. Agreement For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs.

For single RACH configuration, the main motivation is to introduce additional-ROs to improve RACH capacity and reduce PRACH collision probability in legacy-ROs or reduce the random-access latency. If SBFD-aware UE is capable to support PRACH transmission in the additional-ROs, it would be beneficial if UE selects the additional-ROs in the SBFD symbols. By using additional-ROs to transmit Msg1 is also beneficial for early indication of the UE SBFD-awareness. Thus, for single RACH configuration, it is good that UE have priority to select additional-ROs in the SBFD symbols.

For the two separate RACH configurations, the additional PRACH resource like additional SBFD-ROs or different type of preamble supporting SBFD operation are separately configured. Similar to legacy 2-step and 4-step RACH resource selection, it is good to base on certain configuration criteria, e.g., the RSRP of the selected SSB for UE evaluation whether the link is good enough to select one type of RACH configuration rather than the other type.

There are other considerations that network may identify the type of UE (i.e., SBFD-aware UE) via the selected ROs, and further considerations that SBFD-ROs may be less congested than TDD-ROs and the probability of RACH collision can be reduced. In that sense, network may directly indicate which type of RO that SBFD-aware UE is allowed to initiate RACH. Therefore, there should be certain specified/configured conditions/prioritizations to configure UE for RO type selection between legacy-ROs and additional-ROs.

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if no additional indication is from network, UE is allowed to select one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations.

In case of PRACH attempt failures, UE may perform PRACH re-attempt. There were also extensive discussions in RAN1 and several options were on the table for further down selection.

Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random-access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random-access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

The SBFD-aware UE will increase the PRACH transmission power and should re-transmit the preamble in the earliest RO. In some cases, the SBFD-aware UE may be stick to the same type of RO for PRACH-retransmission, and it is not suitable and also not necessary to frequently switch between two types of RO especially for the two separate RACH configuration cases. On the other hand, UE could keep attempting the same type of PRACH transmission with a number of times and if it is still failed and some additional conditions are met, UE may select the other type of RACH resource (e.g., possible less collision) to increase RACH success possibilities.

Proposal 7: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in one type of RACH occasions, and if additional certain conditions are met, UE is allowed to switch to another type of RACH occasions for RACH re-attempt.

It is important for the gNB to know the UE SBFD-aware capability in early stage of the random-access procedure to leverage the SBFD operation. The SBFD-aware indication is useful to inform the UE SBFD capability at the earliest time during RACH, e.g., network may schedule the resource for Msg3 in the SBFD resource in Msg2 if network can be aware the SBFD capable UE performing the RACH. The similar studies were done for many features in the previous NR release. The design is also heavily depending on the solution of RACH resource configuration for SBFD.

If network has configured the RACH occasions on SBFD symbols and UE also supports it, the SBFD-aware UE sends the Msg1 in the SBFD RACH occasions as the SBFD-aware indication to network. So, by using the SBFD RACH occasions can be already an implicit way as the SBFD-aware indication. If the SBFD-aware UE selects the legacy RACH resource (e.g. RACH occasions) to perform RACH, the additional SBFD-aware indication is needed.

Usually, in the previous NR release discussion, whether to configure separate PRACH resource (i.e., Msg1 based SBFD-aware indication) for specific type of UE is decided by RAN1. If Msg1 based SBFD-aware indication is not configured (or not supported), the Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 8: RAN2 to wait for RAN1 further discussion and decision on whether/what is Msg1 based SBFD-aware indication.

Proposal 9: If Msg1 based SBFD-aware indication is not configured (or not supported), the Msg3 based SBFD-aware indication is supported from RAN2 perspective.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols.

Observation 2: Both RAN2 and RAN1 has agreed to at least support 4-step RACH and FFS on 2-step RACH.

Proposal 1: RAN2 confirms the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Proposal 2: For a SBFD-aware UE in RRC_CONNECTED state and RRC_IDLE/INACTIVE state, it is up to network to configure the UE with either a single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1) or two separate RACH configurations (i.e., RACH configuration Option 2) for SBFD operation.

Proposal 3: If SBFD-aware UE supports the RACH configuration for SBFD according to UE capability and network configures the RACH resource in SBFD symbols matching the UE capability, UE can use SBFD RACH resource.

Proposal 4: For single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1), whether the configured ROs in SBFD symbols are valid or not is indicated in system information.

Proposal 5: RAN2 waits for RAN1 further inputs on the detailed RACH configuration parameters for SBFD operation.

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if no additional indication is from network, UE is allowed to select one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations.

Proposal 7: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in one type of RACH occasions, and if additional certain conditions are met, UE is allowed to switch to another type of RACH occasions for RACH re-attempt.

Proposal 8: RAN2 to wait for RAN1 further discussion and decision on whether/what is Msg1 based SBFD-aware indication.

Proposal 9: If Msg1 based SBFD-aware indication is not configured (or not supported), the Msg3 based SBFD-aware indication is supported from RAN2 perspective.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei