3GPP TSG-RAN WG2 Meeting #127bis	R2-2408802

Hefei, China, 14th October – 18th October, 2024

Agenda item:	8.9.2

Source:	Samsung

Title:	Discussion on Store and Forward

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#105 the latest WID for IoT NTN Phase 3 is provided [1]:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide some discussions related Store and Forward.

# Discussion

## Indication of S&F cell and barring

In last meeting (RAN2#127) the following was agreed:

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

The agreements potentially allow for legacy UEs to operate in an S&F cell. This implies that for these legacy UEs, an indication of the S&F cell is not needed. However, a network may still want to bar legacy UEs from operating in an NTN cell.

Given the above, we think that the barring indication, i.e barring Rel-19 S&F capable UEs, and the indication of the UE operating as an S&F shall be the same indication. We think that a simplified approach is that a new barring bit cellBarred-S&F is introduced whereby the presence of the bit, regardless of it being set to barred or notBarred, indicates that the cell is an S&F satellite. When the cell stops or starts operating as an S&F cell, in other words the satellite gains or loses the feeder link to the ground, the cell uses system information update procedures. The change of the cell operating as an S&F is not likely not very dynamic and should not occur very often.

Proposal 1: The indication to bar Rel-19 S&F-capable UEs and the indication that the UE is currently operating in S&F should be the same indication.

Proposal 2: New cellBarred-S&F is introduced whose presence indicates whether the cell is currently operating as an S&F cell, regardless of field set to barred or notBarred.

Proposal 3: If cell status (operating or not operating as S&F, i.e feeder link is available or not available) changes, system information update is used to indicate cellBarred-S&F is configured or not.

## Relation to discontinuous coverage network

In Release 17, discontinuous coverage was introduced. The enhancement to allow for discontinuous coverage are the following:

    - SIB32 indicates long term satellite ephemeris elements.

    - Enhancements to idle mode procedure where the UE is allowed to not perform any idle mode procedures related to NTN if the UE detects that there are no satellites that will provide coverage.

    - UE can indicate over NAS (using TAU procedure) when it is expected that a UE will enter discontinuous coverage [3].

As part of the SA2 conclusion, SA2 has captured the following:

---------------------------- 23.700-29 ----------------------------

2)	When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes:

a)	A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network.

b)	Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN.

c)	Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

---------------------------- 23.700-29 ----------------------------

In other words, SA2 has decided that a satellite ID can be sent in a NAS message. A satellite ID can potentially identify satellite assistance information in SIB31 (serving cell), SIB32 and SIB33. The satellite assistance information in SIB31 and SIB33 are short-term satellite assistance information, so we believe it is unlikely that this shall be used for S&F operation.

If a UE receives a satellite ID in NAS that matches with a satellite ID in SIB32, the UE may use this as the next S&F satellite where the UE can connect, and the UE may thus sleep until the satellite of the specific satellite ID come to pass. So it is more reasonable that the signalled satellite IDs in NAS identify long term ephemeris for discontinuous coverage in SIB32. This implies that according to SA2, discontinuous coverage and S&F are heavily intertwined.

So we believe that a UE capable of S&F can be assumed to be discontinuous coverage-capable. This also means that S&F information should be taken into account for discontinuous coverage operation.

Proposal 4: The signalled satellite ID signalled in NAS, shall refer to a long-term ephemeris in SIB32.

Proposal 5: Satellite ID provided by NAS can be used for discontinuous coverage operation, i.e the UE receiving a satellite ID over NAS can use the associated TLE ephemeris in SIB32 to sleep until the associated satellite is approaching.

# Conclusion

In this contribution we discussed issues related to S&F network.

Proposal 1: The indication to bar Rel-19 S&F-capable UEs and the indication that the UE is currently operating in S&F should be the same indication.

Proposal 2: New cellBarred-S&F is introduced whose presence indicates whether the cell is currently operating as an S&F cell, regardless of field set to barred or notBarred.

Proposal 3: If cell status (operating or not operating as S&F, i.e feeder link is available or not available) changes, system information update is used to indicate cellBarred-S&F is configured or not.

Proposal 4: The signalled satellite ID signalled in NAS, shall refer to a long-term ephemeris in SIB32.

Proposal 5: Satellite ID provided by NAS can be used for discontinuous coverage operation, i.e the UE receiving a satellite ID over NAS can use the associated TLE ephemeris in SIB32 to sleep until the associated satellite is approaching.

# Reference

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#105, Melbourne, Australia, September 2024.