3GPP TSG-RAN WG2 Meeting #127bis	R2-2408803

Hefei, China, 19th October– 23rd October, 2024

Agenda item:	8.9.3

Source:	Samsung

Title:	Procedures for uplink capacity enhancements for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#105 the latest WID for IoT NTN Phase 3 is provided [1]:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide some initial discussions on the objective of Support of Capacity enhancements for the uplink.

# Discussion

- Agreements:

- RAN2#125bis:

Agreements:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements

- Both C-plane and U-plane solutions are within scope of uplink capacity enhancements.

- Only CIoT EPS is within scope of uplink capacity enhancements

- RAN2#126:

Agreements:

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

- RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

- If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

- RAN2#127:

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

## Scenario of RACH-less EDT

In RAN2#125bis meeting it was discussed whether the RACH-less EDT-like feature is only introduced for NTN or may also be applicable to terrestrial networks [2]. In our understanding, the RACH-less EDT-like feature is introduced due to the fact that the NTN UE can autonomously compensate the timing advance due to the geometry of a satellite deployment. A terrestrial network does not exhibit the same geometry, which makes it difficult for a terrestrial UE to use the same methods – when the UE moves on the surface of the earth the Euclidean distance from a UE to a point (representing the eNB) changes differently depending on the height of the point. As result the required GNSS position fix accuracy would be much higher in a terrestrial network. Thus we believe that it is not technically useful to discuss such a scenario. Furthermore it is clear that the WID is for non-terrestrial networks.

Another important topic is whether Msg4 enhancements may be deployed without using the RACH-less EDT feature. This may be useful as the Msg4 enhancements can in general provide benefit for the network. However, we believe that decoupling these features may require significant amount of work, for instance if the Msg4 enhancements are introduced for random access with RACH, the UE has to indicate support for the Msg4 enhancements. This always tend to introduce complexity, which makes the Msg4 enhancement less likely to be deployed as a standalone enhancement. We also think that there may need to be Msg4 enhancement specifically to support RACH-less EDT, so it is more suitable to introduce all of the enhancements for a single feature. This can easily serve to ensure that any Msg4 enhancement does not have a lot of optionality, for ease of deployment.

- Observation 1: The required GNSS position fix accuracy for RACH-less EDT in a terrestrial network would be significantly higher compared to NTN owing to the geometry.

Proposal 1: RACH-less EDT-like procedures and any Msg4 enhancement are only introduced for NTN.

Proposal 2: Any Msg4 enhancement is introduced in combination with RACH-less EDT, and not considered as a standalone enhancement deployed without RACH-less EDT.

## Discussion on LS reply

In R2-2407920, RAN2 received a reply to the LS sent to RAN1 in the RAN2#126 meeting. The LS originally asked whether there would be any RAN1 impact related from synchronization from sending Msg3 without having sent Msg1. RAN4 also replied to the LS in R2-2407931.

In essence, RAN1 and RAN4 replied that they did not foresee any issues as long as the timing error is within RAN4 specified-limits. The RAN4 specified timing limits can be seen in Table 1 and 2 for NB-IoT NTN and LTE-M NTN respectively.

Table 1 (Table 7.20A.2-1 in 36.133): Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

Table 2 (Table 7.24A.2-1 in 36.133): Te_NTN_M1 Timing Error Limit

| CE Mode | Te_NTN_M1_ |
| --- | --- |
| A | 41*TS |
| B | 65*Ts |
| Note 1: TS is the basic timing unit defined in TS 36.211. Note 2: This requirement applies regardless of the downlink carrier bandwidth. |  |

As seen in 36.133, the timing limits to in Table 1 applies to NPUSCH and NPRACH, and Table 2 applies to PUSCH and PRACH. It is clear from this that both RAN1 and RAN4 think that it is possible for a UE to directly send an Msg3 without first performing Msg1.

Proposal 3: RAN2 confirms that transmitting Msg3 without Msg1 is possible if the UE transmission timing error limit is satisfied.

Thus RAN2 needs to discuss how and what is needed in order for to ensure that the timing error limit is satisfied. It should be possible to assume that a device needs to have both valid GNSS and recent ephemeris. One way to ensure that the ephemeris is recent enough is that a shorter uplink sync validity duration is introduced specifically for RACH-less EDT to ensure that the UE does not operate when the uplink sync validity timer has already ran for a long time.

Proposal 4: RAN2 to discuss how to ensure timing error limit:

- UE ensures GNSS position fix is valid,

- UE ensures recently acquired ephemeris, through a shorter uplink sync validity duration timer for RACH-less EDT.

## EDT without RACH procedures

One of the key differences between EDT and PUR, is that PUR is entirely pre-configured, whereas EDT is largely dedicatedly configured. For PUR, this makes it difficult to deploy as in practice it is difficult for both network and UE to predict when and how large future data transmissions will be in the future, outside a few select use cases. EDT on the other hand only requires the UE to be in a suspended RRC states with security configured for UP EDT, while CP EDT does not require any pre-configuration. For RACH-less Early Data transmissions, we think it would be crucial for this feature to require minimal if not any pre-configuration.

Proposal 5: Contention-based RACH-less EDT-like procedure is not dedicatedly pre-configured, but uses broadcasted common resources.

For Release 13, eMTC and NB-IoT introduced extended coverage, which continues to be one of the main selling points of CIoT. In early releases of the two technologies, a lot of time was spent on introducing different amounts of repetitions to all channels. This includes repetitions of all messages of the random access procedure, i.e Msg1, Msg2, Msg3 and Msg4. Similarly, depending on the deployment and capability of the satellite network, it could be valuable to introduce repetitions for RACH-less Msg3 transmissions.

Proposal 6: Introduce possibility to configure repetitions for RACH-less EDT-like transmissions.

In some circumstances, depending on the configuration, it may be expected that RACH-less EDT-like transmission would be less reliable compared to legacy random access. Failure rate can be dependent on the synchronization of the network and the UE, the radio characteristics and the load on the RACH-less Msg3 resources. Given that the legacy random access should be more reliable, a UE should be able to fall back towards legacy random access. There are two ways in which fallback to random access with RACH; 1) the UE detects the failure and initiates the fallback and 2) the network detects a failure and signals to a UE to fallback to random access with RACH. We think that it would make sense to have both options available.

Proposal 7: If RACH-less Msg3 fails, it should be possible to fallback to random access with RACH.

Proposal 8: RAN2 to discuss whether RACH-less Msg3 fallback should be UE initiated, or network signals to UE to fallback, or both.

## DSA and CRDSA

In RAN2#127, the following agreement was taken:

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

Figure 1. Transmissions of UEs in a configured CRDSA or DSA frame, with size 6 slots, and two replicas are sent in the frame. Collisions are seen in Slot 2, 3 and 5.

DSA

Diversity Slotted Aloha is a random access technique that can increase the likeliness of reception by transmitting multiple replicas of the same message in random slots to ensure that at least one message may be successfully received. An example of this can be seen in Figure 1, where users are configured to transmit 2 replicas of the same frame. In Figure 1, there are collisions in Slot 2, 3 and 5. In these collisions, the technique does not offer any way to resolve the collisions, but they may however be resolved if for instance the difference in power is large enough. This means that in the worst case, considering all non-collisions to be successfully decoded, User 2 and User 4 will be considered successful. DSA can be shown to have clear gains, normalized over the resource-usage, compared to Slotted aloha. RACH-less EDT Msg3 transmission is a form of Slotted Aloha transmission.

From the perspective of the UE, the UE only needs to transmit its RACH-less Msg3 in randomly selected multiple slots with in a frame, which is not very complicated. For the network, the network has to configure a frame where the replicas are transmitted in and configure the number of replicas. The algorithm is also not very complicated for the network, as the network will attempt to first detect whether there are any transmissions in the slot, and if there are, the network will attempt to decode it. The network does not even need to try to detect whether there are collisions or not.

DSA can be implemented by RAN2, as RAN2 only needs to consider in which slots that RACH-less EDT Msg3 replicas shall be transmitted in and how to handle receiving Msg4. Deciding the slots to use for RACH-less EDT Msg3 is something RAN2 already needs to design. So DSA can be introduced by introducing a random scheme for selecting the slots within the frame, which cannot be considered big impact for RAN2.

Observation 2: Difference between RACH-less EDT and RACH-less EDT using DSA is a random slot selection technique for the RACH-less EDT Msg3 transmission.

CRDSA

Contention-Resolution Slotted Diversity Aloha is a random access technique that is based on Successive Interference Cancellation [2] and Diversity Slotted Aloha.

The technique is fairly easy to understand intuitively. For each CRDSA attempt, each UE will transmit multiple replicas randomly selected slots in a configured frame, similar to DSA. If a set of UEs collide in a single slot, then the successive interference cancellation algorithm at the receiver will cancel out the interference contribution of a UE based on a successful decoding in another slot. An example can be seen in Figure 1. In this case the UE transmits two replicas in a set of 6 slots. There are collisions in slots 2, 3, 5, which normally may not be decodable. This is resolved by the network being able to successively remove interference contributions. As seen in Figure 1, the packet p2 of User 2, can be resolved since there is no collision in Slot 1. For Slot 2, the packet of User 1 can be resolved via p2 = r2 – p2 = r2 – r1. User 4 and 3 cannot be resolved as there is a collision of these packets in Slot 3 and Slot 4.

From the perspective of the UE there are not much implementation difficulty in CRDSA. Same as for DSA, a random slot selection needs to be introduced. Furthermore, there also needs to be an indication of the slots that the UE selected, so that the network can identify the other slots where the UE has replicated its transmission. This is can be introduced by RAN2 in a MAC CE for instance.

From the perspective of the network, the algorithm is undoubtedly more complicated. The network has to first save all of the IQ samples of all slots where the network has detected transmissions in. This means that the network has to save all of the slots, and then process them after a CRDSA frame has finished. For instance if the CRDSA frame is defined to have 6 subframes, then the network cannot fully start processing these 6 subframes after all of the 6 subframes have elapsed. The network then has to perform successive interference cancellation potentially a number of times to remove all interference contributions from the resolvable slots.

In total the network has to implement collision-detection, multi-subframe IQ sample storing and successive interference cancellation. However, the UE impacts are fairly limited.

Observation 3: Standards difference between RACH-less EDT and RACH-less EDT using CRDSA is a random slot selection technique for the RACH-less EDT Msg3 transmission and including a ID to identify where other replicas where transmitted for Successive Interference Cancellation.

Given this we propose that as a first step, it is agreed that DSA may be introduced. For CRDSA, we think that it can be confirmed, without agreeing to CRDSA, that the RAN2 UE impact is that the UE signals which subframes it was transmitted in. RAN2 then sends an LS regarding the feasibility of the identified impacts, which are to store multi-subframe samples, detect collisions and performing successive interference cancellation.

Proposal 9: RAN2 introduces DSA for RACH-less EDT, by introducing RACH-less EDT Msg3 to be transmitted in multiple random subframes in a pre-configured set of subframes.

Proposal 10: RAN2 confirms that RAN2 impacts of CRDSA compared to DSA is to signal the subframes the frame was replicated in.

Proposal 11: Send LS to RAN1/RAN4 on the identified impacts of CRDSA and on the identified impacts: 
- Successive Interference cancellation.
- Multi-subframe sample storing, i.e not being able to process subframe until full frame transmitted. 
- Collision detection.

# Conclusion

In this contribution we discussion initial issues on uplink capacity enhancements:

- Observation 1: The required GNSS position fix accuracy for RACH-less EDT in a terrestrial network would be significantly higher compared to NTN owing to the geometry.

Observation 2: Difference between RACH-less EDT and RACH-less EDT using DSA is a random slot selection technique for the RACH-less EDT Msg3 transmission.

Observation 3: Standards difference between RACH-less EDT and RACH-less EDT using CRDSA is a random slot selection technique for the RACH-less EDT Msg3 transmission and including a ID to identify where other replicas where transmitted for Successive Interference Cancellation.

Proposal 1: RACH-less EDT-like procedures and any Msg4 enhancement are only introduced for NTN.

Proposal 2: Any Msg4 enhancement is introduced in combination with RACH-less EDT, and not considered as a standalone enhancement deployed without RACH-less EDT.

Proposal 3: RAN2 confirms that transmitting Msg3 without Msg1 is possible if the UE transmission timing error limit is satisfied.

Proposal 4: RAN2 to discuss how to ensure timing error limit:

- UE ensures GNSS position fix is valid,

- UE ensures recently acquired ephemeris, through a shorter uplink sync validity duration timer for RACH-less EDT.

Proposal 5: Contention-based RACH-less EDT-like procedure is not dedicatedly pre-configured, but uses broadcasted common resources.

Proposal 6: Introduce possibility to configure repetitions for RACH-less EDT-like transmissions.

Proposal 7: If RACH-less Msg3 fails, it should be possible to fallback to random access with RACH.

Proposal 8: RAN2 to discuss whether RACH-less Msg3 fallback should be UE initiated, or network signals to UE to fallback, or both.

Proposal 9: RAN2 introduces DSA for RACH-less EDT, by introducing RACH-less EDT Msg3 to be transmitted in multiple random subframes in a pre-configured set of subframes.

Proposal 10: RAN2 confirms that RAN2 impacts of CRDSA compared to DSA is to signal the subframes the frame was replicated in.

Proposal 11: Send LS to RAN1/RAN4 on the identified impacts of CRDSA and on the identified impacts: 
- Successive Interference cancellation.
- Multi-subframe sample storing, i.e not being able to process subframe until full frame transmitted. 
- Collision detection.

# Reference

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#105, Melbourne, Australia, September 2024.

R2-2403206, Clarifications on the Scope of EDT enhancement for IoT-NTN, RAN2#125bis, Interdigital, April 2024.

Alessio M. et al, Random Access in DVB-RCS2: Design and Dynamic Control for congestion avoidance, IEEE transactions on Broadcasting, Vol: 60, Issue 1, 2014.