3GPP TSG-RAN WG2 Meeting #127bis	R2-2408804

Hefei, China, 14th – 29th October, 2024

Agenda item:	8.9.4

Source:	Samsung

Title:	Impact of PWS broadcasting for NB-IoT

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#105 the latest WID for IoT NTN Phase 3 is provided [1]:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide some discussions related to PWS broadcasting for NB-IoT.

# Discussion

## PWS broadcasting in LTE

- Public Warning System indication have been introduced in LTE from the very first releases.

- Earthquake Tsunami Warning System (ETWS) was introduced in Release 8 LTE and is also supported for LTE-M. The ETWS is indicated via SIB10 and SIB11, where the SIB10 provides the primary indication and SIB11 provides secondary notification.

- Commercial Mass Alert System (CMAS) was introduced in Release 9 LTE and is also supported for LTE-M. The CMAS is indicated via SIB12.

Observation 1: PWS is broadcasted via ETWS in SIB10 and SIB11 and CMAS in SIB12.

- In LTE there are two ways in which PWS signalling may be indicated. For normal LTE devices, i.e non-eMTC devices, ETWS or CMAS is indicated via Paging, where the UE is indicated in the Paging RRC message whether ETWS or CMAS is being broadcasted. Thus the UE monitors for P-RNTI and once a Paging message is received, a flag indicates either ETWS or CMAS. For eMTC, Direct Indication is supported. Direct Indication is an 8-bit message that is inserted directly into the PDCCH message with a specific format, which means that the UE does not need to acquire the corresponding PDCCH message. The eMTC UE monitors P-RNTI or SI-RNTI to receive the Direct Indication. The Direction Indication has 1-bit flags that indicate whether CMAS or CMAS is indicated. System information update may also be indicated via the Direct Indication. The Direct Indication was also adopted for 5G NR.

Observation 2: UE monitors P-RNTI for ETWS and CMAS indication in RRC idle and eMTC UE monitors SI-RNTI in connected mode.

## PWS broadcasting for NB-IoT

- For signalling PWS broadcast, it should be possible to copy LTE and eMTC PWS signalling to a large degree.

- For signalling ETWS and CMAS, we propose to use the same SIBs as for LTE. In other words, SIB10 to SIB12-NB are introduced. Using the same numbering introduces a lot of commonality making the specifications significantly easier to use and this is what LTE has been employing thus far. We believe that all the fields of SIB10, SIB11 and SIB12 may be reused for NB-IoT.

Proposal 1: ETWS is indicated in SystemInformationBlockType10-NB and SystemInformationBlockType11-NB.

Proposal 2: CMAS is indicated in SystemInformationBlockType12-NB.

Proposal 3: The fields of SIB10, SIB11 and SIB12 are reused for the NB-IoT equivalents.

- For indicating the ETWS or CMAS, we propose that NB-IoT Direct Indication is used. In other words, two new fields in the Direction Indication table 6.7.5-1 in 36.331 are introduced.

Proposal 4: ETWS and CMAS being broadcasted is indicated in NB-IoT Direct Indication for UEs in RRC idle. FFS for RRC connected UEs.

## Applicability for TN

- In the WID the following is stated regarding whether the PWD:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

- While the primary use case of PWS signalling is for NB-IoT NTN, if the signalling is reused from LTE, then there is no reason to think that it would only be applicable for NTN. Remember that no adaptations for PWS broadcasting was introduced neither for eMTC NTN nor for NR NTN. In our understanding, there has been no adaptation or enhancement across SA or CT to support PWS broadcasting over NTN. Thus we believe that the enhancements shall be applicable to NB-IoT TN. This also follows the principle of how NTN was developed - if nothing is stated or if it is not obvious, a TN feature may be considered applicable for NTN.

Proposal 5: NB-IoT PWS broadcasting introduced for IoT NTN can be assumed to be applicable for TN NB-IoT.

## SA/CT aspects of PWS broadcasting for NB-IoT

- Most of the specification on PWS signalling is in SA and CT, for instance how an MME is triggered by the CBC (Cell Broadcast Centre) and this is already supported for E-UTRAN. it is not believed that introducing PWS broadcasting to NB-IoT will have a lot of specification impact, because in the end NB-IoT (NTN) is over EPC, and the specific technology within E-UTRAN should not impact the core network signalling. However, in current SA2 specifications, it is made clear that NB-IoT does not support PWS signalling. This can for instance be seen in the following part of 23.401 [2]:

- ------------------------------------- 23.401 -------------------------------------

Table M-5: Misc.

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.8 | MBMS (3) | (3) | (3) | (3) | (3) |
| 5.10 | Multiple PDN support and PDN activation for UEs supporting Attach without PDN connectivity | Yes (1) | Both |  |  |
| 5.11 | UE capability handling | Yes | Both |  |  |
| 5.12 | Warning message delivery | No | N/A | N/A | N/A |
| 5.13 | DRX and UE-specific DRX parameter handling | Yes (2) | Both |  |  |
| 5.13a | Extended idle mode DRX | Yes | Both | Optional | Optional |
| 5.15 | RAN information management procedures | No | N/A | N/A | N/A |
| 5.16 | MME-initiated procedure on UE's CSG membership change | No | N/A | N/A | N/A |
| 5.17 | HeNB multicast packet forwarding function | No | N/A | N/A | N/A |
| NOTE 1: Multiple PDN optional. NOTE 2: UE-specific DRX not supported in NB-IoT RAT. NOTE 3: See Table M-6. |  |  |  |  |  |

- ------------------------------------- 23.401 -------------------------------------

- Thus we think that it could be prudent to indicate to SA2 that RAN2 introducing PWS broadcasting to NB-IoT (NTN). Then SA2 may initiate changes if required, or else change their specifications to indicate that it is supported for NB-IoT.

Proposal 6: Send an LS to SA2 regarding RAN2 introducing PWS broadcasting to NB-IoT (NTN and TN).

# Conclusion

In this contribution, we discussed issues related to PWS signaling for NB-IoT.

Observation 1: PWS is broadcasted via ETWS in SIB10 and SIB11 and CMAS in SIB12.

Observation 2: UE monitors P-RNTI for ETWS and CMAS indication in RRC idle and eMTC UE monitors SI-RNTI in connected mode.

Proposal 1: ETWS is indicated in SystemInformationBlockType10-NB and SystemInformationBlockType11-NB.

Proposal 2: CMAS is indicated in SystemInformationBlockType12-NB.

Proposal 3: The fields of SIB10, SIB11 and SIB12 are reused for the NB-IoT equivalents.

Proposal 4: ETWS and CMAS being broadcasted is indicated in NB-IoT Direct Indication for UEs in RRC idle. FFS for RRC connected UEs.

Proposal 5: NB-IoT PWS broadcasting introduced for IoT NTN can be assumed to be applicable for TN NB-IoT.

Proposal 6: Send an LS to SA2 regarding RAN2 introducing PWS broadcasting to NB-IoT (NTN and TN).

# Reference

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#105, Melbourne, Australia, September 2024.

3GPP TS 23.401 “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”, Release 19, V19.0.0, September 2024.