3GPP TSG-RAN WG2 Meeting #127bis	R2-2408806

Hefei, China, 14th – 18th October, 2024

Agenda item:	8.8.5

Source:	Samsung

Title:	On adaptations regenerative payload for NR NTN

WID/SID:	NR_NTN_Ph3

Document for:	Discussion and Decision

# Introduction

In RAN#103 the NR-part of the WID NR NTN Phase in Rel-19 was agreed. The updated objectives of the WID on the topic of Regenerative payload is the following [1]:

4. Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we provide our views on regenerative payload for NR NTN.

# Discussion

## Satellite switch with resync

In RAN2#126, the following was discussed:

R2-2405021	Support of regenerative payload	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: PCI unchanged solution is beneficial for signaling overhead reduction in NTN system due to without L3 mobility.

Proposal 2: To support PCI unchanged for regenerative payload with gNB on board, keeping same security key between source satellite gNB and target satellite gNB based on ISL coordination is demanded.

-	LG wonders if different satellites can use the same PCI. CMCC thinks this is possible

-	Nokia thinks satellite switch with resync could be supported in this case, but with impacts on RAN3 and SA3 (and this would probably need a new WI). Ericsson agrees

-	Thales thinks we should support this feature in a clean way, involving RAN3 and SA3. CMCC agrees

-	Ericsson thinks we should not support satellite switch with resync with regenerative payload

-	Ericsson thinks that the possible involvement of other groups needs to be discussed in the plenary. LG agrees

Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary

In the RAN#104 and RAN#105, there were contributions proposing that satellite switch with resync with regenerative payload should be considered for Release 19. However, all of the contributions related to this were noted and no conclusion was reached, because it would be considered to be up-scoping. So we believe that RAN2 should confirm RAN plenary’s intention and not consider satellite switch with resync for regenerative payload in release 19. While main bulk of the work would not be in RAN2, RAN2 can better assess the benefits of introducing such enhancement. And in this case, the benefits are not considered to be very large.

Proposal 1: RAN2 confirms that satellite switch with resync for regenerative payload is not addressed in Rel-19.

## RA SDT

In the last meeting, the following was agreed and discussed:

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

R2-2406636	Discussion on regenerative payload	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 4	After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

-	Apple wonders if this is a NW implementation issue

Come back in the next meeting

In [2], the issue of waiting until a DL data response is discussed. This can be seen in Figure 1.

Figure 1. Figure from [2].

While the suggested enhancement may improve power consumption, it relies an assumption that DL data will be sent directly in response to the UL data. This assumption cannot always be assumed to be correct as DL data response may take time to transmitted back to the UE, and there is no guarantee that there will be any DL data in response to the UL data. A network may not be implemented to wait for DL data, as the network cannot make this assumption. Therefore, if there is no downlink data, then the UE would have to wait for an RTT, which itself causes power consumption.

Given that RA-SDT is not a vital feature for NR NTN regenerative payload, and that RA-SDT works in the current specification for regenerative payloads, we do not see a lot of benefit of introducing an enhancement for at most fractional power saving specifically for RA-SDT. Similarly, we do not think that the WID supports power saving enhancements to support an optional and non-vital feature for regenerative payloads. Another reason to not support such an enhancement is related to how to handle UEs that support or do not support this feature. Handling support and non-support of feature during random access introduces complexity.

Proposal 2: Do not optimize RA-SDT procedure for regenerative payload architecture.

## RRC inactive and RRC re-establishment

In RAN2#126, the issue of context retrieval during RRC re-establishment was discussed and in RAN2#127 the following was discussed:

R2-2407054	Discussion on the support of regenerative payload	Xiaomi	discussion

- RAN paging

Observation 1: The UE cannot receive RAN paging if there is no ISL between the serving cell and the last serving cell of the UE.

Proposal 1: Consider the following two solutions to address RAN paging:

•	The RAN notification area only includes cells deployed with the serving satellite.

•	The UE performs a RAN update when there is no ISL between the serving cell and the last serving cell of the UE.

-	Lenovo wonders in the UE needs to know about the presence of ISL

-	Apple thinks we don’t have to worry whether ISL is available or not

-	vivo thinks this is in RAN3 scope and we should not discuss this unless asked by Ran3

-	Xiaomi is ok to discuss this in RAN3

In RAN3#125, the following was agreed:

[Agreement] RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

Therefore we think that RAN2 can confirm the RAN3 agreement and avoid to further discuss the issue.

Proposal 3: RAN2 confirms RAN3 agreement that current mechanisms for RRC resume and RRC re-establishment may be reused for regenerative payload and is not further discussed in RAN2 in Release 19.

# Conclusion

In this contribution we discussed issues related to regenerative payload for NR NTN.

Proposal 1: RAN2 confirms that satellite switch with resync for regenerative payload is not addressed in Rel-19.

Proposal 2: Do not optimize RA-SDT procedure for regenerative payload architecture.

Proposal 3: RAN2 confirms RAN3 agreement that current mechanisms for RRC resume and RRC re-establishment may be reused for regenerative payload and is not further discussed in RAN2 in Release 19.

# Reference

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN#104, Shanghai, China, June 2024.

R2-2406636, Discussion on regenerative payload, Qualcomm Incorporated, RAN2#127, Maastricht, Netherlands, August 2024.