3GPP TSG-RAN WG2 #127bis	R2-2408824

Hefei, China, Oct 14 – 18, 2024

Agenda Item:	8.10.2

Source:	Ericsson

Title:	SON support for MRO

Document for:	Discussion/Decision

# Introduction

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

•	Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

•	Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

This contribution discusses considerations regarding SON enhancements for LTM and CHO with candidate SCGs.

# Discussion

## SON for LTM

Per the agreements from previous meetings, we will focus on MCG LTM, and discuss the enhancement for the RLF report, the SHR and the RA report for LTM optimization. Considering that the LTM cell switch procedure highly relies on underlying features such as early DL/UL synchronization and a RACH-less access procedure, we will also discuss the possible MRO enhancements for those aspects.

### Enhancement of RLF-Report

Logging specific access type, whether RACH-based or RACH-less LTM

In the last RAN2#127 meeting, the following is agreed regarding to RACH-based and RACH-Less LTM type:

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

The ra-InformationCommon field in RLF-Report is used to indicate the RA related information according to TS38.331:

1> if connectionFailureType is hof and if the failed handover is an intra-RAT handover: 2> set the ra-InformationCommon to include the random-access related information as described in clause 5.7.10.5;

In case of LTM cell switch failure, if ra-InformationCommon is included in the RLF-Report, the gNB can deduce that it is a RACH-based LTM cell switch failure. Thus, in case when connectionFailureType is hof and the extended lastHO-Type indicates LTM cell switch, the present of ra-InformationCommon can implicitly indicate if it is RACH-based or RACH-less LTM.

- In case of LTM cell switch failure, the presence of ra-InformationCommon can implicitly indicate if LTM cell switch is RACH-based or RACH-less LTM.

However, after a successful LTM cell switch, if radio link failure happens shortly after, there will be no implicit or explicit indication if the previous successfully executed LTM is RACH-less or RACH-based. To detect the root cause of RLF, gNB may need to be informed if the previous LTM was RACH-less or RACH-based. lastHO-Type which is agreed to be extended to indicate LTM cell switch can be further extended to indicate if it is RACH-less or RACH-based LTM. Alternatively, a new field can be introduced to indicate the specific access type, which can also benefit normal handover, except the LTM cell switch, to differentiate RACH-less and RACH-based access type.

In case of RLF after the successful LTM cell switch, RAN2 agree to explicitly indicate whether the last executed LTM was RACH-less or RACH-based.

We also note that additional information that would be valuable for optimization includes whether the UE switched beam in the target cell prior to the RLF. To this end, it would be beneficial if the UE also includes information indicating whether the UE has switched the beam in target cell or not prior to the RLF. If the UE changes the beam in the target cell, perhaps it is the target node that need to take actions to optimize the UE configuration, e.g. selection of candidate beams/TCI states, but if the UE has not changed the beam prior to the failure in the target cell, the DU controlling the source cell of the LTM cell switch should take counter actions to avoid failures for subsequentLTM cell switches. In light of above, we propose the following.

In case of RLF after the successful LTM cell switch, RAN2 should agree that the RLF report should indicate whether the UE has switched the beam in the target cell prior to the RLF in the target cell.

Indication of LTM candidate cell in RLF report

Last RAN2 meeting agrees:

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

However, for LTM candidate cell indication, only SHR is reached in this agreement, there is no discussion yet on the LTM candidate cell indication in RLF report.

In RAN2#126 meeting, L1 measurement is agreed to be logged in RLF report. Thus, if there are L1 measurement results reported for a cell in RLF report, then the NW can understand that this cell is an LTM candidate cell. However, there may still be no available L1 measurements even for a LTM candidate cell as the gNB may trigger LTM cell switch, e.g. based on L3 measurements. In a summary, the presence of L1 measurements for a cell can be seen as an implicit indication that it is a LTM candidate cell, on the other hand, the absence of L1 measurements for a cell cannot conclude that the cell is not an LTM candidate cell. Therefore, an explicit indication of whether a neighbour cell is an LTM candidate cell or not is needed.

Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell, when L1 measurement is not available for the candidate cell.

Logging beam information

The LTM cell switch command includes the target beam to be used by the UE when executing the LTM cell switch. However, the beam indicated in the LTM cell switch command may be sub-optimal and the LTM cell switch may fail due to a bad choice of target beam, the UE may re-establish/recover or reconnect to the same cell but in a better beam or in a different cell.

Currently the UE logs the target cell ID, the re-established cell ID and, when applicable, the reconnected cell ID in the RLF report. Given that the LTM cell switch relies heavily on the beam information, the beam information can also help the network to optimize future LTM cell switch procedures, in particular in selecting the optimal beam for early DL sync and in the LTM cell switch command and also in the selection of LTM candidate beams.

Enhance the RLF report with beam information of.

  - the beam toward which the UE failed to perform LTM cell switch,

  - The beam indicated in the last successful LTM cell switch command, when failure occurs shortly after the LTM cell switch,

  - The beam the UE succeeded to recover/re-establish or reconnect in.

Logging early DL/UL sync information in the RLF report

In the LTM procedure, before receiving the cell switch command, the UE may acquire DL sync in the LTM candidate cell(s) as triggered by the gNB-DU. The network may activate and deactivate TCI states of LTM candidate cell(s) for early DL sync by sending the Candidate Cell TCI States Activation/Deactivation MAC CE.

The UE may also obtain UL sync with LTM candidate cell(s) using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, if triggered by a PDCCH order from the gNB (in the latter case the UE will receive the TA in the cell switch command). Given that these new features (as of now designed for the LTM procedure) play important roles in the performance of LTM (in terms of interruption time) we believe optimization of these features is necessary.

In case of RLF/HOF, logging early DL/UL sync related information in the RLF report enables the network to determine if any optimization is needed for the LTM early DL/UL sync configurations and procedures.

Early DL sync related information should be logged in the RLF report. Detailed information is FFS

Early UL sync related information should be logged in the RLF report. Detailed information is FFS

We note that if the estimated TA value turns out to be incorrect (irrespective of early UL sync method), e.g. it becomes outdated because the UE significantly relocated/moved between the time of the early UL sync estimation and the time of the LTM execution, this may lead to failure of RACH-less LTM cell switch. In addition, if there is no available estimated TA value (because early UL sync was not configured or was configured but failed) at the time of the LTM execution, the UE will execute a RACH-based LTM cell switch procedure, which is likely to lead to longer interruption time than the RACH-less LTM cell switch procedure. Hence, in our understanding, the early UL sync procedure and in particular TA estimation validity and accuracy (which is loosely correlated with the age of the TA when the LTM cell switch is executed) will impact the LTM cell switch performance. In light of the above, we propose the following.

RAN2 to discuss the following information regarding the early UL sync to be included in the RLF report.

  - TA estimation method, whether UE based TA estimation or PDCCH ordered TA acquisition,

  - Time since the last PDCCH ordered PRACH towards the recovery/re-establishment cell till connection failure,

  - Time since the PDCCH ordered PRACH transmission till the reception of the LTM cell switch command,

  - Indication of whether there is valid UE based TA estimation towards the recovery/re-establishment cell before connection failure,

  - Time between the last UE autonomous TA estimation and the reception of the LTM cell switch command,

  - Value for UE estimated TA,

  - TA difference between UE estimated TA and TA received in RAR for recovery/re-establishment cell,

  - TA difference between TA in LTM cell switch command and the TA estimated by UE, if available

Proper early DL sync can reduce the interruption time for LTM cell switch, and the time when the early DL sync is triggered is important, for example, if the early DL sync is triggered too early, the UE may need to keep monitoring the candidate cell for a long time before LTM cell switch, which is power consuming. Besides, the information on whether there is an early DL sync towards to a candidate cell or beam in case of connection failure can help to optimize the early DL sync configuration and activation for later LTM cell switch procedure. In light of the above, we propose the following:

RAN2 to discuss the following information regarding the early DL sync to be included in the RLF report.

  - Time since early DL sync towards the target cell, recovery/re-establishment cell, or any LTM candidate cell till connection failure,

  - indication of whether there is early DL sync towards the target cell, recovery/re-establish cell, or any LTM candidate cell.

### Enhancement of SHR

Logging L1 measurements in SHR

Similar to the inclusion of L1 measurement results in the RLF report, which was agreed in RAN2 #126, in case of near failure (suboptimal) LTM procedure, the network cannot only rely on L3 measurement in the SHR to verify the neighbouring cells’ status (and the neighbouring beams’ status) for the sake of LTM optimization. In other words, since the network makes decisions based on the L1 measurements, enhancing the SHR with the L1 measurement results would provide a better insight for the network to tune suboptimal LTM configurations.

UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

Indication of LTM candidate cell in SHR

Last RAN2 meeting agrees:

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

If Proposal 7 can be agreed, similar with the problem of LTM candidate cell indication in RLF report, an explicit indication of whether a neighbour cell is an LTM candidate cell or not is still needed if there are no available L1 measurements for the cell. Introducing of the explicit indication should take both RLF report and SHR into account, and try to use unified indicator or similar way of indication in both reports as much as possible.

Introduce an explicit indication in SHR to indicate whether a neighbour cell is an LTM candidate cell, if L1 measurements are not available for such cell.

Successful LTM cell switch to wrong beam

As discussed above, the beam indicated in the LTM cell switch command may be wrong or suboptimal. With the wrong or suboptimal beam, the LTM cell switch can still be successful, but beam failure may occur shortly afterwards. The UE then executes the beam failure recovery procedure towards a better beam in the same cell. To report this scenario by SHR may require a relatively significant change, as the current SHR determination is performed upon successful completion of the RA or mobility procedure and the SHR availability indication successHO-InfoAvailable is included in Msg5, whereas the BFR may happen shortly afterwards.

An alternative solution to not change the procedure of SHR reporting is to use a NW-based solution, if BFR happens shortly after the successful LTM cell switch, the target DU informs the source DU about the LTM cell switch to wrong beam.

RAN2 to discuss the scenario of successful LTM cell switch to wrong beam i.e. BFD followed by BFR shortly after the successful LTM cell switch.

## SON features for CHO with candidate SCGs

### Time information regarding condition fulfilment

In RAN2#127 meeting, the following was agreed-

UE includes following information in RLF report:

b.    Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

In our view, the agreement to log time information regarding condition fulfilment for CHO with candidate SCG requires further clarification. In this context, both CHO and CPC configuration can each have two triggering conditions configured which may trigger at different times (and together they constitute the execution condition). Hence, different time information can be logged. In the following table, we summarize all the possible time differences, without considering the time between two events of the same HO, i.e. time between CHO events, and time between CPC events. That is because the time between CHO events is already considered in legacy, and it can just be reused in the context of CHO with candidate SCG; whereas  for the time between CPC events, a new time to be logged in RLF-Report could be introduced:

|  | t1 | t2 | t3 | t4 | Time that can be logged |
| --- | --- | --- | --- | --- | --- |
| Scenario-1 | 1st triggering condition of CHO fulfilled | 1st triggering condition of CPC fulfilled | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | t4-t3 t4-t1 t2-t1 t3-t2 |
| Scenario-2 | 1st triggering condition of CHO fulfilled | 1st triggering condition of CPC fulfilled | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | t4-t3 t4-t2 t2-t1 t3-t1 |
| Scenario-3 | 1st triggering condition of CPC fulfilled | 1st triggering condition of CHO fulfilled | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | t4-t3 t4-t2 t2-t1 t3-t1 |
| Scenario-4 | 1st triggering condition of CPC fulfilled | 1st triggering condition of CHO fulfilled | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | t4-t3 t4-t2 t2-t1 t3-t1 |
| Scenario-5 | 1st triggering condition of CPC fulfilled | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | 1st triggering condition of CHO fulfilled | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | t4-t2 t3-t1 |
| Scenario-6 | 1st triggering condition of CHO fulfilled | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | 1st triggering condition of CPC fulfilled | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | t4-t2 t3-t1 |

Since, the CHO (or CPC) execution condition is fulfilled once both triggering conditions are fulfilled, the reported time should be the difference between fulfilments of the second triggering conditions of the respective CHO/CPC configurations, as marked green in the table.

- Since for CHO (or CPC) execution condition is fulfilled once both triggering conditions are fulfilled (assuming two configured triggering conditions in the most general case), the reported time difference should be the time between the point when the CHO (or CPC) has both triggering conditions fulfilled and the point when the other CPC (or CHO) has both triggering condition fulfilled.

However, if only one or none of the CHO (or CPC) execution condition is fulfilled when the UE declares RLF, and it is required to log the time between RLF and fulfilment of the execution condition. Different scenarios arise for RLF also, as summarised below for the scenarios where three triggering conditions are fulfilled (i.e. the execution condition for one of CHO and CPC, and one triggering condition for the other one of CHO and CPC):

|  | t1 | t2 | t3 | t4 | Time that can be logged |
| --- | --- | --- | --- | --- | --- |
| Scenario-1 | 1st triggering condition of CHO fulfilled | 1st triggering condition of CPC fulfilled | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | RLF | t4-t3 t4-t1 t4-t2 |
| Scenario-2 | 1st triggering condition of CPC fulfilled | 1st triggering condition of CHO fulfilled | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | RLF | t4-t3 t4-t2 t4-t1 |
| Scenario-3 | 1st triggering condition of CPC fulfilled | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | 1st triggering condition of CHO fulfilled | RLF | t4-t2 t3-t1 |
| Scenario-4 | 1st triggering condition of CHO fulfilled | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | 1st triggering condition of CPC fulfilled | RLF | t4-t2 t3-t1 |
| Scenario-5 | 1st triggering condition of CPC fulfilled | 1st triggering condition of CHO fulfilled | 2nd triggering condition of CHO fulfilled (CHO execution condition fulfilled) | RLF | t4-t3 t4-t2 t4-t1 |
| Scenario-6 | 1st triggering condition of CHO fulfilled | 1st triggering condition of CPC fulfilled | 2nd triggering condition of CPC fulfilled (CPC execution condition fulfilled) | RLF | t4-t3 t4-t2 t4-t1 |

In these scenarios, the UE may log the time between RLF and fulfilment of first triggering condition of CHO (or CPC). However, to be consistent with the previous table, the UE may log the time between RLF and fulfilment of the second triggering condition, i.e., when the CHO (or CPC) execution condition is fulfilled. However, in the example above, if at the moment of RLF, only one triggering condition is fulfilled, either for the CHO or CPC, then it is considered that neither the CHO nor the CPC execution condition is fulfilled, and no time difference with respect to the RLF will be logged. This approach is the same as in legacy Rel.17 SON for CHO, in which case the timeBetweenEvents is also logged only in cases in which both CHO triggering conditions were fulfillled at time of failure.

If HOF is detected during execution of “CHO with candidate SCG” or SHR is generated, the UE includes the following information in the RLF-Report or SHR:

- Indication of which execution condition (CHO or CPC), was fulfilled first in time.

- The time gap between the fulfillment of second triggering condition of CHO execution conditions and the fulfillment of second triggering condition of CPC execution condition, or viceversa in case the second triggering condition of CPC execution condition is fulfilled after the second triggering condition of CHO execution conditions.

If the RLF is detected before the execution of the “CHO with candidate SCG”, and one of the execution conditions (i.e. the execution condition for CHO or the execution condition for CPC) is fulfilled , the UE includes the following information in the RLF-Report:

  - Indication of which execution condition (CHO or CPC) was fulfilled.

  - The time gap between the fulfillment of the second triggering condition, i.e. the fulfillment of the execution condition (for CHO or CPC) and the RLF detection.

On other hand, in the legacy RLF report, information regarding the CHO triggering condition fulfilment is logged. The information includes the CHO configuration, time between the fulfilments of the triggering conditions and information regarding the first triggered condition.

- In Rel. 17 SON for CHO, the UE includes information regarding triggering conditions for CHO in the RLF report.

Since, in CHO with candidate SGC, execution of the mobility procedure depends on fulfilling both CHO and CPC execution conditions, similar information for CPC needs to be included in the RLF report.

RAN2 agree to include information regarding the CPC triggering conditions as part of the RLF report similar to the information regarding CHO triggering conditions, i.e. first CPC triggered condition, and time between fulfilments of the two triggered CPC triggering conditions if both are triggered.

Furthermore, if the UE declares RLF after successfully performing CHO with candidate SCG, it will generate an RLF report. It is important for the network to understand from the RLF report that the last successfully executed mobility procedure was a CHO with candidate SCG. Hence, the IE for the type of thelast executed mobility procedure, i.e. the lastHO-Type IE, should be extended with a new value indicating CHO with candidate SCG in the RLF report.

RAN2 agree to define a new value for the lastHO-Type IE in the RLF report for CHO with Candidate SCG.

### Measurement result for serving PSCell

In RAN2#127 meeting, the following was also agreed:

UE includes following information in RLF report:

c.    Measurement results of PCells and PSCells

From TS 38.331- Section 5.3.10.5, the below is found regarding measurement results logging in RLF report-

1> if the SS/PBCH block-based measurement quantities are available: 2> set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure; 1> if the CSI-RS based measurement quantities are available: 2> set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure; 1> for each of the configured measObjectNR in which measurements are available: 2> if the SS/PBCH block-based measurement quantities are available: 3> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure; 4> for each neighbour cell included, include the optional fields that are available; NOTE 0a: For the neighboring cells included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, UE also includes the CSI-RS based measurement quantities, if available. 2> if the CSI-RS based measurement quantities are available: 3> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure; 4> for each neighbour cell included, include the optional fields that are available;

As we can see from the highlighted text above, in legacy the UE includes the measurement results associated to the PCell in the measResultLastServCell, and all the other measurement result for each configured measurement object inthe measResultNeighCells. Since PSCells are part of measObjectNR, the UE would include the measurement results in the measResultNeighCells, according to the legacy procedure.

- According to legacy procedures, the UE includes the measurement results of the serving PCell/source PCell in measResultLastServCell and all the other measurement results for each measurement object (including the PSCell) in measResultNeighCells within the RLF report.

However, from these measurement results included in measResultNeighCells, the UE does not indicate which measurement result correspond to the serving PSCell, which is an important information for the network to know, especially in the case of CHO with candidate SCG wherein the fulfilment (or not fullfillment) of the CPC triggering conditions may play an important role in the radio link failure or mobility failure.

- According to legacy procedures, in the RLF-Report, the UE does not include any information indicating whether certain the measurement results are related to the serving PSCell/source PSCell.

In our view, either the PSCell related radio measurements are logged separately (as for the case of the PCell in legacy), or an indication is included in measResultNeighCells to indicated whether a measurement result is associated to the PSCell.

RAN2 to discuss the options to include measurement results for last serving PSCell/source PSCell:

  - Include PSCell measurement result as separate IE in the RLF report (similar as the measResultLastServCell for the PCell).

  - Indicate the PSCell in the entry of the list of neighbor cell measurements (i.e. measResultNeighCells) associated to the PSCell measurements.

### Correlation between SHR and SPR

Furthermore, since the UE performs both PCell handover and PSCell change, it is possible that it may generate both an SHR and an SPR, if configured by the network. In our view, the SHR and SPR are correlated, and the network needs to consider this correlation while analyzing the reports.

RAN2 to discuss how to indicate correlation between SHR and SPR, if the SHR and SPR are generated because the UE performed simultaneous PCell handover and PSCell change due to CHO with candidate SCG.

## SON for Subsequent CPAC

In RAN2#126, it was agreed that RAN2 will look into failure and near failure scenarios for S-CPAC to see what, if any, MRO enhancements are needed. In RAN2#127, there was a proposal to include S-CPAC related information in the Mobility History Information (MHI) [1]. With the current specification, MHI will include the visited PSCells also during S-CPAC, but it will not be visible from the MHI whether a cell addition/change was made based on a S-CPAC configuration or not. Such information could be used for optimizing the list of candidate cells for S-CPAC, however it would not be needed to address the failure scenario or near failure scenario.

- Information on whether a cell addition/change was made based on a S-CPAC configuration or not is not needed to address the failure scenario or near failure scenario.

Based on this, we propose the following.

No MHI enhancements are needed to support MRO for S-CPAC.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In case of LTM cell switch failure, the presence of ra-InformationCommon can implicitly indicate if LTM cell switch is RACH-based or RACH-less LTM.

Observation 2	Since for CHO (or CPC) execution condition is fulfilled once both triggering conditions are fulfilled (assuming two configured triggering conditions in the most general case), the reported time difference should be the time between the point when the CHO (or CPC) has both triggering conditions fulfilled and the point when the other CPC (or CHO) has both triggering condition fulfilled.

Observation 3	In Rel. 17 SON for CHO, the UE includes information regarding triggering conditions for CHO in the RLF report.

Observation 4	According to legacy procedures, the UE includes the measurement results of the serving PCell/source PCell in measResultLastServCell and all the other measurement results for each measurement object (including the PSCell) in measResultNeighCells within the RLF report.

Observation 5	According to legacy procedures, in the RLF-Report, the UE does not include any information indicating whether certain the measurement results are related to the serving PSCell/source PSCell.

Observation 6	Information on whether a cell addition/change was made based on a S-CPAC configuration or not is not needed to address the failure scenario or near failure scenario.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In case of RLF after the successful LTM cell switch, RAN2 agree to explicitly indicate whether the last executed LTM was RACH-less or RACH-based.

Proposal 2	In case of RLF after the successful LTM cell switch, RAN2 should agree that the RLF report should indicate whether the UE has switched the beam in the target cell prior to the RLF in the target cell.

Proposal 3	Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell, when L1 measurement is not available for the candidate cell.

Proposal 4	Enhance the RLF report with beam information of.

a.	the beam toward which the UE failed to perform LTM cell switch,

b.	The beam indicated in the last successful LTM cell switch command, when failure occurs shortly after the LTM cell switch,

c.	The beam the UE succeeded to recover/re-establish or reconnect in.

Proposal 5	Early DL sync related information should be logged in the RLF report. Detailed information is FFS

Proposal 6	Early UL sync related information should be logged in the RLF report. Detailed information is FFS

Proposal 7	RAN2 to discuss the following information regarding the early UL sync to be included in the RLF report.

a.	TA estimation method, whether UE based TA estimation or PDCCH ordered TA acquisition,

b.	Time since the last PDCCH ordered PRACH towards the recovery/re-establishment cell till connection failure,

c.	Time since the PDCCH ordered PRACH transmission till the reception of the LTM cell switch command,

d.	Indication of whether there is valid UE based TA estimation towards the recovery/re-establishment cell before connection failure,

e.	Time between the last UE autonomous TA estimation and the reception of the LTM cell switch command,

f.	Value for UE estimated TA,

g.	TA difference between UE estimated TA and TA received in RAR for recovery/re-establishment cell,

h.	TA difference between TA in LTM cell switch command and the TA estimated by UE, if available

Proposal 8	RAN2 to discuss the following information regarding the early DL sync to be included in the RLF report.

a.	Time since early DL sync towards the target cell, recovery/re-establishment cell, or any LTM candidate cell till connection failure,

b.	indication of whether there is early DL sync towards the target cell, recovery/re-establish cell, or any LTM candidate cell.

Proposal 9	UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

Proposal 10	Introduce an explicit indication in SHR to indicate whether a neighbour cell is an LTM candidate cell, if L1 measurements are not available for such cell.

Proposal 11	RAN2 to discuss the scenario of successful LTM cell switch to wrong beam i.e. BFD followed by BFR shortly after the successful LTM cell switch.

Proposal 12	If HOF is detected during execution of “CHO with candidate SCG” or SHR is generated, the UE includes the following information in the RLF-Report or SHR:

a.	Indication of which execution condition (CHO or CPC), was fulfilled first in time.

b.	The time gap between the fulfillment of second triggering condition of CHO execution conditions and the fulfillment of second triggering condition of CPC execution condition, or viceversa in case the second triggering condition of CPC execution condition is fulfilled after the second triggering condition of CHO execution conditions.

Proposal 13	If the RLF is detected before the execution of the “CHO with candidate SCG”, and one of the execution conditions (i.e. the execution condition for CHO or the execution condition for CPC) is fulfilled , the UE includes the following information in the RLF-Report:

a.	Indication of which execution condition (CHO or CPC) was fulfilled.

b.	The time gap between the fulfillment of the second triggering condition, i.e. the fulfillment of the execution condition (for CHO or CPC) and the RLF detection.

Proposal 14	RAN2 agree to include information regarding the CPC triggering conditions as part of the RLF report similar to the information regarding CHO triggering conditions, i.e. first CPC triggered condition, and time between fulfilments of the two triggered CPC triggering conditions if both are triggered.

Proposal 15	RAN2 agree to define a new value for the lastHO-Type IE in the RLF report for CHO with Candidate SCG.

Proposal 16	RAN2 to discuss the options to include measurement results for last serving PSCell/source PSCell:

a.	Include PSCell measurement result as separate IE in the RLF report (similar as the measResultLastServCell for the PCell).

b.	Indicate the PSCell in the entry of the list of neighbor cell measurements (i.e. measResultNeighCells) associated to the PSCell measurements.

Proposal 17	RAN2 to discuss how to indicate correlation between SHR and SPR, if the SHR and SPR are generated because the UE performed simultaneous PCell handover and PSCell change due to CHO with candidate SCG.

Proposal 18	No MHI enhancements are needed to support MRO for S-CPAC.

# References

R2-2406959, Discussion on MRO enhancements for R18 mobility features, RAN2, 3GPP TSG RAN WG2#127, Maastricht, Netherlands, 19th - 23rd August 2024.