3GPP TSG-RAN WG2 Meeting #127bis	R2-2408831

Hefei, China, 14 – 18 October 2024

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on UL capacity enhancement for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As one target of WID[1], support of capacity enhancement for UL needs to be studied and specified. One of the motivations is to reduce the necessary uplink and downlink signalling to complete an Early Data Transmission (EDT) transaction. More specifically, it includes enhancement to transmit Msg3 without Msg1/Msg2 and enhancement to efficiently deliver Msg4.

- Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2-27 meeting, RAN2 agreed to continue working on a Contention-Based (CB) msg3 EDT-like mechanism.

Agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In this contribution, we provide our view on possible enhancements for EDT transaction for IoT NTN.

# 2	Discussion

For Contention-Based Msg3 transmission based on Shared resources (CBS-Msg3 transmission), it is assumed UE shall complete an Msg3 transaction without Msg1/Msg2. Therefore, the UE needs to autonomously pre-compensate the TA for Msg3 transmission in which the TA is estimated by UE based on its GNSS position, the NW broadcasted satellite ephemeris, and the Common TA parameters (in case the transparent architecture is used). In the previous discussions, RAN2 is not clear whether the UE estimated TA is accurate enough for the direct Msg3 transmission, hence RAN2 decided to send LS [2] to RAN1 and RAN4 to clarify the feasibility of the direct Msg3 transmission without Msg1/Msg2.

In LS response [3], RAN1 confirms that, for eMTC CE Mode A and NB-IoT with 3.75 kHz SCS, an RRC Idle UE with a pre-compensated TA can satisfy the timing requirement for Msg3 transmission/reception with the assumption of NTA=0. But for eMTC CE Mode B and NB-IoT with 15 kHz, RAN1 indicates it may need further discussion. Similarly, in LS response [4], RAN4 confirms UE can meet the UL transmission timing requirement from UE RRM point of view. But RAN4 observes that for NB-IoT with 15kHz SCS, the timing error may be larger than half CP. Based on the responses, we think RAN2 can conclude the CBS-Msg3 transmission can be supported for eMTC CE Mode A and NB-IoT with 3.75 kHz SCS for both LEO and GEO, but it is up to RAN1 and RAN4 to decide whether the CBS-Msg3 transmission can be supported on the eMTC CE Mode B and NB-IoT with 15 kHz SCS.

Proposal 1: RAN2 confirms the CBS-Msg3 EDT-like mechanism can be supported for eMTC CE Mode A and NB-IoT with 3.75 kHz SCS in both LEO and GEO.

Proposal 2: RAN2 waits for further feedback from RAN1 and RAN4 on whether CBS-Msg3 EDT-like mechanism can be supported for eMTC CE Mode B and NB-IoT with 15 kHz SCS.

CBS-Msg3 resource configuration:

On how to configure the CBS-Msg3 shared resource, there are some differences for GEO and LEO cases. In GEO-based cells the UEs may not experience the change of a serving cell, unless the UEs are mobile. Therefore, the use of dedicated signalling for CBS-Msg3 configuration seems quite applicable to GEO cells. The legacy mechanism to configure PUR resource (via RRCConnectionRelease message) can be reused and the resource configuration is only valid in the cell where the configuration was received. On the other hand, for UE in LEO cells, it will experience a cell change frequently, independently of whether Earth-fixed or Earth-moving cells are deployed. It is not suitable to use the dedicated signalling to configure the CBS-Msg3 resource. Otherwise, the UE may need to enter RRC Connected mode to get the configuration when it enters each new cell. Instead, if the configuration is provided in system information for LEO case, the solution is valid for all kinds of UEs including the UE accessing the network for the first time. In our understanding, the system information broadcasted CBS-Msg3 resource is also appliable to GEO case.

Proposal 3: System Information is used to configure CBS-Msg3 resource.

In legacy eMTC and NB-IoT, the PRACH configuration is configured per-CE level. The UE selects the corresponding Preamble resource from proper CE level based on the measured RSRP level against the CE level RSRP-threshold. For direct Msg3 transmission in CBS-Msg3 resource, the similar mechanism can be adopted. The RSRP-based CE level selection will ensure the uplink configuration in terms of MCS and resources is suitable for the current radio conditions. The UE should select the proper CBS-Msg3 resource based on CE level determination according to the measured RSRP and CE level RSRP-threshold.

Proposal 4: Support per-CE level configuration for shared CBS-Msg3 resource.

CBS-Msg3 resource usage control:

For each of the CE level, the configured CBS-Msg3 resource is shared between UEs and multiple UEs may transmit Msg3 simultaneously, thus it may cause a collision which will bring some adverse consequence such as the increased signalling overhead, additional UE power consumption and waste of the resources. If the CBS-Msg3 configuration is provided to UE via SIB, the NW has no idea how many idle UEs will use the resource simultaneously which may cause unexpected Msg3 collision. Therefore, we believe the NW should have a mechanism to control the possible Msg3 collision in order to mitigate the Msg3 collision impact.

Proposal 5: If the shared CBS-Msg3 resource is provided via SIB, a mechanism to control the Msg3 collision should be supported. FFS on the details.

If CBS-Msg3 resource is configured by NW, the presence of the resource configuration can be regarded as a common control flag to indicate that UEs can use the shared resource for direct Msg3 transmission in cell level. Furthermore, for the TA estimation for direct Msg3 transmission, it requires accurate GNSS information for the correct TA estimation for PUSCH transmission. But the TA estimation inaccuracy may be caused by poor GNSS or other factors, hence the corresponding PUSCH transmission may interfere other UEs or cost consecutive TA adjustment from NW when the direct Msg3 transmission is initiated. Considering the large number of repetitions can be configured for Msg3 transmission which may interfere other UEs a lot, it is beneficial for the NW to have an approach to disable a specific UE to use direct Msg3 transmission. For example, the NW may disable the use of any future direct Msg3 transmission for a specific UE via a dedicated RRC message when it was in RRC Connected state.

Proposal 6: The presence/absence of the shared CBS-Msg3 resource configuration can be regarded as common control flag to enable/disable direct Msg3 transmission in cell level.

Proposal 7: Direct Msg3 transmission can be disabled for a specific UE via UE-specific signalling.

CBS-RNTI and Contention Resolution:

Since there is no Msg2 for CBS-Msg3 transmission, the RNTI to be used for Msg3 transmission cannot be indicated in RAR. On how to decide the CBS-RNTI for a Msg3 transmission, there may be two options. The first option is to use a common RNTI for all the CBS-Msg3 resource occasions. The other option is to use the RNTI derived from the Msg3-CBS resource to be used for the PUSCH transmission. The first option seems simple as only one RNTI is used by the UE and NW and it is convenient to enable multiplexing more than one contention resolution to multiple UEs into one Msg4. However, it may need UE to monitor and receive more than one Msg4 to get the contention resolution MAC CE of its own. The issue can be addressed by the latter option since each CBS-Msg3 resource can have its own CBS-RNTI hence UE only needs to receive the Msg4 using the targeted CBS-RNTI. But the latter option needs more standardization effort to define the mapping between the RNTI and the CBS-Msg3 resources.

Proposal 8: RAN2 to discuss how to allocate CBS-RNTI for direct Msg3 transmission and corresponding Msg4 decoding, considering:

- Option1: Common RNTI for all CBS-Msg3 resources occasions

- Option2: RNTI derived from Msg3-CBS resource

In legacy 4-step RACH the RACH collisions are handled using the contention resolution scheme, which relies on uniquely identifying the UEs in Msg3 and NW acknowledgement in Msg4. In legacy PUR mechanism (Msg3 transmission without Msg1/RAR), there is no need for contention resolution since the PUR resource allocated via RRCConnectionRelease message is dedicated to a specific UE (i.e., contention free resource). For CBS-Msg3 transmission in shared resource, there may occur Msg3 collision between UEs. Therefore, the contention resolution scheme should be supported. The most straightforward way is to reuse the contention resolution scheme in 4-step RACH.

Proposal 9: Reuse the contention resolution scheme in legacy 4-step RACH for CBS-Msg3 transmission.

CBS-Msg3 procedure fallback:

The CBS-Msg3 procedure is subject to failures due to, e.g., RF conditions, interference or collision. Thus, another important issue to consider is the UE behavior in case of a CBS-Msg3 failure or consecutive CBS-Msg3 failures. For example, conditions for the UE to keep using CBS-Msg3 or to fall back to using, e.g., the legacy 4-step RACH procedure after a CBS-Msg3 failure or consecutive CBS-Msg3 failures need to be studied and specified. However, it is important to ensure that the UE may have a successful EDT within a time limit, assuming that the UE has periodic EDT as applied for the legacy PUR. This should be considered for fallback option after a CBS-Msg3 failure or consecutive CBS-Msg3 failures. Furthermore, if the per-CE level CBS-Msg3 resource are configured, it needs to be discussed how to consider the fallback between the CE levels. For example, if the consecutive CBS-Msg3 failures reached a maximum threshold in CE level 0, should the UE increase to CE level 1 (as legacy UE behavior for RACH) to have multiple CBS-Msg3 retries again, or the UE should fallback directly to the legacy 4-step RACH?

Proposal 10: RAN2 to discuss UE behavior in case of CBS-Msg3 procedure failure, including:

- CE level increase in case of consecutive CBS-Msg3 failures.

- Fallback options for the UE to ensure that the UE may have a successful EDT within a time limit.

Efficient delivery of msg4

On the objective of efficient delivery of msg4, according to the WID [1], the intention is to identify methods to de-couple the UL from the DL as much as possible hence unlock the additional UL capacity potential. We understand it is to avoid the PDCCH/PDSCH congestion which may impact the UL scheduling as well as the UL capacity. Therefore, how to reduce the Msg4 overhead should be studied.

In legacy, a dedicated Msg4 with both PDCCH and PDSCH is always needed for the corresponding Msg3 for contention resolution. To reduce the Mgs4 overhead, one possible enhancement is to reduce the PDCCH usage for Msg4 scheduling. For example, it seems a possible optimization way-forward to support one Msg4 to deliver contention resolution result for multiple UEs or preconfigure the PDSCH for Msg4 delivery to save the scheduling PDCCH at least. The other possible enhancement is to reduce the PDSCH overhead for Msg4 content delivery. For example, some of the IEs in Msg4 can be reduced or reused, especially considering the case when multiple UEs can be scheduled in a single Msg4.

Proposal 11: To support efficient delivery of Msg4, RAN2 to study on how to reduce the PDCCH usage for Msg4 scheduling and the PDSCH usage for Msg4 transmission.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: RAN2 confirms the CBS-Msg3 EDT-like mechanism can be supported for eMTC CE Mode A and NB-IoT with 3.75 kHz SCS in both LEO and GEO.

Proposal 2: RAN2 waits for further feedback from RAN1 and RAN4 on whether CBS-Msg3 EDT-like mechanism can be supported for eMTC CE Mode B and NB-IoT with 15 kHz SCS.

Proposal 3: System Information is used to configure CBS-Msg3 resource.

Proposal 4: Support per-CE level configuration for shared CBS-Msg3 resource.

Proposal 5: If the shared CBS-Msg3 resource is provided via SIB, a mechanism to control the Msg3 collision should be supported. FFS on the details.

Proposal 6: The presence/absence of the shared CBS-Msg3 resource configuration can be regarded as common control flag to enable/disable direct Msg3 transmission in cell level.

Proposal 7: Direct Msg3 transmission can be disabled for a specific UE via UE-specific signalling.

Proposal 8: RAN2 to discuss how to allocate CBS-RNTI for direct Msg3 transmission and corresponding Msg4 decoding, considering:

- Option1: Common RNTI for all CBS-Msg3 resources occasions

- Option2: RNTI derived from Msg3-CBS resource

Proposal 9: Reuse the contention resolution scheme in legacy 4-step RACH for CBS-Msg3 transmission.

Proposal 10: RAN2 to discuss UE behavior in case of CBS-Msg3 procedure failure, including:

- CE level increase in case of consecutive CBS-Msg3 failures.

- Fallback options for the UE to ensure that the UE may have a successful EDT within a time limit.

Proposal 11: To support efficient delivery of Msg4, RAN2 to study on how to reduce the PDCCH usage for Msg4 scheduling and the PDSCH usage for Msg4 transmission.

# Reference

[1] RP- RP-242397 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] R2-2405769 LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

[3] R1-2407548 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

[4]	R4-2414114	Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2