3GPP TSG-RAN WG2 Meeting #127bis                                                             R2-2408834

Hefei, China, Oct 14th – 18rd, 2024

Agenda Item	:	8.2.4

Title	:	Discussion on multiple access for ambient IoT

Source	:	Google Ireland Limited

Work Item	:	FS_Ambient_IoT_solutions

Document for	:	Discussion and Decision

# 1		Introduction

In this paper, we discuss how to obtain AS ID and perform re-access when contention resolution fails. We also propose to discuss whether there is a need to introduce access/paging round, R2D round trigger, and R2D trigger to ambient IoT.

# 2	Discussion

In RFID, the reader (i.e. interrogator) transmits a Select command to select devices (i.e. tags) and then transmits a Query command to start an inventory round. The inventory round consists of at least one contention period followed by a non-contention period. Devices contend to transmit a 16-bit random ID in the contention period. If the reader receives a random ID, the reader transmits an ACK to acknowledge the 16-bit random ID. Then the winning device transmits data in the non-contention period.

Fig. 1: RFID link timing

In the [POST127][033], the access round and the paging round are discussed. The access round is similar to the inventory round in RFIS. A R2D Round Trigger, similar to Query command, is used to initiate an access round. A R2D Trigger is used to start a contention period in an access round.

For RFID, there are some philosophy underlying the design of the Select, Query, and QueryRep commands. RAN2 should discuss whether there is a need to introduce similar designs, i.e. as access round, R2D Round Trigger message, and R2D Trigger message, to ambient IoT.

Proposal 1:  RAN2 to discuss whether to introduce access/paging round, R2D round trigger, and R2D trigger to ambient IoT.

In the RAN2 #127 meeting, RAN2 discussed the need for a AS ID and whether the AS ID is assigned by a reader. RAN1 also discussed a ID for R2D reception or D2R scheduling. Most companies think this AS ID is shorter than application layer ID.

There are different views on how to obtain the AS ID. One of the options is to have the reader assign a AS ID. The other option is to reuse the random ID included in Msg1. We think if AS ID is only used in the non-contention period following the Msg2, both options work, but it is simper to reuse the random ID.

However, if the AS ID is used during the whole access/paging round, the AS ID should be assigned by the reader in order to prevent collision. RAN2 should first discuss the usage of AS ID before determining how to obtain the AS ID.

Proposal 2:  RAN2 to discuss the use case of the AS ID before determining how to obtain the AS ID.

There are also some discussion on slotted ALOHA in the [POST127] [033]. Slotted ALOHA is a multiple access scheme based on the ALOHA multiple access. In ALOHA protocol, device can immediately transmit data when it has data in buffer. Slotted ALOHA improves the channel utilization of ALOHA by dividing time into equal-size slot. Device can only transmit data at the beginning of a time slot.

To distribute a large amount of devices in time requires a backoff scheme. In the legacy random access procedure, gNB may indicate a BI to UE. UE uses the BI to determine how long it should backoff before transmitting a random access preamble. In RFID, devices selects one of Q slots to transmit the random ID.

Backoff mechanism is also useful to ambient IoT, especially in the case that there are a large amount of IoT devices near the reader. For example, a device can be attached to each item on a rack in a warehouse. In such a case, a reader should be able to indicate a backoff value to spread devices’ transmissions in times.

Proposal 3:  Support backoff procedure for Msg1 transmission.

There are also some discussion on re-access, i.e. whether re-access should be in the same access round, next access round, or next paging round. In legacy random access procedure, if the contention resolution fails, UE will perform backoff and then transmit a random access preamble again.

For ambient IoT, device can follow a similar procedure to transmit Msg1. When device detects contention resolution fails, device chooses a new access occasion and transmit Msg1 in the new access occasion regardless of which access rounds.

Proposal 4:  If contention fails, device selects a new access occasion and transmit Msg1 in the new access occasion regardless of which access rounds.

# 3	Conclusions

In conclusion, we have the following proposals:

Proposal 1:  RAN2 to discuss whether to introduce access/paging round, R2D round trigger, and R2D trigger to ambient IoT.

Proposal 2:  RAN2 to discuss the use case of the AS ID before determining how to obtain the AS ID.

Proposal 3:  Support backoff procedure for Msg1 transmission.

Proposal 4:  If contention fails, device selects a new access occasion and transmit Msg1 in the new access occasion regardless of which access rounds.