3GPP TSG-RAN WG2 Meeting #127	bis						R2-2408838

Hefei, China, 14 - 18 October, 2024

Title: 	Discussion on NW-sided data collection for training

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3

Document for:	Discussion/Decision

- Introduction

In the last meeting, the following agreements are achieved for NW-sided data collection [1]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

There are still some FFSs that need to be analyzed. This paper will analyze the network-sided data collection and the FFSes.

- Discussion

Discussion on data reporting for low power state UE

In the last meeting, RAN2 agreed that measurements for data collection purposes and logging can be controlled based on the power state of the UE. Whether the UE stops measurement autonomously and reports to the NW still needs to be discussed. Due to the discussion of NW-sided data collection, we think the UE should notify the NW to inform about the low power state. Then, the NW indicates the UE to stop measurement.

Proposal 1: When the UE enters a low-power state, it should notify the NW to indicate the low-power state and wait for the NW to stop the data collection procedure.

Another issue is that the NW configures data collection configuration to the low power state UE. For example, one UE is in a low-power state and it receives the data collection configuration from the NW. However, the UE cannot or doesn’t want to collect the data due to the low power state. What the UE should do? This configuration should be configured as completed or failed? We think this scenario should be clarified in RAN2.

Proposal 2: RAN2 to discuss how the UE in a low power state should behave when it receives data collection configuration from the NW.

Discussion on multiple data collection configurations for NW-sided data collection

In the last meeting, there is an FFS for multiple configurations. Normally, one UE supports multiple AI use cases, e.g. CSI prediction, beam management, and positioning. It means that the UE may support collecting training data for different use case. However, we don’t see any requirements for multiple configurations for same use case. Thus, The NW can configure one configuration per use case, i.e. multiple configurations for the same use case are not supported.

Proposal 3: The NW can configure one configuration per use case, i.e. multiple configurations for the same use case are not supported.

Another FFS is whether dynamic activation/deactivation should be supported. Since we do not believe multiple configurations for the same use case are needed, then also dynamic activation/deactivation is not required. Even when multiple configurations per use case would be supported, dynamic activation/deactivation is still not a good solution for activating/deactivating the configurations in our view. This is because the NW should know the UE side situation before the NW activation/deactivation. For example, if the NW wants to collect the data in area #1, it needs to acquire the UE’s location in advance, or it needs some coordination between UE and NW in advance. It is also needed to configure triggering conditions for immediate UE location reports. Thus, the event-based logging configuration would serve the purpose better in this situation and should be considered with priority.

Proposal 4: For one specific use case, if there are more than one data collection configuration at the UE side, the dynamic activation/deactivation among these data collection configurations are not supported for NW side data collection.

Discussion on event-based reporting and on-demand request reporting

The network request training data is supported in the last meeting and there is an FFS on the details of the signaling. For the network request reporting, the logging configuration and contents of the collected data are configured by the NW in advance, i.e., these configurations have been sent before the NW requests the collected data. The contents of this configuration are similar to periodical reporting.

Regarding the on-demand request, once the NW sends the data collection configuration to the UE, the UE collects the data until the NW requests the collected data. However, if the NW sends the request message too late, the collected data may exceed the buffer limitation of the UE. Thus, we think it is reasonable that when the UE buffer becomes full, the UE can send data available indication to the NW so that the NW can collect the data from UE as soon as possible.

Proposal 5: Regarding the on-demand request, the UE can send data available indication to the NW when its buffer becomes full.

Another possible solution is that the NW configures the reporting condition in advance for data collection configuration. For example, the NW can configure the data volume threshold to the UE, then the UE send the data available indication to the NW when the data volume exceeds the threshold. Compared with the above proposal, this solution is flexible, and it could let NW knows the UE buffer status more efficiently. Thus, RAN2 can discuss whether the NW should configure the data reporting configuration to the UE, e.g.  data volume threshold for on-demand request.

Proposal 6: RAN2 to discuss whether the NW should configure the data reporting configuration to the UE, e.g. data volume threshold for on-demand requests.

If multiple configuration is supported, the UE can log the data based on the configurations and wait for NW to request the collected data. Even if the UE may have different training data for different use cases, the NW may not need all the collected data from the UE at the same time. For example, the gNB may be only interested in BM related training data collection for fine tuning the BM model(s), and it may not need training data for other use cases. Thus, we think that the NW can be able to request some data from the UE.

Proposal 7: The NW should be able to request the UE to report the data only for a certain data logging configuration (e.g. corresponding to a specific use case).

Discussion on the event-based data logging

In the last meeting, there is an FFS about event-based logging. In our opinion, once the UE gets the data collection configuration from NW side, the UE should do the data collection continuously, unless the UE thinks that it cannot be able to continue due to some serious reasons. In order to better discuss event-based data logging, it's worth looking at what are the reasons why the UE has to stop logging.

Proposal 8: It is proposed RAN2 to discuss why the UE has to suspend the data logging before discussing details of event-based data logging.

Discussion on RRC message Capacity

Issue analysis and solutions

In the last meeting, it was agreed that the UE stores the logged training data at the AS layer with a minimum AS layer memory size supported by the UE.  This minimum AS layer memory size is left FFS, which also needs to be covered across all use cases.

According to the RAN1 LS [3], the data sizes for different use cases are different. And RAN1 has no agreement on data size for each use case. Thus, RAN2 should wait for RAN1's progress on data size for each use case.

Proposal 9: The discussion of memory size can wait for the RAN1 progress.

Discussion on data compression for logged L1 measurement results

Currently, UDC (Uplink Data Compression) has been defined, and DEFLATE-based UDC solution could achieve higher compression efficiency which would save more uplink resources and reduce the transmission latency. The NG-RAN node can configure the UE to use UDC or not. If UDC is enabled, one byte UDC header is used to indicate whether the PDCP SDU is compressed by UDC or not. For UDC, the feature summary can be found in [4].

As shown in the performance evaluation [5], the KPI compression efficiency is defined as below:

Compression efficiency			1 – (output compressed data size / input uncompressed data size)

And then, UDC can achieve 40% ~ 80% compression efficiency for different service types.

The simulation results of UDC solution based on DEFLATE (solution 3) and Solution 4 (APDC) have shown a similar trend in terms of the compression efficiency for uplink on various input traffic profiles simulated including FTP, SIP signalling, video and web surfing in case 1 and case 2 type traffic scenarios. Wherein, about 40% to 50% compression efficiency is shown for FTP traffic, over 80% of compression efficiency is shown for SIP signalling and about 60% to 75% compression efficiency is shown for video traffic. Similarly, over 60% of compression efficiency can be obtained with web surfing data. From technical point of view, solution based on DEFLATE and solution based on APDC have shown significant and similar compression efficiency.

If UDC can be applied for RRC messages carrying logged L1 measurement results, the RRC signaling capacity problem can be alleviated. In other words, more logged L1 measurement results can be transmitted with the same RRC signaling.

Proposal 10: It is proposed that RAN2 discuss the necessity of data compression for logged L1 measurement results.

Discussion on the type of data required to be collected for NW sided model and UE-sided model

As indicated in RAN2#127bis agenda below, this section is to discuss type of data.

8.1.3 NW side data collection

Contributions should focus on the mechanisms and principles identified for data collection for network side model training during rel-18.   Contributions should discusss type of data required to be collected for NW sided model and UE sided model (common to NW sided and different).  Question to RAN1 should also be identified.

For beam management, the previous RAN1 LS [3] has provided some inputs. Here we copied the relevant part here.

**************************************************************

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |

Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on the existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. The payload size may not be fixed.

**************************************************************

Based on the LS [3], we observe that the required training data is generated from the UE side, and the data is common for both UE-side and NW-side models. The data content is standardized because Note 1 mentions that the data size can be calculated for some examples. Besides the two measurements, we have not seen other measurements for training data for BM, so they can be used for RAN2 discussions for now. Once RAN1 has more progress regarding the training data, RAN2 may review them later.

Observation 1: RAN1 has agreed that L1-RSRPs and/or beam-IDs are the training data for both UE-sided and NW-sided models. Both measurements are standardized.

Observation 2: Once RAN1 has more progress regarding the training data, RAN2 may review them later.

- Conclusion

In this paper, we mainly address the FFSes left from RAN2#127. We have the following observations and proposals:

Observation 1: RAN1 has agreed that L1-RSRPs and/or beam-IDs are the training data for both UE-sided and NW-sided models. Both measurements are standardized.

Observation 2: Once RAN1 has more progress regarding the training data, RAN2 may review them later.

Proposal 1: When the UE enters a low-power state, it should notify the NW to indicate the low-power state and wait for the NW to stop the data collection procedure.

Proposal 2: RAN2 to discuss how the UE in a low power state should behave when it receives data collection configuration from the NW.

Proposal 3: The NW can configure one configuration per use case, i.e. multiple configurations for the same use case are not supported.

Proposal 4: For one specific use case, if there are more than one data collection configuration at the UE side, the dynamic activation/deactivation among these data collection configurations are not supported for NW side data collection.

Proposal 5: Regarding the on-demand request, the UE can send data available indication to the NW when its buffer becomes full.

Proposal 6: RAN2 to discuss whether the NW should configure the data reporting configuration to the UE, e.g. data volume threshold for on-demand requests.

Proposal 7: The NW should be able to request the UE to report the data only for a certain data logging configuration (e.g. corresponding to a specific use case).

Proposal 8: It is proposed RAN2 to discuss why the UE has to suspend the data logging before discussing details of event-based data logging.

Proposal 9: The discussion of memory size can wait for the RAN1 progress.

Proposal 10: It is proposed that RAN2 discuss the necessity of data compression for logged L1 measurement results.

- References

RAN2#126 Chair Notes

TS 38.323

R2-2311720/R1-2310681, Reply LS on Data Collection Requirements and Assumptions, Source: RAN1, To: RAN2

RP-220142, Summary for WI NR Uplink Data Compression (UDC), CATT

TR 36.754, Study on UL data compression for E-UTRA