3GPP TSG-RAN WG2 Meeting #127bis	R2-2408840

# Heifei, China, 14 - 18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1185 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TS 38.306 for HST FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_Enh |  |  |  |  |  | Date: |  |  | 2024-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18 HST FR2, RAN2 agreed a CR to TS 38.331 (R2-2401565), which updates the RRC field description for highSpeedMeasFlagFR2-r17. According to the agreed CR, when highSpeedMeasFlagFR2-r17 is present, the RRM requirement that the UE shall apply is different for UEs with different UE capability. For example: - when highSpeedMeasFlagFR2-r17 is present, and the UE supports ue-PowerClass-v1700 set to pc6, the UE shall apply enhanced intra-frequency RRM requirement to the serving frequency of SpCell - when highSpeedMeasFlagFR2-r17 is present for SpCell, and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced inter-frequency RRM requirement In addition, according to TS 38.306, a UE supporting measEnhCAInterFreqFR2-r18 shall also indicate support of PC6 in ue-PowerClass-v1700. According to the above analysis, a UE supporting pc6 but not supporting measEnhCAInterFreqFR2-r18 shall apply one RRM requirement. A UE supporting both pc6 and measEnhCAInterFreqFR2-r18 shall apply another RRM requirement. However, in the current TS 38.306 description for ue-PowerClass does not distinguish different RRM requirements, as shown below, which does not align with TS 38.331 and causes misunderstanding. “UE indicating support for pc6 supports the enhanced intra-NR RRM and demodulation processing requirements for FR2 to support high speed up to 350 km/h as specified in TS 38.133 [5].” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the UE capability description for ue-PowerClass: “UE indicating support for pc6 supports the enhanced intra-frequency RRM requirement to the serving frequency of SpCell …” Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: HST FR2 Inter-operability: If the network is implemented according to the CR and the UE is not, the UE may be unclear about what RRM requirement the UE shall apply for. If the UE is implemented according to the CR and the network is not, the network may be unclear about what RRM requirement the UE shall apply for. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not agreed, the TS 38.306 does not align with TS 38.331, and causes confusion on which RRM requirement that the UE shall apply for. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.7.2	BandNR parameters

| ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 For FR1, if the UE supports the different UE power class than the default UE power class as defined in clause 6.2 of TS 38.101-1 [2], or in clause 6.2 of TS 38.101-5 [34], the UE shall report the supported UE power class in this field. For FR2, UE shall report the supported UE power class as defined in clause 6 and 7 of TS 38.101-2 [3] in this field. UE indicating support for pc6 supports the enhanced intra-frequency RRM requirement to the serving frequency of SpCell and demodulation processing requirements for FR2 to support high speed up to 350 km/h as specified in TS 38.133 [5]. This capability is not applicable to IAB-MT. The power class pc7 is only applicable for RedCap UEs operation in FR2. This capability is not applicable for UEs indicating support of maxOutputPowerATG-r18. | Band | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |