3GPP TSG-RAN WG2 Meeting #127bis                                                                   R2-2408847

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.3

Source:	China Telecom

Title:	Ambient IoT Paging Procedure

Document for:	Discussion

- Introduction

In the RAN#102 meeting, a new SID on solutions for Ambient IoT (Internet of Things) in NR was approved [1].

Agreements

1	Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

2	Wait for further RAN1 progress on indication of the start of access occasion.

Agreements

1	Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

2	Wait for further RAN1 progress on indication of the start of access occasion.

In RAN2#127 [1], the following agreements were made. The reader could trigger multiple/subsequent A-IoT paging for the same service request from CN. But how to avoid devices duplicate response for the same service request is FFS.

In RAN2#126[2], it was agreed to study the cases that device ID or group ID contained in A-IoT paging message to reach A-IoT device(s). Based on the above progress, this paper mainly analyses the paging procedure and functionality.

- Discussion

2.1 Multiple/Subsequent AIoT Paging

At the last meeting, RAN2 agreed to study how to avoid duplicated response from devices. But how to avoid devices duplicate response for the same service request is FFS. In general, the current agreements assume that paging always requires a response. This is clearly appropriate for the basic paging use cases: The reader needs to inventory one or multiple devices, receive data from device(s), or establish communication for data exchange.  However, Readers usually cannot reach all target devices within the coverage area through a single round of paging. The reason may be that some devices are not sufficiently charged, so they cannot respond to paging. Or there are a large number of devices, and there is contention for uplink resources when multiple devices attempt to respond.

There should be a mechanism for indicating which of the addressed device(s) should actually respond to a paging message. The principle of controlling the responses seems valid. Cause codes, explicit flags and a transaction identifier can serve as options to be considered. During the inventory or command process, the trigger message may optionally include filter criteria sent by the upper layer to identify the AIoT device that should respond to the inventory request or command message. The reader forwards the filter criteria from the inventory signal to the AIoT device. Before initiating an AIoT access, the AIoT device needs to check if the conditions fulfil the criteria.

In our view, the AIoT device should check the filter criteria provided by the reader before performing the AIoT access. Only if the conditions are met, the AIoT device is allowed to perform an AIoT access to the reader.

Proposal 1: The reader could include filter/mask criteria in the AIoT paging message sent to the AIoT device.

It previously agreed that an AIoT paging message may or may not contain any device ID, in the latter case it is considered to be addressed to all AIoT devices within the coverage of the reader that can receive the AIoT message. If a filtering criterion is introduced, RAN2 shall modify the previous protocol taking into account the (absence of) filtering criterion.

Proposal 2: If no filter/mask criteria or device ID is included in the AIoT paging message, then the message can be considered as paging all devices that can receive the paging message.

2.2 Contents of AIoT Paging Message/Initial Trigger Message

Regarding the paging cause, the paging cause “Inventory” and “command” can be included. Commands are instructions (e.g., Read, Write, Disable) sent by an AF to an A-IoT device and are protected end-to-end between AF and A-IoT Device. Therefore, the AMF may include one or more command information in the paging message. Then, the Command Information as String type can be also included in the paging message.

According to SA2, one AIoT service may be transferred to multiple AIoT readers with a same higher layer session ID. For example, inventory towards the same bunch of devices may be passed to readers within a local range. From the device side, it is of no use to repeatedly respond to all the paging procedures induced by the same service. Additionally, storing session ID and recording response status on the device helps to avoid duplicated response and reduce power consumption.

Apart from that, since the possibility of the access failure still exists, the reader may trigger multiple rounds of paging corresponding to the same service via air interface. In this sense, the paged devices need to know whether it has successfully been paged and found. That is, if the device receives the paging identified by the same identifier as it responded to before, it will know that the current paging is another round of the one it has responded and accessed; and there is no need to respond again. Thus, the lower layer session ID should be introduced to resolve such power consumption issue from the device point of view. To include a session ID indicating that paging is brought by a specific service, device can respond at will.

In conclusion, the following information can be included in the A-IoT paging message:

- UE paging ID (an ID or a group ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

Proposal 3: the following information can be included in A-IoT paging message:

- UE paging ID (an ID or a group ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

- Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1: The reader could include filter/mask criteria in the AIoT paging message sent to the AIoT device.

Proposal 2: If no filter/mask criteria or device ID is included in the AIoT paging message, then the message can be considered as paging all devices that can receive the paging message.

Proposal 3: the following information can be included in A-IoT paging message:

- UE paging ID (an ID or a group ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

- References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- R2-24079xx, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

- R2-240xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- TR 23.700-13	Study on Architecture support of Ambient power-enabled Internet of Things