3GPP TSG-RAN WG2 Meeting #127bis	R2-2408850

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.2

Source:	China Telecom

Title:	Discussion on Functionalities Required for A-IoT

Document for:	Discussion and Decision

- Introduction

The A-IoT topic was widely discussed in the previous meetings, and the following agreements were made for functionality aspects.

RAN2#126 Agreements

1	Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed)

RAN2#127 Agreements

1	Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size. The LS is approved in R2-2407831.

2	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

3	RAN2 will study the need and use of a simple message “size” reporting to the reader

4	At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

5	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

In this paper, we will discuss some leftover issues regarding functionality aspects and provide some proposals.

2 Discussion

2.1 Energy Status Report

In the last meeting, it has agreed that the device sends the energy status report to the reader when the A-IoT device has no enough energy to process the subsequent communication. Specifically, the A-IoT device can send the energy status report in the following cases.

Case1( No energy after Msg 1):  In this case, the A-IoT device may have very limited energy to only transmit Msg 1 to network in case of the paging message received. To address this issue, the A-IoT device can consider to send one-bit energy status report to indicate no energy status for subsequent Msg 2. Upon this energy status received, the reader could abandon the ongoing access procedure, and not consider the subsequent transmission of the Msg 2. Furthermore, the reader may also transfer this energy status report to the core network to reduce the paging repetitions. Anyway, whether to transfer status report is up to RAN3 decision.

Case2( No energy after Msg 3). In this case, the energy of A-IoT devices can only support to receive/send Msg2 and Msg 3, but has no additional energy to receive any other subsequent messages. Therefore, we understand the energy of A-IoT device is enough for the whole inventory procedure. However, the main issue in this case is that the energy could not support to receive/send subsequent command procedures. In order to provide energy status to network, the MSG3 can include one-bit energy status report. With this energy status report received, the reader may terminate the transmission of  Command message to A-IoT device, and only sends the device ID and energy status report to the core network to assist the core network make further discussion.

From the perspective of the complexity of procedure and impact on design of A-IoT device, we believe that one-bit energy status report is enough for the above cases. With this new energy status indication, the reader can consider to adjust the subsequent schedule policy or provide assistance information the core network.

Proposal 1: One-bit Energy Status Report to the reader is sufficient to address the issue that the A-IoT device has no enough energy to process the subsequent communication.

2.2 Message Size/Status Indication

In SI phase, RAN2 had confirmed that the maximum message size transmitted or received by A-IoT device is approximately 1000bits. In RAN2 #126 meeting, it was agreed that the legacy NR BSR/SR is not needed for A-IoT communication. In the last meeting, some companies proposed that the device needs to send a simple message “size” reporting to the reader. However, the message size reporting needs to introduce additional steps/signalling between reader and A-IoT devices. Furthermore, the indication of message size may introduce more complexity for MAC PDU design and increase the energy consumption of A-IoT device. Therefore, we propose to not introduce a new message size indication for D2R transmission.

Proposal 2: It is proposed to not introduce a new message size indication for D2R transmission.

There exists another UP issue, namely whether to support segmentation at AS layer. Currently this issue is pending on RAN1 and SA2 decision. In our view, the segmentation at application layer may also be an alternative. Based on this, the device could request the reader to transmit the remaining data with more radio resource allocated. To reduce the complexity, one-bit message status indication can be used to indicate whether remaining D2R data exits at device. The message status indication can be included at the end of the previous transmission. With this information, the reader can allocate appropriate radio resource for remaining D2R data.

Proposal 3: One-bit message status indication may be introduced to indicate whether the remaining D2R data exists at the device.

2.3 Visibility of A-IoT Information

It was agreed that the reader can have the knowledge of the service type of A-IoT (e.g. inventory, command) from the CN. In order to perform the subsequent AS procedure, e.g. read/write command procedure, the reader also needs to acquire the device ID and corresponding read/write command from the core network. For instance, during the write procedure, the reader can send the command containing the write indication and the associated data to the target A-IoT device. In addition, if the service type is Command, the reader can allocate appropriate resources for the target A-IoT device. To this end, it is reasonable for reader to get the command type (read, write) from the core network to schedule suitable resources for A-IoT device.

Proposal 4: The reader needs to acquire the device ID and corresponding read/write command from the CN.

In addition, the reader can receive the indication of whether the service is triggered for one or more than one device from the CN. Based on it, the reader can determine whether to trigger contention-free access procedure or contention-based access procedure. For instance, if the service targets on one device, the reader can trigger contention-free access procedure. Otherwise, the reader can trigger contention-based procedure. Therefore, it is necessary for reader to acquire the indication on whether the service is triggered for one or multiple devices from the CN.

Proposal 5: It is necessary for reader to acquire the indication on whether the service is triggered for one or multiple devices from the CN.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: One-bit Energy Status Report to the reader is sufficient to address the issue that the A-IoT device has no enough energy to process the subsequent communication.

Proposal 2: It is proposed to not introduce a new message size indication for D2R transmission.

Proposal 3: One-bit message status indication may be introduced to indicate whether the remaining D2R data exists at the device.

Proposal 4: The reader needs to acquire the device ID and corresponding read/write command from the CN.

Proposal 5: It is necessary for reader to acquire the indication on whether the service is triggered for one or multiple devices from the CN.

- References

- R2-24079xx, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands