3GPP TSG-RAN WG2 #127bis	R2-2408855

Hefei, China, Oct 14th – 18th 2024

Agenda Item:	8.11.2

Source:	Ericsson

Title:	SBFD RA aspects

Document for:	Discussion

# 1	Introduction

In the WID [1], the following objective regarding random access operation has been defined for RAN2

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

We discuss the RAN2 aspects for the above objective in this paper. In the below, we use the term SBFD RA operation to stand for random access operation in SBFD symbols unless otherwise stated. We use the term legacy RA operation to stand for random access operation during non SBFD symbols.

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

# 2	Discussion

## 2.1	General RA configuration

In RAN1 #117 RAN1 agreed to support two options for RACH configuration, a single RACH configuration and an additional RACH configuration. It was also agreed that a UE is not required to support both configurations (cf. yellow highlighting), considering the scenarios for the different configurations are substantially different.

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

The above agreement does not say anything on whether a cell can both SBFD RACH configuration options (i.e., Option 1 with Alt 1-1 and Option 2). In our view, the use case for a cell supporting both SBFD RACH configuration options is non-existent, considering the highly different properties of the two configuration options. Whereas the single RACH configuration i.e., RACH configuration Option 1 with Alt 1-1 may provide larger RACH capacity and lower latency, the second SBFD RACH configuration i.e., RACH configuration Option 2 will provide increased coverage and range (and higher RACH capacity). Higher RACH capacity comes at the expense of increased overhead, in particular for requiring more ROs in already scarce UL resources, and adding even more ROs. In practise, a cell needs to provide both lower latency, and increased coverage and range, which is not reasonable.

In addition to the network aspects of supporting both RACH configuration options within a cell, made agreements and the remaining specification effort would also suffer. RAN1 has not had this use case in mind and has not at all discussed in during previous sessions. That results in that made agreements have not taken it into account. Additionally, agreements may turn out to be ambiguous in the sense that they have not at all considered, e.g., prioritization between SBFD ROs and only prioritization between SBFD ROs and legacy ROs.

Supporting both SBFD RACH configuration options in a cell is not justified since:

  - the use case is non-existing,

  - overhead will substantially increase,

  - it will complicate the remaining specification work, and

  - made agreements risk becoming ambiguous.

Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Regardless of whether RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2 is applied,  it is beneficial to aim for a unified design/framework between RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 to simplify specification complexity. In an example, we can avoid specifying different UE behaviours and procedures for different RACH configuration options.

To aim for a unified RACH design/framework between RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2.

Specifically for RACH configuration Option 2, it is worth to discuss how to implement the additional SBFD RACH configuration in relation to the legacy RACH configuration. In the current MAC spec, RACH configuration is not visible to the MAC layer. In other words, there is no UE procedure to select a specific RACH configuration. It is worth to keep this legacy modelling method when introducing additional RACH configuration for SBFD RA operation.

In the current MAC spec, RACH configuration is not visible to the MAC layer. In other words, there is no UE procedure to select a specific RACH configuration. It is worth to keep this legacy modelling method when introducing additional RACH configuration for SBFD RA operation.

Therefore, we suggest.

As in the legacy, for RACH configuration option 2, the additional SBFD RACH configuration is not visible in the MAC spec. In other words, the introduction of selection of RACH configuration should be avoided in the MAC spec.

## 2.2 SBFD RA Enabling

Even though a network supports SBFD feature, it may not be desirable for it to enable SBFD RA operation. SBFD RA operation requires additional RA resources (e.g., ROs) to be allocated in the cell. This may result in fewer resources left for non RA transmissions (e.g., data and control signalling transmissions) in the cell. On one hand, SBFD RA operation can be beneficial to improve RACH performances for some UEs. On the other hand, some other UEs may suffer from performance degradation due to fewer resources for data and control signalling transmissions. It would be preferred to have a good balance between SBFD RA operation and other non RA transmissions.

We propose to explicitly indicate whether SBFD RA operation is supported for a cell supporting SBFD feature.

Even if the network supports SBFD, the additional overhead associated with the introduction of more ROs should not be mandated.

Network can decide if SBFD RA operation is enabled in a cell supporting SBFD feature.

In addition, if RAN2 agrees to introduce enabling signalling for cell supporting SBFD feature, we think the cell specific signalling option should be sufficient. There is no motivation to introduce UE dedicated signalling option.

Support cell specific signalling for enabling SBFD RA operation.

## 2.3	RO Prioritization and trigger events

RAN1 has made the below agreements regarding RO selection for initial transmission and retransmissions.

Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

In addition, RAN2 had also discuss this issue in RAN2#127. However, no consensus was made. We think the issue would affect both RAN1 and RAN2. Given RAN1 has made further progress on the issue. RAN2 should also continue discussion, instead just waiting for outcome from RAN1 discussions.

Although RAN1 has agreed two RACH configuration options, i.e., Option 1 with Alt 1-1 and Option 2, we believe the issue is valid for both options, especially for RO selection. Given RAN1 has only discussed selection criteria for ROs, RAN2 can follow RAN1 and focus on RO selection.

Same as RAN1, RAN2 focus on discussion of prioritization/selection of ROs.

It would be beneficial from a MAC perspective to have a unified solution including both Option 1 and Option 2 for RO (re)selection.

SBFD RACH configuration can be provided by the gNB via either one single RACH configuration (i.e., the legacy RACH configuration with possible enhancement) or separate RACH configuration. In either of these options, a legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common). A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL by TDD-DL-UL common).

A legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common).

A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL or FL by TDD-DL-UL common).

For initial PRACH transmission attempt, we prefer Option 2 (as described in the above) versus Option 3. The UE can select legacy ROs or SBFD ROs based on certain conditions. One of the possible conditions would be RACH trigger event. SBFD operation is supposed to achieve merits for SBFD aware UE including latency reduction. Certain trigger events including beam failure recovery, handover, or RACH-based LTM cell switch may be sensitive to latency, it may be beneficial to only apply SBFD RACH access for those specific trigger events. In addition, allowing a UE to perform SBFD RA for a RACH trigger event which is delay insensitive, may cause congestion of SBFD RA resources, which would negatively affect RACH performance of other UEs with delay sensitive RACH trigger event. Therefore, we think it is reasonable to limit SBFD resources to delay sensitive RACH trigger events.

Allowing a UE to perform SBFD RA for a RACH trigger event which is delay insensitive, may cause congestion of SBFD RA resources, which would negatively affect RACH performance of other UEs with delay sensitive RACH trigger event.

Further, RAN2 has made the below working assumption regarding RACH trigger events:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Given the above reasons, we think it is reasonable to not confirm the above working assumption.

In contrast to the working assumption agreed in RAN2#127, for delay sensitive RACH trigger events e.g., BFR, handover and RACH-based LTE cell switch, SBFD aware UE selects SBFD RO for initial PRACH transmission attempt.

An SBFD aware UE can use SBFD ROs and/or legacy ROs. It may be necessary to discuss if the UE can mix ROs of different RO types when performing RA operation. Given the UE may apply different power control parameters/settings for RACH operation in SBFD symbols and non SBFD symbols, it may be not preferred for the UE to mix ROs of different RO types when performing RA operation.

It may be also necessary for RAN2 to study whether the SBFD aware UE to use both SBFD ROs and legacy ROs at the same time especially when the UE has detected congestion or RA problem for the selected ROs. In an example, the UE first uses legacy ROs, since the triggered RA procedure is not delay sensitive. However, the UE detects RA failures/problem after a while, then there is risk that the UE cannot fulfil the latency requirement for the RA procedure, it would be helpful if the UE is allowed to fall back to SBFD ROs. It may be also possible that the UE initially selects SBFD ROs, however the UE later experiences RA problem due to interference e.g., CLI introduced by other UEs or gNBs, thus it may be beneficial to allow the UE to fall back to legacy ROs.

SBFD aware UE always selects ROs of the same type in the same RA procedure unless RA problem is being detected.

SBFD aware UE is allowed to fall back to another type of ROs when RA problem for the initially selected RO type is being detected.

## 2.4	SBFD aware early indication

In RAN1, there is on-going discussion on the need of SBFD awareness early indication. It was declared by some companies that It may be beneficial for the gNB to know the UE SBFD-awareness capability in an early stage during a random-access procedure so that the gNB can leverage the SBFD random access operation as early as possible (before the UE can provide its capability report to the gNB).

RAN2 has also agreed to study this issue

FFS whether/how early indication is used during a SBFD RA procedure.

Some options are:

Option 1: PRACH preamble partitioning. In this option, SBFD aware UE and legacy UE would use different PRACH preamble partitions for perform RACH operation. Upon reception of a PRACH preamble from a UE, the gNB can identify whether the UE is SBFD operation aware or not based on whether the detected preamble is split for SBFD operation or non SBFD operation.

Option 2: indication included in a MAC CE. This option may be applicable in case an SBFD aware UE can choose either SBFD ROs or legacy ROs.

Different options may require/give different design complexity to RAN2. In addition, different options may have pros and cons depending on whether it is RACH configuration option 1 (i.e., single RACH configuration) or RACH configuration option 2 (i.e., separately RACH configurations) is chosen by the gNB.

First, we think the issue is only relevant to legacy RACH resources. The issue is actually mainly due to that both legacy UEs and SBFD aware UEs can use legacy RACH resources. That would make it impossible for the gNB to distinguish between legacy UEs and SBFD aware UEs based on the RACH resources used by the UEs.

It is impossible for the gNB to distinguish between legacy UEs and SBFD aware UEs based on the legacy RACH resources used by the UEs.

In addition, Option 1 should be avoided since it will require PRACH preamble partitioning. Given there are already PRACH preamble split for indicating feature or feature combination (e.g., Redcap etc). Further PRACH preamble split may cause too much fragmentation of PRACH preambles, and therefore is not preferred.

Option 1 (i.e., PRACH preamble partitioning) should be avoided since it will require PRACH preamble partitioning. Further PRACH preamble split may cause too much fragmentation of PRACH preambles, and therefore is not preferred.

In addition, Option 2 is not preferred either due to below reasons

- Option 2 requires a new MAC CE to be introduce, which would increase unnecessary design efforts for RAN2

- With early indication via Msg3, the gNB may be only feasible to schedule SBFD resources to subsequent Msg5. The achieved latency reduction would be marginal given that the UE will anyway provide its capability report to the gNB in Msg5.

Option 2 (i.e., indication via a MAC CE) requires a new MAC CE to be introduce, which would increase unnecessary design efforts for RAN2.

Therefore, in our view, it is unnecessary to introduce early indication for SBFD awareness. Anyway, an SBFD aware UE can be identified by the gNB at an early stage when the UE uses SBFD RO, which is sufficient.

In addition to SBFD RO based indication option, additional early indication for SBFD aware UE in RRC_IDLE or RRC_INACTIVE is not supported.

## 2.5	SBFD Indication for CFRA

RAN2 has agreed to strive for a common SBFD CBFA framework independent of RRC state.

RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

In addition, it is also very natural to support contention free RA for SBFD.

To support CFRA SBFD operation.

In the legacy, contention free RA is configured by the gNB via RRC signaling (e.g., RRCReconfigurationwithSync) including a field rach-ConfigDedicated, which comprises dedicated PRACH resources, based on which, the UE can initiate a contention free RA.

As another example, dedicated PRACH resources can be also configured to the UE for beam failure recovery (e.g., candidateBeamRSList in BeamFailureRecoveryConfig indicates set of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameter) so that the UE an apply contention free RA for beam failure recovery.

As yet another example, the gNB can send a PDCCH order to the UE indicating a contention free RA to be triggered. The legacy PDCCH order cannot indicate the triggered RA is for SBFD or non SBFD.

For either of the above cases, a SBFD aware UE is not able to interpret whether the configured dedicated PRACH resources based on the legacy signaling, is for SBFD operation or for non SBFD operation.

Therefore, an indicator can be included in the dedicated RA configuration or a PDCCH order signaled to a UE, indicating the RA resource types for the contention free RA initiated by the UE according to the dedicated RA configuration or the PDCCH order.

An indicator is included in the dedicated RA configuration or a PDCCH order indicating the RA resource types, i.e., legacy RA resource, or SBFD RA resource.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Supporting both SBFD RACH configuration options in a cell is not justified since:

a.	the use case is non-existing,

b.	overhead will substantially increase,

c.	it will complicate the remaining specification work, and

d.	made agreements risk becoming ambiguous.

Observation 2	In the current MAC spec, RACH configuration is not visible to the MAC layer. In other words, there is no UE procedure to select a specific RACH configuration. It is worth to keep this legacy modelling method when introducing additional RACH configuration for SBFD RA operation.

Observation 3	Even if the network supports SBFD, the additional overhead associated with the introduction of more ROs should not be mandated.

Observation 4	Same as RAN1, RAN2 focus on discussion of prioritization/selection of ROs.

Observation 5	It would be beneficial from a MAC perspective to have a unified solution including both Option 1 and Option 2 for RO (re)selection.

Observation 6	A legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common).

Observation 7	A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL or FL by TDD-DL-UL common).

Observation 8	Allowing a UE to perform SBFD RA for a RACH trigger event which is delay insensitive, may cause congestion of SBFD RA resources, which would negatively affect RACH performance of other UEs with delay sensitive RACH trigger event.

Observation 9	It is impossible for the gNB to distinguish between legacy UEs and SBFD aware UEs based on the legacy RACH resources used by the UEs.

Observation 10	Option 1 (i.e., PRACH preamble partitioning) should be avoided since it will require PRACH preamble partitioning. Further PRACH preamble split may cause too much fragmentation of PRACH preambles, and therefore is not preferred.

Observation 11	Option 2 (i.e., indication via a MAC CE) requires a new MAC CE to be introduce, which would increase unnecessary design efforts for RAN2.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Proposal 2	To aim for a unified RACH design/framework between RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2.

Proposal 3	As in the legacy, for RACH configuration option 2, the additional SBFD RACH configuration is not visible in the MAC spec. In other words, the introduction of selection of RACH configuration should be avoided in the MAC spec.

Proposal 4	Network can decide if SBFD RA operation is enabled in a cell supporting SBFD feature.

Proposal 5	Support cell specific signalling for enabling SBFD RA operation.

Proposal 6	In contrast to the working assumption agreed in RAN2#127, for delay sensitive RACH trigger events e.g., BFR, handover and RACH-based LTE cell switch, SBFD aware UE selects SBFD RO for initial PRACH transmission attempt.

Proposal 7	SBFD aware UE always selects ROs of the same type in the same RA procedure unless RA problem is being detected.

Proposal 8	SBFD aware UE is allowed to fall back to another type of ROs when RA problem for the initially selected RO type is being detected.

Proposal 9	In addition to SBFD RO based indication option, additional early indication for SBFD aware UE in RRC_IDLE or RRC_INACTIVE is not supported.

Proposal 10	To support CFRA SBFD operation.

Proposal 11	An indicator is included in the dedicated RA configuration or a PDCCH order indicating the RA resource types, i.e., legacy RA resource, or SBFD RA resource.

# 4	References

- [1] 	RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei,

- Shanghai, China, June 17-20, 2024, RAN#104.