3GPP TSG-RAN WG2 Meeting #127bis	R2-2408858

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.7.4.2

Source:	China Telecom

Title:	Discussion on Remaining Issues for DSR Enhancement

Document for:	Discussion and Decision

- Introduction

The DSR enhancement had been widely discussed in the RAN2#127 meeting, and the corresponding agreements are as follows:

- RAN2#127 agreements For enhanced DSR: - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

This paper aims to discuss the remaining issues with respect to DSR enhancement for XR.

2 Discussion

In Rel-18, there is only one remaining time threshold. Once the remaining time falls below the threshold, the data becomes delay critical and a DSR is triggered to report the delay critical data. To increase the granularity of delay information, RAN2 had agreed that multiple remaining time thresholds could be configured for each LCG to report multiple pairs of remaining time and buffer size. However, it is undetermined when to trigger DSR in case multiple remaining time thresholds exist. One simple way is to configure multiple DSR triggering thresholds corresponding to multiple remaining time thresholds. In this way, there will be multiple DSR triggered, which may increase the signalling overhead and cause traffic congestion. Consequently, it is not preferable to configure multiple DSR triggering thresholds. In our understanding, to reduce the complexity, one efficient way is to select one of the remaining time as the triggering threshold. Specifically, the network configures which remaining time threshold is used as the DSR triggering threshold. Once a DSR is triggered, multiple pairs of remaining time that smaller than the threshold and its related buffer size will be reported.

Proposal 1: One of the remaining time threshold can be configured as the DSR triggering threshold.

In Rel-18, in case the remaining time of any data falls below the threshold, it is reported as delay critical data. With multiple reporting thresholds, multiple pairs of data will become delay critical data. For instance, it is assumed that the remaining time threshold of PDU set1 and PDU set2 are 10ms and 20ms respectively. For PDU set1, once the remaining time falls below 10ms, it is considered as delay critical data. As for PDU set2, once the remaining time falls below 20ms, it is considered as delay critical data. The UE can report all delay critical data to the network with different pairs of remaining time and buffer size. Upon reception of DSR, the network can schedule resources based on the each pair of remaining time and buffer size of data. From our understanding, the introduction of multiple reporting thresholds has limited impact on UE and network behaviour. To reduce specification impact, the definition of delay critical data may not be affected due to multiple reporting thresholds in DSR.

Proposal 2: The definition of delay critical data may not be affected due to multiple reporting thresholds in DSR.

It is undetermined that whether non-delay critical data ahead of delay critical data will be included during the buffer size calculation for DSR. Based on the current spec, the data within an LCH is assembled in a first-in and first-out manner. It means that the non-delay critical data ahead of delay critical will be assembled first and consume some UL grant. If the gNB is unaware of non-delay critical data, after non-delay critical data is assembled, the remaining resources may be not enough to delay critical data. As a result, the delay critical data transmission would be impacted.

Furthermore, if the non-delay critical data has higher priority than the delay-critical data, upon reception of the UL grant, the non-delay critical data will be assembled first. In this case, the non-delay critical data information also needs to be reported to the gNB. Based on it, the gNB will allocate enough UL grants to ensure the delay-critical data transmission.

From our point of view, reporting both non-delay critical data and delay critical data can help gNB have a better understanding of the whole buffer status of the UE. If the non-delay critical data is not included in the buffer size calculation, the non-delay critical data may consume the resource allocated for the delay critical data. As a result, the delay critical data would be impacted or even discarded.

Proposal 3: Non-delay critical data ahead of delay critical data or with higher priority also need to be included in the buffer size calculation for DSR.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: One of the remaining time threshold can be configured as the DSR triggering threshold.

Proposal 2: The definition of delay critical data may not be affected due to multiple reporting thresholds in DSR.

Proposal 3: Non-delay critical data ahead of delay critical data or with higher priority also need to be included in the buffer size calculation for DSR.

- References

- R2-240xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- R2-24079xx, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands