3GPP TSG-RAN WG2 Meeting #127bis	R2-2408859

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.7.5

Source:	China Telecom

Title:	Discussion on RLC Enhancements

Document for:	Discussion and Decision

- Introduction

One of objectives in Rel-19 XR WID is to specify the RLC re-transmission enhancements solution for RLC AM transmission with small packet delay budget. Based on the progress of previous RAN2 meetings, this paper continue to discuss study RLC enhancements regarding to unnecessary retransmission avoidance and timely RLC retransmission.

- Discussion

2.1 Timely RLC retransmissions

In RAN2 #126 meeting, the agreements on timely retransmission on RLC layer were as follows.

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

For UL traffic, when the transmitting entity receives the status reporting including NACK, it will trigger a RLC PDU retransmission procedure. However, the legacy status reporting procedure can obtain the data transmission successful but it also introduce extra latency with RLC PDU retransmission, which is not suitable for RLC PDUs with low packet delay budget (PDB). One potential solution to reduce latency efficiently is to allow RLC PDU retransmission without waiting status report.

With regard to the trigger condition of the blind retransmission, one efficient way is to specify the condition as the remaining time of the PDU is less than the configured threshold. Based on this, the UE can retransmit the RLC PDU(s) quickly within small delay budget. However, this solution may also waste radio resource and has negative impact on system capacity, especially in a congestion scenario.

Observation 1: Autonomous retransmission may result in resource waste and system capacity reduction, especially for a congestion system.

Proposal 1: RLC autonomous retransmission is not preferred.

- Enhanced Status Report based Retransmission

Currently the Rel-19 RLC enhancement for XR only considers UL direction, another alternative is to introduce an enhanced status report mechanism for retransmission. The network could determine whether to trigger the status report based on configured parameters or policy, e.g. the received time of the PDU, the congestion status, etc to reduce the latency of RLC retransmission. Anyhow, the enhanced status report has no impact on UE side and is left to the network implementation.

Proposal 2: The enhanced status report based retransmission has no impact on UE side and is left to the network implementation.

- Retransmission based on enhanced polling

Per TS 38.322, the UE can poll the network to trigger status reporting in case of the following situations are satisfied:

    - The amount of AMD PDUs/data bytes sent since the most recent poll bit was transmitted is larger than a threshold (i.e., PDU_WITHOUT_POLL >= pollPDU; or BYTE_WITHOUT_POLL >= pollByte), where the “pollPDU” indicates that the transmitter trigger a poll for every pollPDU PDUs, and “pollByte” indicates that the transmitter trigger a poll for every pollByte bytes;

    - Both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU;

    - No new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

    - Expiry of t-PollRetransmit.

It can be observed that the above configured variables, namely  PDU_WITHOUT_POLL, BYTE_WITHOUT_POLL or the expiry of t-PollRetransmit can be used to trigger a poll of AMD PDU. One efficient way to reduce the latency is to configure the parameters of PDU_WITHOUT_POLL/BYTE_WITHOUT_POLL as small values or  reduce the length of t-PollRetransmit. For instance, to enable timely RLC retransmission for time-sensitive UL XR traffic, the network can configure shorter pollPDU, pollByte or t-PollRetransmit values for UE by RRC message RLC-Config. Compared to an extra timer to trigger polling, to specify new values in polling has limited impact on capacity and standard.

In addition, the status report will only be triggered upon the expiration of t-StatusProhibit timer. Therefore, if shorter pollPDU, pollByte or t-PollRetransmit values are configured, the network could also configure shorter t-StatusProhibit timer value.

Proposal 3: For UL traffic, the network can configure short pollPDU, pollByte or t-PollRetransmit values to trigger the polling for timely RLC retransmission.

In XR, PDU set containing one or more PDUs are introduced to carry the payload of one unit of information generated at the application level. Per TS38.322, the UE polls the gNB after a configured number of PDUs or bytes. It is most likely that the UE may trigger a poll in the middle of a PDU set. As a result, the status report may only contain the reception status of a part of PDUs in a PDU set. If the other part of PDUs have not been retransmitted promptly, the PDU set containing the previous successfully transmitted PDUs will also be dropped, resulting in resource waste.

Therefore, for XR traffic, the polling mechanism may need to be enhanced to enable the UE trigger the poll based on the PDU set. For instance, the poll bit can be set based on PDU set traffic pattern. In this way, the UE can receive the status report of one or more PDU sets.

Proposal 4: Study polling enhancement to enable the UE poll the RLC status based on the PDU set.

2.2 Avoid unnecessary retransmission

Based on the previous meeting agreements, there are three options to avoid unnecessary retransmission.

For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. For combined Rx and Tx approach - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first.

In the Rx initiated approach and combined approach, a new timer will be introduced to perform the discard decision. However, the PSDB and buffered time are maintained at Tx side. The receiver cannot exactly determine whether the RLC exceeds the packet delay budget. Therefore, it is difficult to determine the start point and length of the timer. On the other hand, the Rx also needs to indicate the discarded PDU or segment to Tx side to avoid unnecessary retransmission. The status report needs to be enhanced to indicate the discarded RLC PDU or segmentation.

Observation 2: For Rx initiated and combined approach, it is difficult to determine the start point and length of the new timer, and the status report needs to be enhanced to indicate the discarded RLC PDU or segmentation.

Per current specification, the transmitter has the knowledge of the remaining time of the PDUs. Based on the PSDB and buffered time, the transmitter can assess the remaining time for PDU retransmission. Once the remaining time runs out, the transmitter can discard PDUs from PDU retransmission buffer promptly. In addition, if the PDCP layer indicates RLC layer to discard the PDCP PDU, but the RLC SDU or part of the RLC SDU have already been delivered to the lower layer, the transmitter can avoid retransmitting these RLC SDUs based on the PDCP layer discard indication.

Observation 3: Based on the remaining time of RLC PDU or PDCP layer discard indication, Tx initiated approach can avoid unnecessary RLC PDU retransmission promptly.

For Tx initiated approach, after the transmitter discards the RLC PDU, it updates the state variables and moves the Tx window. After that, the transmitter sends the SN gap indication to the receiver, and the receiver updates the variables accordingly. In this way, even if the SN gap report and the status report from the receiver are lost, the transmitter can resolve the Tx window stalling problem.

Observation 4: To resolve the Tx window stalling problem, the transmitter can move the Tx window after discarding the RLC PDU.

Proposal 5: Tx initiate approach is preferred for avoiding unnecessary RLC retransmission.

- Conclusions

Based on the above analysis, we have the following observations and proposals:

Observation 1: Autonomous retransmission may result in resource waste and system capacity reduction, especially for a congestion system.

Observation 2: For Rx initiated and combined approach, it is difficult to determine the start point and length of the new timer, and the status report needs to be enhanced to indicate the discarded RLC PDU or segmentation.

Observation 3: Based on the remaining time of RLC PDU or PDCP layer discard indication, Tx initiated approach can avoid unnecessary RLC PDU retransmission promptly.

Observation 4: To resolve the Tx window stalling problem, the transmitter can move the Tx window after discarding the RLC PDU.

Proposal 1: RLC autonomous retransmission is not preferred.

Proposal 2: The enhanced status report based retransmission has no impact on UE side and is left to the network implementation.

Proposal 3: For UL traffic, the network can configure short pollPDU, pollByte or t-PollRetransmit values to trigger the polling for timely RLC retransmission.

Proposal 4: Study polling enhancement to enable the UE poll the RLC status based on the PDU set.

Proposal 5: Tx initiate approach is preferred for avoiding unnecessary RLC retransmission.

- References

- R2-24079xx, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands