3GPP TSG-RAN WG2 Meeting #127bis                                    	R2-2408863

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.9.3

Source:	Deutsches Zentrum für Luft- und Raumfahrt (DLR) e.V., ESA

Title:	Discussion on throughput and delay performance of slotted ALOHA and diversity slotted ALOHA

Document for:	Discussion and Decision

- Introduction

RAN WID [1] has following objective for the EDT enhancements.

Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signalling to complete an Early Data Transmission (EDT) transaction [RAN2]:

- Msg3 transmission without msg1/ Random Access Response (RAR)

In [2] it was proposed to study EDT enhancements based on Diversity Slotted Aloha and Contention Resolution Diversity Slotted Aloha [4,5]. In this document, we provide some additional results on the performance gains that the two methods can provide, which need to be discussed.

- Performance of SA and DSA

EDT Enhancements using Diversity Slotted ALOHA

As noted in recent contributions, e.g., [3-6], the performance of Msg3-EDT transmission without Msg1 in NB-IoT can benefit significantly when considering the use of packet replications. In particular, the use of solutions inspired by diversity slotted ALOHA (DSA) [7] or its improvements, such as contention resolution diversity slotted ALOHA (CRDSA) [8,9] has been shown in [4,6] to enable substantial throughput gains with minor modifications required to the specifications. More advanced techniques to exploit the energy used to transmit packet copies for improving the energy efficiency of CRDSA-like protocols are also discussed in [10].

The idea behind these schemes is to allow a terminal transmit two or more replicas of a packet in randomly selected resources within a set of slots (e.g., within a set of EDT occasions). This proactive behavior triggers collision diversity, increasing for low to moderate channel loads the probability of successfully delivering the message.

Contributions [4,6] focuses on a single transmission attempt (possibly involving sending multiple replicas when using DSA or CRDSA). However, due to the very nature of a random access channel, when a certain level of reliability has to be granted, retransmission have to be considered in case of failure. Based on this remark, we provide in this document some considerations regarding the performance of DSA with respect to the existing SA solution when retransmissions are in place. Implications in terms of throughput and delay are discussed, having in mind a satellite setting (GEO).

Considered setup

In the remainder, we assume that a terminal can attempt transmission of a packet by placing it within a set of N consecutive EDT occasions, as illustrated in [4]. Each EDT occasion has duration Tedt, and consists of Nsc subcarriers.

When a terminal attempts transmission, it can follow two different approaches:

- Slotted ALOHA (SA): the terminal sends a single copy of its packet (k=1), placing it in one of the Nsc subcarriers

- Diversity slotted ALOHA (DSA): the terminal sends k>1 copies of its packet in k different subcarriers.

If at least one of the copies (or the only one, for SA) is decoded, the message is successfully delivered. Otherwise, the node will attempt another transmission as described above, after a certain amount of time.

The time between successive retransmissions is assumed to be at least equal to a round trip time (RTT), considering that the terminal has to wait for a potential acknowledgement. It is noted that additional latency components may be present (e.g., random backoff or further processing times). A maximum number of n transmission attempts is considered, i.e., the first attempt followed by at most n-1 retransmission attempts.

We assume that a target packet loss rate (PLR) is targeted. This is the probability that a message is still not decoded after all of the possible n transmission attempts, i.e., all of the n attempts fail. We denote the target PLR as and observer that for practical applications values of  << 1, e.g.,  = 10-2 are reasonable, when retransmissions are considered [11]. We remark that is the PLR at service level, and not the reliability with a single transmission attempt. The parameters considered in the study are reported in Table 1.

Table 1 Configuration parameters

| Parameter | Value |
| --- | --- |
| Number of EDT resources (subcarriers) per occasion, Nsc | 48 |
| Number of EDT occasions, N | 4 |
| Duration of an EDT occation, Tedt | 96 ms |
| Replicas for DSA | 2 and 3 |
| Round trip time, RTT | 540 ms |

- Results and discussion

For results and discussion, we assume that a large population of terminals can contend for the resources. As a result, the overall traffic injected in the channel, including new packets and retransmissions can be modeled as a Poisson process.

We abstract the physical layer by assuming that collision results in the impossibility of correct decoding and that transmission w/o collision are successfully decoded. It is to be noted that this represents a pessimistic assumption, as in reality packets may be retrieved even in the presence of collisions due to power unbalance and FEC. The outcomes of successive retransmission by a terminal are considered to be statistically independent, as is the case when a sufficiently large time elapses between successive attempts.

Under these conditions, if a maximum of n retransmissions is allowed, a target packet loss rate  is achieved only if each transmission attempt is successful with probability

ps(n) ≥1-ε1/n

The maximum aggregate throughput, measured in(n) packet per slot, i.e., average number of users decoded over a set of N consecutive EDT occasions, is [7]

S(n,k)= psk ln1-1-ps1/k

where k=1 corresponds to SA, whereas k>1 denotes DSA with k replicas.

In turn, the average delay experienced by a successfully delivered message can be approximated as

D(n)=RTT+NoppTedtps(n)

where the numerator expresses the time needed for the transmission of a message (NoppTedt) and reception of an acknowledgement (RTT), i.e., a lower bound to the time between successive retransmissions, whereas the term 1/ps denotes the average number of transmission attempts needed to successfully deliver a message. Note that, for a target ε, D(n) does not depend on k, as it is inversely proportional to ps. The impact of using more replicas emerges in the throughput performance, as will be discussed later.

Numerical results

Figure 1 reports the maximum achievable throughput S for a target service packet loss rate. Values of  between 10-2 and 10-1 (i.e., reliability of 0.9 to 0.99) were considered for practical relevance, where 10-2 is a more realistic value for ensuring a good quality of service. In the plot, solid lines report the behavior of the legacy SA, whereas dashed and dotted lines refer to DSA with k = 2 and k = 3 replicas, respectively.

Black lines (circle markers) refer to the case n = 1, i.e., to a single transmission attempt (no retransmissions in case of failure). In turn, blue lines (square markers) report the case of one retransmission attempt (n = 2), and the red lines (cross markers) of two retransmission attempts (n = 3).

Figure 1: Maximum achievable throughput vs target packet loss rate. Results shown for SA and DSA, considering the case of no retransmissions (n=1) as well as the case with 1 or 2 retransmissions in case of failures (n=2, n=3).

The plot shows that, without retransmissions, the targeted level of reliability can only be attained over a random access channel by operating at low loads, and thus obtaining a low throughput. In this case, the benefits of relying on DSA are more than evident. For instance, DSA with three replicas, offers a 3-fold improvement of throughput when targeting  = 0.05. The results are in line with the discussions in [3-6].

Observation 1: When retransmissions are not considered, DSA can improve throughput substantially with respect to SA for a broad range of packet loss rates of practical relevance. For instance, for a target service PLR of 0.05 (reliability 0.95), a 3-fold throughput increase is achieved with DSA-3 over SA.

When retransmissions are considered (n > 1), the throughput improves in all cases. This is expected, as a higher collision probability on a single attempt can be tolerated, aiming to recover with successive transmissions. As a consequence, the channel can be operated at higher load, leading to higher throughput (we note that we are still operating below the critical load after which throughput starts plummeting due to collisions [4,6]). More importantly, also under these conditions DSA improves over SA significantly for almost all PLR values of interest. For instance, with n = 2, throughput increases by 127% when targeting = 0.05. Similarly, for n = 3 DSA with 2 replicas outperforms the legacy SA up to a target service packet loss rate of approximately 0.06.

It shall be noted however that, while increasing n can improve throughput, it also has a cost in terms of delay, especially when the round-trip time is relevant, as in the case of GEO satellite links. This can be especially critical when considering the overall idea of EDT, meant to favor quick transmissions.

To better evaluate this aspect, we report in Figure 2 the average delay against the target packet loss rate . In the plot, the black line (circle markers) refers to the case of no retransmissions (n = 1), whereas the blue (squared markers) and red (cross markers) consider one and two retransmission attempts (i.e., n = 2 and n = 3, respectively. Incidentally, we remark that a single curve is present for each case, as the performance is the same for SA and DSA. Indeed, an attempt is performed within NoppTedt seconds in both cases, the only difference being in the number of replicas that are sent within this time.

The plot clearly highlights how, if we consider the region where SA was outperforming DSA in Figure 1, ie., n = 3, and  > 0.5, a delay which exceeding 1.5 seconds is undergone. This result buttresses the conclusion that DSA can trigger significant throughput improvements even when retransmissions are considered, for practical values of PLR, and when it is desirable to have a lower delay.

Observation 2: DSA leads to significant throughput improvements over SA even when retransmissions are considered, for practical values of service level PLR, and when it is desirable to have reasonably low delay.

Figure 2: Average delay vs packet loss rate

To conclude our discussion, we show a convenient and concise combination of the results reported so far in Figure 3. The graph reports the average delay attained for a maximum target throughput. Specifically, each point in the curves is obtained by targeting a different target service packet loss rate , in the range always in the range 10-1 to 10-2, and plotting the corresponding values of S and D.

We only report for the sake of clarity the case n = 1 (no retransmission) – denoted by black lines -, and the case n = 3 (two retransmissions) – denoted by red lines. Solid lines show the behavior of SA, whereas dashed lines of DSA with two replicas (k = 2).

Once again, we see that, without retransmissions, DSA significantly increases throughput for a target delay. Moreover, when retransmissions are allowed, DSA improves throughput when lower delays are targeted. SA can provide better throughput only when large delays are tolerated and rather high service level PLR have to be undergone.

Figure 3 Packet Error Rate vs. Channel Load, Configuration A. Comparison between 2 and 3 replicas.

Proposal 1: RAN2 to study improved random access protocols for Msg3-EDT transmissions without msg1/ Random Access Response, with focus on low-complexity techniques as DSA and CRDSA.

- Conclusion

Following observations and proposals are made:

Observation 1: When retransmissions are not considered, DSA can improve throughput substantially with respect to SA for a broad range of packet loss rates of practical relevance. For instance, for a target service PLR of 0.05 (reliability 0.95), a 3-fold throughput increase is achieved with DSA-3 over SA.

Observation 2: DSA leads to significant throughput improvements over SA even when retransmissions are considered, for practical values of service level PLR, and when it is desirable to have a reasonable delay.

Proposal 1: RAN2 to study improved random access protocols for Msg3-EDT transmissions without msg1/ Random Access Response, with focus on low-complexity techniques as DSA and CRDSA.

- References

[1] RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

[2] R2-2402811, “Discussion on EDT enhancements”, Qualcomm INC., 3GPP TSG-RAN WG2 Meeting #125bis, Changsha, China, April 2024.

[3] R2-2405202, “Discussion on EDT enhancements”, ESA, 3GPP TSG-RAN WG2 Meeting #126, Fukuoka, Japan, May 2024.

[4] R2-2406593, “Discussion on DSA and CRDSA Performance,” ESA, 3GPP TSG-RAN WG2 Meeting #127, Maastricht, the Netherlands, Aug. 2024.

[5] R2-2406640, “Discussion on EDT enhancements,” Qualcomm INC., 3GPP TSG-RAN WG2 Meeting #127, Maastricht, the Netherlands, Aug. 2024.

[6] R2-2406593, “Performance of Advanced Random Access Protocols,” DLR, 3GPP TSG-RAN WG2 Meeting #127, Maastricht, the Netherlands, Aug. 2024.

[7] G. Choudhury abd S. Rappaport, “Diversity ALOHA—A Random Access Scheme for Satellite Communications,” in IEEE Transactions on Communications, Vol. COM-31, No. 3, pp. 450-457, Mar. 1983.

[8] E. Casini, R. De Gaudenzi and O. Del Rio Herrero, "Contention Resolution Diversity Slotted ALOHA (CRDSA): An Enhanced Random Access Schemefor Satellite Access Packet Networks," in IEEE Transactions on Wireless Communications, vol. 6, no. 4, pp. 1408-1419, April 2007.

[9] G. Liva, “Graph-Based Analysis and Optimization of Contention Resolution Diversity Slotted ALOHA,” in IEEE Transactions on Communications, vol. 59, no. 2, pp. 477–487, February 2011.

[10] P. Agostini et al., “Evolution of the 5G New Radio Two-Step Random Access towards 6G Unsourced MAC,” May 06, 2024, arXiv: arXiv:2405.03348. Also available at http://www.smalldatanet.com.

[11] TR-45.820, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, 3GPP, Aug. 2015.