3GPP TSG RAN WG2 #127bis	R2-2408869

Hefei, China, October 14th - October 18th, 2024

Agenda Item:	8.2.3

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Discussion on A-IoT paging

Document for:	Discussion & decision

# Introduction

In the RAN#102 meeting, a new SID on solutions for Ambient IoT (Internet of Things) in NR was approved [1]. Ambient IoT relies on ultra-low complexity devices with ultra-low power consumption for very-low-end IoT applications. The RAN2-led objectives are listed as follows:

Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In previous RAN2 meetings [2][3], the following agreements related to paging/initial trigger message were made:

RAN2#126 Meeting Agreements: - RAN2 will study the following cases for A-IoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the A-IoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. - A-IoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. - From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details. RAN2#127 Meeting Agreements: Support mechanism for reader to be able to trigger multiple/subsequent A-IoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In this contribution, we further discuss the considerations on the A-IoT paging procedure.

# Discussion

## Multiple A-IoT paging messages

Based on the agreements from RAN2#127, multiple A-IoT paging messages associated with the same CN request can be transmitted sequentially. However, solutions to avoid duplicated responses need to be studied. During the meeting, the discussion addressed the approach where comparing paging frames could be sufficient to let A-IoT devices know that multiple paging messages belong to the same request. However this approach may be insufficient.

As per the SID, neither ARQ nor HARQ are supported in A-IoT, and therefore, there is no inherent feedback or acknowledgment to indicate successful reception of the device’s response. As a result, without a way to filter or differentiate the paging messages, A-IoT devices that successfully transmitted their data may intend to transmit again. Such effect may be even exacerbated if subsequent paging frames are highly separated on time, as A-IoT devices may consider them as new requests. This behavior is particularly problematic as repeated transmissions not only add to network congestion, but also lead to unnecessary energy consumption.

Observation 1: Two different A-IoT paging messages can have the same parameters but may not be associated to the same request.

Figure 1. Time difference between multiple A-IoT paging frames

A straightforward way to tackle this issue would be add a session or request identifier in each paging message that indicates weather it is a repeat of a previous request, letting the A-IoT devices to remember their response status (i.e., successfully inventoried or not). Hence, mimicking the behavior of a session identifier in RFID. However, it would be at the expenses of increasing the overhead. A more efficient way to achieve the same can be to use the time difference between paging frames, as depicted in Figure 1

From RAN1#116 [4], the following was agreed in regards of timing aspects,

Agreement - For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device.

Taking into account that RAN1 agreed to have a minimum time TR2D_R2D between two different transmissions to the same device, a preconfigured time threshold interval, i.e., TR2D_R2D< T ≤ TMAX, could be used by A-IoT devices to know that the subsequent paging messages (received within a defined time window) are associated with the same initial request, rather than being interpreted as new ones. Given that the slot duration and the maximum number of slots in the paging cycle are predetermined, A-IoT devices can calculate the time difference between the last slot used from a previous round and the reception of a new paging message. If the difference falls within the preconfigured threshold, the device can infer that the new paging message is a continuation of the previous communication. This approach can ensure that devices which have already responded to a request do not transmit again, thus minimizing redundant transmissions.

Proposal 1: Multiple paging frames need to be performed within a maximum time interval to be associated with the same CN request.

Another aspect could be that the same request is used for different procedures, i.e., inventory + command case, which are addressed to different sets of devices. In [5], the following figure is added regarding the overall AS procedure for A-IoT paging, representing the different steps involving different procedures.

Figure 2. Overall AS procedure for AIoT paging

Additionally, under the agenda item 8.2.2, in RAN#126, the following was agreed

Clarify in TR that inventory and command doesn’t mean that A-IoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer.

As shown, in the case of inventory+command, all three steps, A, B and C are involved; from the highlighted part, it is understood that different procedures, from the same request, can be transmitted over different paging frames, with the purpose of triggering different sets of devices. For example, an initial paging with the inventory procedure could be directed towards a broad set of devices, enabling the network to gather information from a larger pool of devices. Once the inventory is completed, a subsequent paging with the command procedure can then be tailored to a smaller and more specific subset of devices. Additionally, in certain cases, the command procedure might even be targeted at a single device, ensuring precise control over specific devices within the larger group. This approach can be useful to address a CFRA case for multiple devices.

Proposal 2: Paging frames belonging to the same service request containing different procedures may be addressed to different set of devices.

## A-IoT paging message content

In RAN2#126, it was agreed that time and frequency resources should be announced through the paging frame. However, paging frames should also include L2 parameters. Specifically, the Q parameter, which is used to adjust the slot window, should be included in the paging frame as its value can be tailored and optimized for each specific procedure.

As stated in the previous section, multiple procedures can be indicated in different frames. Therefore, we suggest that the Q values to be dynamically adjusted. Specifically, under the inventory+command case, we suggest the Q values to be adjusted in regards of the different procedures, as they can target different groups of devices. This will allow the network to assign the most suitable contention window based on the operational needs within each group and procedure. For example, a command operation targeting a specific subset of devices may require a different Q value compared to an initial inventory request sent to a larger group of devices.

Proposal 3: Paging frames should include channel access-related information, such as the Q parameter, allowing for adjustments in each independent procedure.

Additionally, multiple frames associated with the same request (i.e., inventory first, command later) can reuse the same L1 information, given that mobility is not within the scope of the current study, and rapid environmental changes are unlikely. In this scenario, the headers of subsequent control frames can be simplified by keeping L1 parameters, such as time-frequency resources and/or MCS-like information, constant across all paging frames related to the same request. Once these parameters are established in the initial paging frame, subsequent frames only need to update channel access parameters, such as Q values or timing adjustments.

Proposal 4: For multiple frames associated with the same request, parameters announced in the initial paging frame can be used in subsequent frames to reduce redundancy.

# Conclusion

In this contribution we discussed the A-IoT paging procedure for A-IoT devices. Based on the discussion in the previous sections we made the following observations:

Observation 1: Two different A-IoT paging messages can have the same parameters but may not be associated to the same request.

Based on the discussion in the previous sections, we propose the following:

Proposal 1: Multiple paging frames need to be performed within a maximum time interval to be associated with the same CN request.

Proposal 2: Paging frames belonging to the same service request containing different procedures may be addressed to different set of devices.

Proposal 3: Paging frames should include channel access-related information, such as the Q parameter, allowing for adjustments in each independent procedure.

Proposal 4: For multiple frames associated with the same request, parameters announced in the initial paging frame can be used in subsequent frames to reduce redundancy.

# References

- 3GPP RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG-RAN #102, Edinburg, UK, Dec 11th-15th, 2023.

- 3GPP “Draft Report of 3GPP TSG RAN WG2 meeting #126”, 3GPP TSG RAN WG2 #126, Fukuoka, Japan, May 20th-24th, 2024.

- 3GPP “Draft Report of 3GPP TSG RAN WG2 meeting #127”, 3GPP TSG RAN WG2 #127, Maastricht, Netherlands, Aug. 19th-23rd, 2024.

- 3GPP R1-2401789 “FL summary #2 on frame structure and timing aspects for Rel-19 Ambient IoT”, 3GPP TSG-RAN WG1 Meeting #116, Athens, Greece, February 26th – March 1st, 2024.

- 3GPP TR 38.769 V1.0.0 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on solutions for ambient IoT (Internet of Things (Release 19)” (2024-09).