3GPP TSG-RAN WG2 Meeting #R2-127bis		R2-2408871 Hefei, China, Oct 14-18, 2024

Agenda Item:	8.6.3

Source: 	Rakuten

Title:  	Event based L1 measurements triggered LTM candidate cell addition/release

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

The WID objective includes Event based L1 measurements for LTM.

In this contribution, we discuss the feasibility of configuring Event based L1 measurements and using them for LTM candidate cell addition/release. Further, we discuss redundancy of L3 and L1 measurements for LTM.

# Discussion

## gNB-DU initiated LTM candidate cell addition/release based on L1 measurement event

Following agreements were made in R2#126 regarding Measurement enhancements for LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In Release 18 LTM, RAN2 has agreed that the candidate cell preparation is performed by the gNB-CU based on the L3 measurements, while the LTM cell switch execution is performed by the gNB-DU based on the periodic L1 measurements.

In Release 18 LTM, the L1 measurements was possible to be configured only for LTM candidate cells and was only periodic in nature. With the introduction of event triggered L1 measurements, discovered and unconfigured cells are also feasible to be reported. For example, if a newly discovered cell meets the configured event criteria, it could be reported using L1 measurements. Hence, a new LTM candidate cell addition could be triggered based on these L1 measurements.

A maximum of 8 LTM candidate/target cells are feasible to be configured per UE and it is important that the candidate configuration slot isn’t wasted due to obvious reasons. It creates resource crunch at the network side and holding on to an unwanted candidate cell may deprive the UE of a valuable next candidate cell. Therefore, it is important to release unwanted and irrelevant LTM candidate cells.

In summary, currently since the LTM candidate cell addition and release is based only on L3 measurements, it implies that an LTM UE has to be configured for both L3 and L1 measurements, for the purpose of preparation and execution of LTM. But this could be triggered using event based L1 measurements also, which enhances benefits in terms of both signaling and latency.

With the introduction of event based L1 measurements, newly discovered cells can be reported to the gNB using an L1 measurement report.

LTM candidate cell addition and release procedure could be triggered based on event based L1 measurement reports, therefore enhancing both signaling and latency.

Configuring both L3 and L1 measurements is redundant in our view and causes unnecessary overhead. On the contrary, there are other RAN2 related study items in R19 where the objective of the study is to reduce measurements by using AI-ML methods. Hence, we think that RAN2 should explore the possibility of configuring UEs with only one kind of measurements for LTM, for example, using L1 event based measurements to configure addition and release of LTM candidate cells. This does not preclude the UE from being configured with L3 measurements for other purposes.

RAN2 explores the possibility of using event based L1 measurements for the purpose of triggering LTM candidate cell addition and release.

RAN2 discusses the possibility of avoiding configuring redundant measurements for the same purpose i.e LTM candidate cell addition and release.

# Conclusion

We have the following observations:

Observation 1	With the introduction of event based L1 measurements, newly discovered cells can be reported to the gNB using an L1 measurement report.

Observation 2	LTM candidate cell addition and release procedure could be triggered based on event based L1 measurement reports, therefore enhancing both signaling and latency.

We have the following proposals:

Proposal 1	RAN2 explores the possibility of using event based L1 measurements for the purpose of triggering LTM candidate cell addition and release.

Proposal 2	RAN2 discusses the possibility of avoiding configuring redundant measurements for the same purpose i.e LTM candidate cell addition and release.

# References

RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024

RAN2 Chairman Notes R2_125bis_ChairNotes

RAN2 Chairman Notes R2_126_ChairNotes