3GPP TSG-RAN WG2 Meeting #R2-127bis		R2-2408872 Hefei, China, Oct 14-18, 2024

Agenda Item:	8.3.2

Source: 	Rakuten

Title:  	AI-ML based Inter-frequency measurement prediction

Document for:	Discussion, Decision

# Introduction

The Rel 19 SI description for “AI/ML for Mobility” includes the following objective

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2]

The above highlighted objective includes inter-frequency RRM measurement prediction. In this contribution, we discuss some aspects associated with prediction of inter-frequency RRM measurements. We also propose a way forward for this issue.

# Discussion

## Inter-frequency measurements

Inter-frequency measurements are configured to be performed by a UE when the current serving frequency coverage is about to end or not continuous, for example when there are no neighboring cells belonging to the current serving frequency or are reported with very poor coverage.

A UE needs to be configured with measurement gaps for scanning an inter-frequency cell, UE has to tune its receiver to the new frequency and during that time the UE will not be able to perform any data transmission on serving frequency at all. Hence, inter-frequency measurements are expensive in nature and configured carefully.

Inter-frequency measurements need measurement gaps and are not possible to be measured without pausing the ongoing data transmission at the UE.

## RRM measurement prediction

One of the objectives of RRM measurement prediction is to reduce measurement efforts and signaling over the air interface while retaining the handover performance i.e no handover KPI degradation.

- If the RRM measurement prediction is considered in time domain, it basically means that in a few time instances UE will perform measurements and, in a few others, UE just skip the measurement i.e measurements are performed only in a subset of time instances. The full L3 cell level measurements are predicted by the AI/ML model.

- If it is done in spatial domain, it basically means UE will perform measurements of a subset of reference signals configured in the RRC message. Further, similar to time domain model, the full L3 cell level measurements are predicted by the AI/ML model.

- If it is done in frequency domain, the L3 cell level measurements of the target frequency in all time instances needs to be predicted so that configuring measurement gaps could be avoided for measuring the target frequency.

The primary objective of predicting inter-frequency measurements is to reduce configuring measurement gaps at the UE.

## Inter-frequency measurement prediction

Intra-frequency measurements are always configured for UEs undergoing mobility. Therefore, we think Inter-frequency measurements can be predicted using the actual intra-frequency measurements or predicted intra-frequency measurements of the serving or target cells. In some cases, for example whenever there is a full overlap between serving frequency cell and target frequency cell, prediction is slightly simpler and in some other cases where there is partial overlap, its slightly tricky and probably needs additional assistance information like UE positioning information, neighbour cell topography etc

When inter-frequency measurements predictions have to be performed without intra-frequency measurements, it may be inevitable to configure measurement gap to perform measurements on one target frequency. However, in an overlapping coverage, such an inter-frequency measurement of one target frequency could be used as an input to predict inter-frequency measurements on other target frequencies. In this case, UE location information maybe useful as assistance information to determine overlapping coverage.

Actual Intra-frequency measurements or predicted Intra-frequency measurements could be used to predict inter-frequency measurements.

In an overlapping coverage, Inter-frequency measurements on one target frequency could be used to predict inter-frequency measurements on other target frequencies.

Inter-frequency measurement prediction can be performed using both UE-side and network-side AI/ML models.

RAN2 discusses and agrees to predict inter-frequency measurements using intra-frequency measurements.

RAN2 discusses and agrees the assistance information required by the AI/ML models for inter-frequency measurement prediction.

# Conclusion

We have the following observations:

Observation 1	Inter-frequency measurements need measurement gaps and are not possible to be measured without pausing the ongoing data transmission at the UE.

Observation 2	The primary objective of predicting inter-frequency measurements is to reduce configuring measurement gaps at the UE.

Observation 3	Actual Intra-frequency measurements or predicted Intra-frequency measurements could be used to predict inter-frequency measurements.

Observation 4	In an overlapping coverage, Inter-frequency measurements on one target frequency could be used to predict inter-frequency measurements on other target frequencies.

Observation 5	Inter-frequency measurement prediction can be performed using both UE-side and network-side AI/ML models.

We have the following proposals:

Proposal 1	RAN2 discusses and agrees to predict inter-frequency measurements using intra-frequency measurements.

RAN2 discusses and agrees the assistance information required by the AI/ML models for inter-frequency measurement prediction.

# References

Study Item Description: RP-240082 : AI/ML for Mobility;