3GPP TSG-RAN WG2 Meeting #R2-127bis		R2-2408873 Hefei, China, Oct 14-18, 2024

Agenda Item:	8.10.2

Source: 	Rakuten

Title:  	Configuring UE based TA acquisition for LTM

Document for:	Discussion, Decision

# Introduction

The Rel 19 WI description for “Data collection for SON/MDT in NR standalone and MR-DC Phase 4” includes the following objective

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

The above highlighted objective includes enhancements related to the information of RACH-less access from UE, related to MRO for LTM.

In this contribution, we discuss some aspects associated with configuring UE-capability based Timing Advance (TA) acquisition during LTM, which is a very important aspect in ensuring a successful RACH-less LTM. We also propose a way forward for this issue.

# Discussion

## Enabling UE-capability based TA acquisition

In Rel 18 LTM, RAN2 has agreed two mechanisms for a UE to acquire TA from an LTM candidate cell. The first is by using a PDCCH ordered RACH performed by the UE with the candidate cell, where the candidate cell TA is forwarded to the serving gNB-DU via the gNB-CU. The second mechanism is by configuring the UE with UE-based TA acquisition where the UE derives the candidate cell TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell.

The UE-capability based TA acquisition is an alternative way of acquiring the candidate cell TA, but this has the following characteristics

    - This is based on UE capability and the UE is configured by the gNB-CU using RRC signaling.

    - The TA computation may not be accurate always and hence this method is not always reliable.

    - This doesn’t require UE to perform RACH towards candidate cell to acquire TA.

Since the TA computation based on UE capability is not always accurate and unreliable, the gNB needs to make an informed decision based on the accuracy of the TA computation. UE-based TA measurement may not be reliable, but RACH based TA acquisition is always reliable. Hence, the gNB should be well equipped to determine when to enable UE-based TA measurement and when to disable it.

The UE-based TA measurement computation accuracy could be specific to any two pair of cells. For some cells, the Rx timing difference between the current serving cell and candidate cell may be accurately computed and for some other cells, it may be influenced by other external factors in the cell.

To determine the TA computation accuracy, both mechanisms could be employed to acquire TA, on UEs which are capable of UE-based TA acquisition. For example, an LTM UE which is capable of UE-based TA acquisition is configured to compute the TA of a candidate cell and also simultaneously asked to perform PDCCH ordered RACH to acquire candidate cell TA.

The UEs who have been configured with UE-based TA acquisition could store the TA values corresponding to the candidate cell IDs and report them to the gNB. The gNB could verify the UE computed TA values against the TA values acquired using RACH to determine if the UE-based TA acquisition is accurate between any two pair of cells. This UE-based TA acquisition accuracy indicator could be stored in each cell for all its neighboring cells

Depending on the accuracy indicator determined over a period of time, the gNB could start configuring UEs with UE-based TA acquisition or fallback to PDCCH ordered RACH based TA acquisition.

The UE-based TA measurement computation accuracy could be specific to any two pair of cells i.e the accuracy of UE-based TA acquisition could be different with different candidate cells.

UEs capable of UE-based TA acquisition can be configured to report the TA values computed using the UE-based TA acquisition mechanism and the corresponding serving and candidate cell ID to the gNB.

UE-based TA acquisition accuracy indicator could be computed at the gNB by comparing the TA values derived based on UE-based TA acquisition and RACH based TA acquisition over a period of time.

UE-based TA acquisition accuracy indicator is used to determine which TA acquisition mechanism is used for TA acquisition for a given LTM candidate cell.

# Conclusion

We have the following observations:

Observation 1	The UE-based TA measurement computation accuracy could be specific to any two pair of cells i.e the accuracy of UE-based TA acquisition could be different with different candidate cells.

Observation 2	UE-based TA acquisition accuracy indicator could be computed at the gNB by comparing the TA values derived based on UE-based TA acquisition and RACH based TA acquisition over a period of time.

We have the following proposals:

Proposal 1	UEs capable of UE-based TA acquisition can be configured to report the TA values computed using the UE-based TA acquisition mechanism and the corresponding serving and candidate cell ID to the gNB.

Proposal 2	UE-based TA acquisition accuracy indicator is used to determine which TA acquisition mechanism is used for TA acquisition for a given LTM candidate cell.

# References

RP-234038 : New WID on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

RAN2 Chairman Notes R2_125bis_ChairNotes