3GPP TSG-RAN WG2 Meeting #R2-127bis		R2-2408875 Hefei, China, Oct 14-18, 2024

Agenda Item:	7.4.3

Source: 	Rakuten

Title:  	Coexistence of UE C-DRX and RACH-less LTM

Document for:	Discussion and decision

# Introduction

In 38.321, Sec 5.7, we have the below. When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

in 5.18.35:, i.e. after applying the TA The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> else if the Timing Advance measurement is configured as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the indicated LTM target: 4> process the measured Timing Advance (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing. 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

The interworking between LTM and UE C-DRX was agreed and specified as above. In this contribution we address an issue associated with the timing of when a RACH-less LTM cell switch is considered to be “ongoing”.

# Discussion

The impact of UE C-DRX on LTM could occur either at serving cell or at target cell. Rel 18 LTM specified the interworking between C-DRX and LTM and that was mainly focussed on the target cell behavior. In this paper, we analyse the impact of UE C-DRX on LTM in a serving cell and identify a potential gap in the R18 framework.

## LTM cell switch during UE C-DRX at serving cell

Based on the above snippet from 38.321, RACH-less LTM cell switch is considered to be ongoing after the UE receives LTM Cell Switch Command MAC CE from a serving cell.

This entails that,

- If C-DRX is configured for a UE, the gNB-DU will have to send the PDCCH order to acquire candidate cell TA and LTM cell switch command MAC CE in DL only during UE’s active duration, as RACH-less LTM is not considered to be “ongoing” when these are being peformed.

- After the UE receives an LTM cell switch command MAC CE, the RACH-less LTM cell switch is considered to be ongoing.

- After the reception of cell switch command, there is an ongoing RACH-less LTM cell switch and hence UE ignores DRX cycle and all communication with the target gNB-DU cell is smooth as there is no restriction of active duration.

However, after the UE receives the LTM cell switch command MAC CE, there is no further communication with the source cell. Hence, RACH-less LTM cell switch being considered to be ongoing is not meaningful for the source cell.

Ideally, the source gNB-DU should be able to send the LTM cell switch command using the DL MAC CE to the UE without waiting for the UE’s active duration. Therefore, it is preferable that the definition of RACH-less LTM cell switch “ongoing” should be at least before the LTM cell switch command is delivered to the UE. Since there is no predefined fixed event before this, the best match is when the serving gNB-DU decides based on the reported L1 measurements that the UE has to perform TA acquisition of a given candidate cell.  Hence, RACH-less LTM cell switch “ongoing” could begin with the reception of PDCCH order, when the UE is asked to perform RACH to acquire candidate cell TA.

The UE could return to DRX cycle, if it did not receive RACH-less LTM cell switch command within a predefined time which could be larger than the TA timer.

RACH-less LTM cell switch considered as “ongoing” is not meaningful from the source cell point of view as it does not begin before the LTM cell switch command is delivered by the serving gNB-DU.

Hence, we propose that the RACH-less LTM cell switch is considered to be “ongoing” when the UE receives a PDCCH order from the serving gNB-DU, to ensure that the LTM cell switch command can be delivered without any wait time.

RACH-less LTM cell switch is considered to be “ongoing” when the UE receives a PDCCH order from the serving gNB-DU.

UE shall return to C-DRX cycle if the RACH-less LTM cell switch command is not received within a predefined time.

# Conclusion

We have the following observations:

Observation 1	RACH-less LTM cell switch considered as “ongoing” is not meaningful from the source cell point of view as it does not begin before the LTM cell switch command is delivered by the serving gNB-DU.

We have the following proposals:

Proposal 1	RACH-less LTM cell switch is considered to be “ongoing” when the UE receives a PDCCH order from the serving gNB-DU.

Proposal 2	UE shall return to C-DRX cycle if the RACH-less LTM cell switch command is not received within a predefined time.

# Reference

RP-233930 : Introduction of NR further mobility enhancements in TS 38.321

3GPP TS 38.321 : MAC Protocol Specification v 18.2.0