3GPP TSG-RAN WG2 #127bis	R2-2408880

Hefei, China, 14th – 18th October 2024

Agenda Item:	7.9.4

Source:	Ericsson

Title:	PC5/Uu Relay RLC Channel Definition

Document for:	Discussion, Decision

# 1 	Introduction

The following issue was postponed from the last meeting regarding the addition of a NOTE for the SDUs received over the N3C link.

R2-2407751	RLC correction for multi-path relay with N3C	Xiaomi	CR	Rel-18	38.322	18.1.0	nnnn	-	F	NR_SL_relay_enh-Core

- Postponed

Discussion:

Xiaomi indicate there is still some discussion on the normative text.

Apple think there are issues with the existing NOTEs and the description in the new note does not cover all cases.

Huawei think we should not open the other two NOTEs now and the CR is correct.

OPPO have some sympathy with Apple’s concern regarding whether the RLC channel can be referred to as a relay RLC channel.  They would like some more time to check.

Nokia think the N3C part needs to be added as normative text and we could postpone.

Apple would like to discuss with the spec rapporteur and would be OK to postpone to next meeting.

Xiaomi agree it would be better to postpone.

Ericsson think some clarification is needed about exactly what we are trying to capture in the NOTE; they think the mapping to RLC channels is already clear and we should be careful what is captured in stage 3.

Huawei think the RLC spec today does not refer clearly to the behaviour with the N3C layer and the definition/clarification is needed.  Ericsson understand why something is needed in the PDCP spec where there are procedural changes for N3C, but here there is a one-to-one mapping of channels and they do not see ambiguity, especially considering that N3C is outside 3GPP scope.

# 2 	Discussion

The original issue was related to the addition of a NOTE relating to the N3C link as follows:

NOTE 3:	In case the RLC entity transmits or receives RLC SDU from N3C link as defined in TS 38.300 [2], an RLC channel refers to a Uu Relay RLC channel.

However, this opened the discussion to a misalignment in the definition of the RLC channel and the PC5/Uu RLC Relay RLC channel and companies are also questioning the need for the following NOTE:

NOTE 2:	In case the upper layer is SRAP as defined in TS 38.351 [9], an RLC channel refers to either a PC5 Relay RLC channel or a Uu Relay RLC channel.

Thus, there are two issues to address:

- Definition misalignment between RLC channel and PC5/Uu Relay RLC channel (due to NOTE 2)

- Capturing the N3C-link wrt the Uu Relay RLC Channel (as in NOTE 3)

### 2.1 	Definition Misalignment between RLC channel and PC5/Uu Relay RLC Channel

The current definitions for the PC5 and Uu Relay RLC channel are as follows:

Uu Relay RLC channel: an RLC channel between L2 U2N Relay UE or MP Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network Relay or for indirect path in case of MP.

PC5 Relay RLC channel: an RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, or between L2 U2U Remote UE and L2 U2U Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network/UE-to-UE Relay.

The user plane protocol stack is as shown below:

Figure 1: User Plane Protocol Stack for L2 UE-to-Network Relay

The following are the different interpretations for the RLC channel for different scenarios:

- For both U2N relaying and non-relaying cases, the RLC channel is a logical link between the upper layers and RLC.

- For the U2N relaying case, the PC5/Uu Relay RLC channel is the logical link between the L2 U2N Relay UE and L2 U2N Remote UE/gNB.

- For non-relaying cases, the RLC channel is either a logical link between the UE and gNB over Uu and or between two UEs in PC5.

Based on the above interpretation, companies claim that NOTE2 as shown above is incorrect owing to that the SRAP layer packets cannot map directly to the PC5/Uu Relay RLC channel, the logical interface between the U2N Relay UE and U2N Remote UE/gNB.

With the above interpretation, the SRAP layer packets cannot be directly mapped to the PC5/Uu Relay RLC channel, the logical interface between the U2N Relay UE and U2N Remote UE/gNB.

But considering that the above definitions are modelled on the BH RLC channels as specified for IAB with the protocol stack as shown below:

Figure 2: Protocol Stack for the support of F1-U protocol

Where the definition of the BH RLC channel is as shown below:

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

In addition, the following is the DL L2-structure for the user plane at the IAB-donor.

Figure 3: DL L2-structure for the user plane at the IAB-donor

From the definition of the BH RLC channel, an IAB-node or IAB-donor does not differentiate between the two logical links (i.e., between the nodes and between BAP/RLC) for an RLC channel, both are termed BH RLC channels.

In IAB, there is no differentiation for an RLC channel between the logical links i.e., between the IAB nodes and between BAP/RLC, both are termed BH RLC channel.

If we apply the same logic to the PC5/Uu Relay RLC channel, we should also be able to refer to the RLC channel as the PC5/Uu Relay RLC channel in the context of U2N relaying. The only difference being that the IAB node is a special device as opposed to the U2N Relay UE but the notion of both the logical links being the same is still valid.

From a specification point of view, UE-to-Network relaying is modelled on IAB, thus the same logic of no differentiation between logical channels should also be applied.

As a result, the NOTE2 as shown above can be retained with the understanding that in the context of U2N relaying, there is no difference between the logical Uu/PC5 RLC and RLC channels.

For UE-to-Network Relays, need not differentiate between an RLC channel and PC5/Uu RLC channel.

However, the NOTE2 as shown above, is also not fully correct. As it seems to suggest that the upper layer is the SRAP layer, but upper layers only refer to the PDCP/SDAP layers. In U2N relaying, the data from the upper layers is mapped to the SRAP layer which in turn is mapped to the RLC layer. Thus, NOTE2 should be updated to say ‘If SRAP is configured’.

Update the NOTE as follows “If SRAP as defined in TS 38.351 [9] is configured as in TS 38.331 [5], an RLC channel refers to either a PC5 Relay RLC channel or a Uu Relay RLC channel”. Adopt the TP in the Annex.

### 2.2 	Capturing the N3C-link wrt the Uu Relay RLC Channel

NOTE 3:	In case the RLC entity transmits or receives RLC SDU from N3C link as defined in TS 38.300 [2], an RLC channel refers to a Uu Relay RLC channel.

For the above NOTE to capture the N3C link in the 38.322 specification, from a network configuration perspective, this is incorrect.

- Firstly, because the scope of the N3C-link is outside 3GPP and hence there is no configuration that can be provided for this link i.e., the configuration at the U2N Remote UE to setup the DRB, only consists of the n3c-BearerAssociated identifier (Boolean).

- Second, the configuration for the U2N Relay UE is based on the N3C-MappingConfig which in turn consists of the SL-RemoteUE-RB-Identity-r17. As a result, from the network point of view and from a configuration perspective, there is a direct mapping of the SL-Remote UE radio bearer to the Uu-Relay RLC channel.

From a configuration perspective, the N3C link is not configured by the network. The network only configures the mapping of the MP Remote UE’s radio bearer identity to the N3C Relay’s Uu-RLC channel.

Based on this observation, in the protocol stack for the L2 multi-path relay using N3C, the following is the view from the network configuration viewpoint:

Figure 4: Protocol Stack from the network’s configuration PoV

Thus, the modelling as above should be confirmed.

In MP relays with an N3C link, from a network configuration point of view, the MP Remote UE’s upper layer maps to the N3C Relay’s Uu Relay RLC channel.

With the understanding of P3 and that N3C is not a 3GPP recognized link/layer, there is no need for further clarification in the RLC specification and NOTE is not required.

No need for a NOTE capturing the N3C behaviour in the 38.322 specification.

# 3 	Conclusion

Based on the discussion in the previous sections we have the following observations:

Observation 1	With the above interpretation, the SRAP layer packets cannot be directly mapped to the PC5/Uu Relay RLC channel, the logical interface between the U2N Relay UE and U2N Remote UE/gNB.

Observation 2	In IAB, there is no differentiation for an RLC channel between the logical links i.e., between the IAB nodes and between BAP/RLC, both are termed BH RLC channel.

Observation 3	From a specification point of view, UE-to-Network relaying is modelled on IAB, thus the same logic of no differentiation between logical channels should also be applied.

Observation 4	From a configuration perspective, the N3C link is not configured by the network. The network only configures the mapping of the MP Remote UE’s radio bearer identity to the N3C Relay’s Uu-RLC channel.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE-to-Network Relays, need not differentiate between an RLC channel and PC5/Uu RLC channel.

Proposal 2	Update the NOTE as follows “If SRAP as defined in TS 38.351 [9] is configured as in TS 38.331 [5], an RLC channel refers to either a PC5 Relay RLC channel or a Uu Relay RLC channel”. Adopt the TP in the Annex.

Proposal 3	In MP relays with an N3C link, from a network configuration point of view, the MP Remote UE’s upper layer maps to the N3C Relay’s Uu Relay RLC channel.

Proposal 4	No need for a NOTE capturing the N3C behaviour in the 38.322 specification.

# 4	TP – 38.322

START OF CHANGES

## 4.2	RLC architecture

### 4.2.1	RLC entities

The description in this clause is a model and does not specify or restrict implementations.

RRC is generally in control of the RLC configuration.

Functions of the RLC sub layer are performed by RLC entities. For an RLC entity configured at the gNB, there is a peer RLC entity configured at the UE and vice versa. In NR sidelink communication, in NR sidelink discovery, for an RLC entity configured at the transmitting UE, there is a peer RLC entity configured at each receiving UE.

An RLC entity receives/delivers RLC SDUs from/to upper layer and sends/receives RLC PDUs to/from its peer RLC entity via lower layers.

An RLC PDU can either be an RLC data PDU or an RLC control PDU. If an RLC entity receives RLC SDUs from upper layer, it receives them through a single RLC channel between RLC and upper layer, and after forming RLC data PDUs from the received RLC SDUs, the RLC entity submits the RLC data PDUs to lower layer through a single logical channel. If an RLC entity receives RLC data PDUs from lower layer, it receives them through a single logical channel, and after forming RLC SDUs from the received RLC data PDUs, the RLC entity delivers the RLC SDUs to upper layer through a single RLC channel between RLC and upper layer. If an RLC entity submits/receives RLC control PDUs to/from lower layer, it submits/receives them through the same logical channel it submits/receives the RLC data PDUs through.

NOTE 1:	In case the upper layer is BAP as defined in TS 38.340 [7], an RLC channel refers to a Backhaul RLC channel.

NOTE 2:	If SRAP as defined in TS 38.351 [9] is configured as in TS 38.331 [5], an RLC channel refers to either a PC5 Relay RLC channel or a Uu Relay RLC channel.

An RLC entity can be configured to perform data transfer in one of the following three modes: Transparent Mode (TM), Unacknowledged Mode (UM) or Acknowledged Mode (AM). Consequently, an RLC entity is categorized as a TM RLC entity, an UM RLC entity or an AM RLC entity depending on the mode of data transfer that the RLC entity is configured to provide.

A TM RLC entity is configured either as a transmitting TM RLC entity or a receiving TM RLC entity. The transmitting TM RLC entity receives RLC SDUs from upper layer and sends RLC PDUs to its peer receiving TM RLC entity via lower layers. The receiving TM RLC entity delivers RLC SDUs to upper layer and receives RLC PDUs from its peer transmitting TM RLC entity via lower layers.

An UM RLC entity is configured either as a transmitting UM RLC entity or a receiving UM RLC entity. The transmitting UM RLC entity receives RLC SDUs from upper layer and sends RLC PDUs to its peer receiving UM RLC entity via lower layers. The receiving UM RLC entity delivers RLC SDUs to upper layer and receives RLC PDUs from its peer transmitting UM RLC entity via lower layers.

An AM RLC entity consists of a transmitting side and a receiving side. The transmitting side of an AM RLC entity receives RLC SDUs from upper layer and sends RLC PDUs to its peer AM RLC entity via lower layers. The receiving side of an AM RLC entity delivers RLC SDUs to upper layer and receives RLC PDUs from its peer AM RLC entity via lower layers.

Figure 4.2.1-1 illustrates the overview model of the RLC sub layer.

Figure 4.2.1-1: Overview model of the RLC sub layer

RLC SDUs of variable sizes which are byte aligned (i.e. multiple of 8 bits) are supported for all RLC entity types (i.e. TM, UM and AM RLC entity).

Each RLC SDU is used to construct an RLC PDU without waiting for notification from the lower layer (i.e., by MAC) of a transmission opportunity. In the case of UM and AM RLC entities, an RLC SDU may be segmented and transported using two or more RLC PDUs based on the notification(s) from the lower layer.

RLC PDUs are submitted to lower layer only when a transmission opportunity has been notified by lower layer (i.e. by MAC).

NOTE 3:	The UE should aim to prevent excessive non-consecutive RLC PDUs in a MAC PDU when the UE is requested to generate more than one MAC PDU.

Description of different RLC entity types are provided below.

END OF CHANGES