3GPP TSG-RAN WG2 Meeting #127bis	R2-2408884

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.7.6

Source:	Ericsson

Title:	On XR Rate Control

Document for:	Discussion, Decision

# 1	Introduction

The XR for NR Phase 3 WID was revised in RAN#105 with the inclusion of the following objective on XR rate control:

- Specify uplink congestion signalling [RAN2]:

- Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion

In this contribution, we discuss the MAC layer based XR rate control signaling and provide our views.

# 2	Discussion

As mentioned in the introduction, a new objective was added in RAN#105 to specify uplink congestion signalling from the gNB to the UE to proactively trigger UL congestion control mechanisms i.e., faster source rate adaptation to counter uplink congestion. Further, the objective also mentions specifying the control signalling in the MAC layer over downlink on a per QoS/per DRB level.

We discuss two aspects of this objective in the following, first, about the MAC-layer based control signaling and second, the source rate adaptability at the UE.

## 2.1 	MAC-Layer XR Rate Control Signalling

### 2.1.1	Flow-level vs DRB-level Granularity

Based on an earlier discussion, RAN2 deemed it feasible for RAN to estimate the congestion information on a per-QoS flow and per-DRB in downlink and uplink directions without UE impacts. However, given that RAN plenary decided the XR rate control signalling is in the MAC layer, only a per-DRB level granularity is feasible.

- With MAC-layer XR rate control signaling, only a per-DRB level granularity is feasible.

In the current NR protocol stack, the MAC-layer is not aware/exposed to QoS flow information and would require cross-layer interaction with the upper-layers to obtain such information. Such cross-layer interactions to enable exposure of the QoS flow information is cumbersome at the least and would result in significant delays to trigger the DL XR rate control signaling. The delay exacerbated with an F1-interface in a CU-DU split scenario and not to mention the specification impact in RAN3.

- Cross-layer interactions to expose the QoS information at the MAC-layer is cumbersome, results in significant delays to trigger the XR rate control signaling and has large specification impact.

Given that UL congestion is detected in the scheduler (MAC-layer) e.g., due to lack of scheduling resources for the UEs. Keeping the per-DRB level granularity enables the MAC-layer based control signaling to be generated locally and transmitted with minimal delays. Hence, the granularity should be on the per-DRB level.

Support DRB-level granularity for the MAC layer XR rate control signaling.

Furthermore, with a DRB-level granularity, the UE has the flexibility to choose any QoS flow mapped to the DRB for source rate adaptation i.e., up to UE implementation which QoS flow should be source rate adapted.

Up to UE implementation which QoS flow is chosen for source rate adaptation.

### 2.1.2	XR Rate Control Signaling

We believe it is sufficient to reuse the recommended bit rate MAC CE. Although a new MAC CE can be introduced, it would be redundant as all the necessary information is already in the recommended bit rate MAC CE indicating the LCID with a DRB-level granularity and corresponding bit rates for UL/DL.

Reuse the recommended bit rate MAC CE for XR rate control signaling.

Looking at the values in the bit rate field for the recommended bit rate MAC CE, the minimum value (without the bit rate multiplier) is 0.009 Mbps and the maximum value is 8 Mbps. Based on the XR SI assumptions for video traffic [1], the peak data rates are 10 and 20 Mbps for UL. The current set of values does cover the entire range of required UL data rates but not with the required granularity even when with the current set of multiplier values.

- Current set of recommended bit rates and multiplier values are not well suited for the UL data rates for XR.

For example, the current possible multiplier values are x40, x70, x100 and x200. For XR services (i.e. those with peak data rates of up to 10 and 20 Mbps, the network would have to settled for the x40 multiplier. Meaning that the range would be: 0.009 Mbps × 40 = 360 kbps to 320 Mbps. This range is not suitable for the targeted XR services. To get a range up to 20 Mbps we need a multiplier of 20÷8 = 2.5.

Another approach to cover the whole range (with suitable granularity) is to add more recommended bitrate values. Luckily there are seven spare code points in the table. RAN2 could consider saving one of those code points for future use, but six of them can be used to add values ranging up to 20 Mbps, e.g. 10, 12, 14, 16, 18 and 20 Mbps could be added.

Both these approaches work, and we are fine either way, but we believe adding new multiplier values is simpler and future proof.

Add additional recommended bitrate MAC CE multiplier values to cover the range and granularity of UL data rates for XR.

## 2.2 	Source Rate Adaptation for UL Traffic

For UL traffic, only the UE is aware of the traffic characteristics for e.g., periodicity, jitter, and arrival times. In addition, only the UE is aware if the source rate of a certain service/QoS flow can be adapted. For example, certain services may easily be able to adapt to different bit rates, e.g. services with adaptable codecs. However, some other services may be more binary in the sense that the required bitrate is a hard requirement and going below that means that the service can stop and not provide the required Quality of Experience (QoE). This information is not known to the network, but the UE can learn this information.

- For UL traffic, only the UE is aware if a QoS flow can adapt to a reduced bit rate.

Without this information, the gNB would have to do a trial-and-error approach before finding UEs which can adapt their bitrates. The gNB would have to pick some UEs, ask them to reduce, wait and see if they reduce, if they don’t, try with other UEs, and so on. This trial-and-error approach takes time and hence it takes longer before the congestion can be fixed.

- A trail-and-error approach in finding the appropriate set of UEs for source rate adaption leads to delays in fixing UL congestion.

Making this information available at the gNB would be beneficial and enables choosing the appropriate UEs to perform XR rate control signaling i.e., gNB can transmit the XR rate control signaling only to UE(s) with one or more rate adaptable QoS flows mapped to a DRB, if/when it detects congestion.

UEs can provide information on source rate adaptability of QoS flow(s) to the gNB.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	With MAC-layer XR rate control signaling, only a per-DRB level granularity is feasible.

Observation 2	Cross-layer interactions to expose the QoS information at the MAC-layer is cumbersome, results in significant delays to trigger the XR rate control signaling and has large specification impact.

Observation 3	Current set of recommended bit rates and multiplier values are not well suited for the UL data rates for XR.

Observation 4	For UL traffic, only the UE is aware if a QoS flow can adapt to a reduced bit rate.

Observation 5	A trail-and-error approach in finding the appropriate set of UEs for source rate adaption leads to delays in fixing UL congestion.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Support DRB-level granularity for the MAC layer XR rate control signaling.

Proposal 2	Up to UE implementation which QoS flow is chosen for source rate adaptation.

Proposal 3	Reuse the recommended bit rate MAC CE for XR rate control signaling.

Proposal 4	Add additional recommended bitrate MAC CE multiplier values to cover the range and granularity of UL data rates for XR.

Proposal 5	UEs can provide information on source rate adaptability of QoS flow(s) to the gNB.

# 4	References

[1]	TR 38.838, Study on XR (Extended Reality) evaluations for NR, V17.0.0, 2022-04