3GPP TSG-RAN WG2 Meeting #127bis                                    	R2-2408893

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.8.5

Source:	Qualcomm Incorporated

Title:	Discussion on regenerative payload

Document for:	Discussion and Decision

# Introduction

Following agreements were made in the RAN2#126 meeting.

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

In RAN2#127, following agreements were made.

Agreements:

1.	RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

2.	Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

3.	RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

In this document, we further discuss the left-over issue.

# Discussion

RRC Reestablishment and RRC_INACTIVE state

It was discussed whether any optimization needed for RRC reestablishment procedure or for the support of RRC_INACTIVE state in case of gNB onboard satellite, as source gNB will disappear and target may not have retrieved UE’s context. Since satellite gNB’s path can be predicated and network can decide to transfer UE’s context at the right time. We think how target can retrieve UE’s context and be prepared beforehand can be discuss in RAN3.

But specific to RAN2, we do not foresee any specific impact to UE. As legacy procedure, the target gNB can send RRC setup message if the target does not have UE’s context. RAN2 can revisit the issue if there is any impact foreseen due to RAN3 work on this matter.

From RAN2 perspective, the RRC reestablishment and RRC_INACTIVE state works without any optimization when gNB is on board satellite. RAN2 can revisit the issue if there is any impact foreseen due to RAN3 work on this matter.

Early contention resolution

Figure 1 RA-SDT procedure in regenerative payload architecture

As shown in figure 1, the gNB on-board may want to immediately send the early contention resolution MAC CE (i.e., without RRC message) while waiting for the response from the ground network. The purpose of doing this is to avoid the possibility of expiry of contention resolution timer while the UE is waiting for Msg4. In this case, it certainly is not power efficient for UE to continuously monitor PDCCH unnecessarily after sending the HARQ feedback of the contention resolution MAC CE (i.e., after UE is able to resolve the contention successfully). Similar to delaying the start contention resolution timer, the PDCCH for monitoring further RRC message can be delayed after sending HARQ feedback of the contention resolution MAC CE.

Even if there is retransmission of contention resolution MAC CE, the UE does not need to receive it as it already decoded it successfully and ACK of it is just sent. Then the next task is that gNB knows about it (i.e., UE successfully resolved the contention) and gNB sends the further DL data to UE, which would take one RTT time.

After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

Support of satellite switch with resync

In RAN2#126, following was captured.

Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary

Figure 2 Mirroring of the satellites

As shown in figure 2, it could be possible by network implementation given ideal ISL backhaul. In this case, the old satellite will transfer all data (including gNB context, all UE’s context etc.) to the new satellite serving the same area, i.e., new satellite is the mirror of the old satellite. This can be done by proprietary ISL transport layer. This may take some data transfer time same as in HARD satellite switch with resync. Gateway can handle the satellite routing to new mirror satellite by a proprietary method. No impact to RAN or core network is foreseen due to this unless otherwise RAN3 identifies any work.

However, there is also debate whether switching satellites requires security keys update even though gNB node is to be remained same. This may need to be checked with SA3. If security keys update is needed, then it may not be beneficial to support this feature. There was also no conclusion in RAN plenary. There it can be clarified that in Rel-19, satellite switch with resync over regenerative payload architecture is not supported.

Clarify that satellite switch with resync over regenerative payload architecture is not supported in Rel-19.

# Conclusion

Following proposals are made.

Proposal 1	From RAN2 perspective, the RRC reestablishment and RRC_INACTIVE state works without any optimization when gNB is on board satellite. RAN2 can revisit the issue if there is any impact foreseen due to RAN3 work on this matter.

Proposal 2	After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

Proposal 3	Clarify that satellite switch with resync over regenerative payload architecture is not supported in Rel-19.