3GPP TSG-RAN WG2 Meeting #127bis                                    	R2-2408897

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.9.4

Source:	Qualcomm Incorporated

Title:	Discussion on PWS in NB-IoT NTN

Document for:	Discussion and Decision

- Introduction

The Rel-19 WID has been updated and following adjective has been added in [1].

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this document, we provide details to support PWS in NB-IoT.

- Discussion

As it is clear from the WID that currently the public warning notification procedure is missing in NB-IoT both in TN and NTN. In TS 36.331, following is the clarification.

In particular, the following features are not supported in NB-IoT and corresponding procedures and messages do not apply to the UE in NB-IoT: - Connected mode mobility (Handover and measurement reporting); - Inter-RAT cell reselection or inter-RAT mobility in connected mode; - RRC_INACTIVE; - CSG; - Relay Node (RN); - Carrier Aggregation (CA); - Dual connectivity (DC); - Multi-Radio Dual Connectivity (MR-DC); - PDCP duplication; - GBR (QoS); - ACB, EAB, SSAC and ACDC; - MBMS, except for MBMS via SC-PTM in Idle mode; - Measurement logging and reporting for network performance optimisation; - Public warning systems e.g. CMAS, ETWS and PWS;

We think the procedure for PWS (which includes PTWS and CMAS) in NB-IoT can be considered same as that of eMTC UEs in CE mode B. The eMTC UEs in CE mode B, i.e., when operating in enhanced coverage, can also receive the PWS notification by receiving paging notification and then acquiring corresponding SIB10 or SIB11 or SIB12. Even though the SIB (i.e., SIB11 and SIB12) is segmented, UE can keep receiving the SIBs until the complete warning message and the warning area coordinates (if any) have not been assembled within a period of 3 hours. Therefore, objective is to introduce the signaling and procedure for reception of paging notification and NB-IoT version of SIB10, SIB11 and SIB12.

Similar signaling and procedures of ETWS/CMAS (i.e., SIB10, SIB11 and SIB12) defined for eMTC is reused for NB-IoT.

Currently, the eMTC UE supports the direct indication for ETWS/CMAS notification. This is missing for NB-IoT. For NB-IoT, the reserved bits 3 and 4 can be used for ETWS and CMAS indications.

| Table 6.6-1: Direct Indication information using P-RNTI for non-NB-IoT Bit; Direct Indication information. 1; systemInfoModification. 2; etws-Indication. 3; cmas-Indication. 4; eab-ParamModification. 5; systemInfoModification-eDRX. 6; uac-ParamModification. 6, 7, 8; Not used, and shall be ignored by UE if received.. | Table 6.7.5-1: Direct Indication information for NB-IoT Bit; Field in Direct Indication information. 1; systemInfoModification. 2; systemInfoModification-eDRX. 3, 4, 5, 6, 7, 8; Not used, and shall be ignored by UE if received. |
| --- | --- |

For eMTC, the PWS notification can also be provided via paging message and it can be discussed whether this needs to be added in paging message for NB-IoT since reading PDSCH for paging can add additional delay and resource.

For PWS notification, use bit 3 for ETWS indication and bit 4 for CMAS indication in Direct Indication information for NB-IoT. Decide whether to also add indications in paging message, i.e., in Paging-NB.

Similarly, the SIB10, SIB11 and SIB12 can also be imported to NB-IoT signaling. However, some optimized signaling should be used for NB-IoT for example, removing dummy IEs and properly using the extensions.

#### –	SystemInformationBlockType10

The IE SystemInformationBlockType10 contains an ETWS primary notification.

SystemInformationBlockType10 information element

-- ASN1START

SystemInformationBlockType10 ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningType							OCTET STRING (SIZE (2)),

dummy								OCTET STRING (SIZE (50))	OPTIONAL,		-- Need OP

...,

lateNonCriticalExtension			OCTET STRING				OPTIONAL

}

-- ASN1STOP

The example of SIB10 for NB-IoT is shown below.

#### –	SystemInformationBlockType10-NB

The IE SystemInformationBlockType10-NB contains an ETWS primary notification.

SystemInformationBlockType10 information element

-- ASN1START

SystemInformationBlockType10-NB ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningType							OCTET STRING (SIZE (2)),

lateNonCriticalExtension			OCTET STRING				OPTIONAL

...,

}

-- ASN1STOP

Introduce NB-IoT version of SIB10 for primary ETWS, SIB11 for secondary ETWS and SIB12 for CMAS.

Currently eMTC UEs and NB-IoT UEs are not required to detect the system information change or monitor paging in RRC_CONNECTED. In Rel-16, PWS notification (ce-ETWS-CMAS-RxInConn-r16) in RRC_CONNECTED was introduced but it was mainly intended for non-BL UEs operating in CE mode. In our view, the WID objective is to deliver the PWS notification to only NB-IoT UEs in RRC_IDLE.

ETWS/CMAS is not supported in RRC_CONNECTED.

In TS 36.300, following is clarified.

-	ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. For EAB, if the UE supports SIB14, when in extended DRX, it acquires SIB14 before establishing the RRC connection;

Similarly for NB-IoT it is true that the ETWS, CMAS, PWS requirement may not be met when the UE is using eDRX cycle or in PSM mode.

Confirm that PWS requirement may not be met for NB-IoT UEs using eDRX cycle or PSM mode.

In TS 36.306, currently it is specified in section 6.2 that PWS reception is optional for UE without UE capability signaling. We think this text should be applicable for NB-IoT NTN and no further specification change is expected in TS 36.306.

## 6.2 PWS features ### 6.2.1 ETWS It is optional for UE to support ETWS reception as specified in TS 36.331 [5]. ### 6.2.2 CMAS It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5]. ### 6.2.3 KPAS It is optional for UE to support KPAS reception as specified in TS 36.331 [5]. The Korean Public Alert System (KPAS) uses the same AS mechanisms as defined for CMAS. Therefore a KPAS-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE. ### 6.2.4 EU-Alert It is optional for UE to support EU-Alert reception as specified in TS 36.331 [5]. The Europearn Union Warning System EU-Alert uses the same AS mechanisms as defined for CMAS. Therefore a EU-Alert-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE.

It is optional for NB-IoT UEs to support PWS feature without UE capability signaling. Add PWS (clause 6.2) as applicable in NB-IoT NTN in TS 36.306.

In addition, SA2 may just need minor alignment work, as MME is the host of PWS message to NB-IoT RAT. For example, Table below from TS 23.401, needs update as PWS will be supported by NB-IoT RAT. Similarly, CT1 may need alignment work for PWS in NB-IoT RAT.

Table M-5: Misc.

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.8 | MBMS (3) | (3) | (3) | (3) | (3) |
| 5.10 | Multiple PDN support and PDN activation for UEs supporting Attach without PDN connectivity | Yes (1) | Both |  |  |
| 5.11 | UE capability handling | Yes | Both |  |  |
| 5.12 | Warning message delivery | No | N/A | N/A | N/A |
| 5.13 | DRX and UE-specific DRX parameter handling | Yes (2) | Both |  |  |
| 5.13a | Extended idle mode DRX | Yes | Both | Optional | Optional |
| 5.15 | RAN information management procedures | No | N/A | N/A | N/A |
| 5.16 | MME-initiated procedure on UE's CSG membership change | No | N/A | N/A | N/A |
| 5.17 | HeNB multicast packet forwarding function | No | N/A | N/A | N/A |
| NOTE 1: Multiple PDN optional. NOTE 2: UE-specific DRX not supported in NB-IoT RAT. NOTE 3: See Table M-6. |  |  |  |  |  |

Send LS to CT1 and SA2 for potential alignment work on warning message delivery in NB-IoT RAT.

- Conclusion

Following proposals are made:

Proposal 1	Similar signaling and procedures of ETWS/CMAS (i.e., SIB10, SIB11 and SIB12) defined for eMTC is reused for NB-IoT.

Proposal 2	For PWS notification, use bit 3 for ETWS indication and bit 4 for CMAS indication in Direct Indication information for NB-IoT. Decide whether to also add indications in paging message, i.e., in Paging-NB.

Proposal 3	Introduce NB-IoT version of SIB10 for primary ETWS, SIB11 for secondary ETWS and SIB12 for CMAS.

Proposal 4	ETWS/CMAS is not supported in RRC_CONNECTED.

Proposal 5	Confirm that PWS requirement may not be met for NB-IoT UEs using eDRX cycle or PSM mode.

Proposal 6	It is optional for NB-IoT UEs to support PWS feature without UE capability signaling. Add PWS (clause 6.2) as applicable in NB-IoT NTN in TS 36.306.

Proposal 7	Send LS to CT1 and SA2 for potential alignment work on warning message delivery in NB-IoT RAT.

- References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”.