3GPP TSG-RAN WG2 Meeting #127Bis	R2-2408902
Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.5.3

Source:	InterDigital

Title:	On-demand SIB1 request and reception

Document for:	Decision

# Introduction

RAN approved a new work item on network energy saving (NES) [1], which was revised as [2]. The revised WID includes the following objective where study item objective was converted to the normative phase, as in the following:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

RAN2 made the following agreements in RAN2#127 meeting [3]

Agreements on OD-SIB1

Scenarios:

1.	No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

2.	We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

3.	Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

4.	Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

5.	Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

6.	Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

7.	RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

8.	RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

RAN2 made the following agreements in RAN2#126 meeting [3]

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

This contribution discusses various aspects to support on-demand SIB1 for UEs in idle/inactive mode.

# UL-WUS configuration for On-demand SIB1

According to the WID for enhanced NES, the triggering for on-demand SIB1 will use UL WUS signal based upon existing signal/channel. One important step to enable on-demand SIB1 transmissions is the provisioning of UL WUS configuration for the UEs which are currently in idle/inactive mode.

When Cell A is providing the UL-WUS configuration for one or more NES cells, e.g., to support the scenario 1a, the Cell A may transmit UL-WUS configuration as part of the system information. Although the UL-WUS configuration can be included in one of the legacy SIBs, the overhead will scale with the number of NES cells, which can be cumbersome of existing SIBs. A cleaner design would thus be to define a new SIB where UL-WUS configuration for one or more NES cells is transmitted by the Cell A.

Proposal 1: 	Support defining a new SIB carrying UL-WUS configuration for NES cell(s), e.g., neighbor NES cells.

UL-WUS configuration will allow the UE to identify the NES cell. This is captured in the following RAN2#126 [4] agreement through inclusion of PCI and frequency as part of the UL-WUS configuration:

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Based upon the received UL-WUS configuration, the UE can transmit the UL WUS to the NES cell. Thus, at minimum, the UL-WUS configuration provides the RACH configuration for the NES cell. The RACH configuration should provide the suitable RO and preamble determination for the UE based upon the SSBs being transmitted by the NES cell. This was agreed in RAN2#125Bis [5].

In order to transmit Msg1, the UE needs to know the mapping of SSB to RO. In legacy, SSB to RO mapping is part of the RACH configuration. Thus, following the legacy behavior, the RACH configuration of the NES cell, provided through UL-WUS configuration, should include the SSB to RO mapping to request on-demand SIB1. The reception of RAR in response to the UL-WUS transmission will require that UE knows a CORESET, e.g., CORESET 0, where it can receive and decode the response. The CORESET configuration for RAR can be the CORESET 0 or a different configuration, which can be discussed in RAN2. If the network intends to transmit RAR through CORESET 0 in response to a UE’s UL-WUS transmission, the UE may acquire the CORESET 0 configuration from the SSB (e.g., same as legacy UEs get CORESET 0 configuration through SSB/MIB). Nevertheless, the network may have additional flexibility by providing a CORESET for RAR different from CORESET 0.

Proposal 2: 	RAN2 to discuss whether the UE acquires CORESET 0 configuration (i) through SSB of the NES cell, or (ii) as part of UL-WUS configuration for the NES cell.

Given RAN2 agreed that cell A can provide WUS configuration for more than one NES cell, the UL WUS configuration should support indicating the list of applicable NES cells, along with the associated RACH configuration to transmit the preamble and receive RAR. Thus, overall, we have the following proposal for UL-WUS configuration:

Proposal 3: 	UL-WUS configuration comprises of at least: applicable NES cell(s), associated RACH configuration (including resource partition, power control, SSB to RO association)

One aspect related to the UL-WUS RACH configuration is whether the RACH configuration is as per legacy or if there is a separate RACH configuration to trigger SIB1. Although RACH configuration triggering SIB1 transmissions shared with legacy RACH functionality can work, a separate RACH configuration as trigger can be more advantageous, separate from legacy UEs, and can be used for all UEs in the cell. It is not likely to need too many preambles for requesting OD-SIB1, as in legacy request for other SI. Legacy UEs are also not likely to be camped on such SIB1-less cell.

Proposal 4:	Support a dedicated RACH configuration for requesting OD-SIB1.

# RA procedure for WUS transmission

RAN2 has agreed that Msg3 is not used for OD-SIB1 request. In a legacy RA procedure initiated for SI request, the procedure also does not require Msg3 transmission and is considered successful upon reception of a RAR with the selected RAPID. A similar condition for success can be used for determining the success of an RA initiated for OD-SIB1 request.

Proposal 5:	Similar to legacy RA procedure initiated for SI request, an RA initiated for OD-SIB1 request is initiated with type 4 step RA and is considered successful upon reception of RAR.

In a legacy RA procedure initiated for SI request, the UE can be configured with ra-AssociationPeriodIndex and si-RequestPeriod. si-RequestPeriod is specifies the periodicity of the SI-Request configuration in number of association periods, i.e. the periodicity at which a UE can request SI. RAN2 can thus discuss whether such parameters need to be provided in the UL WUS configuration for OD-SIB1 request, when multiple SSBs are present.

Proposal 6:	RAN2 to discuss whether to configure si-RequestPeriod in the UL WUS configuration for OD-SIB1.

# Acquisition for On-demand SIB1

RAN2 has agreed that a UE will receive an RAR in response to its UL-WUS transmission over an OD-SIB1 NES cell. In legacy RA procedure initiated for acquisition of SI request, the RAR format is simplified to contain the RAPID only. RAN2 also agreed that a combined RA procedure for OD-SIB1 request and initial access in NES cell is not supported. It is therefore assumed that there isn’t a need for msg3 transmission post successful reception of the RAR. Therefore, many of the RAR parameters, e.g., the UL grant (with relevant parameters) and timing advance related information may not be needed as here UEs may not need to perform a Msg-3 transmissions.

The network can provide useful information in RAR related to the scheduling of OD-SIB1 transmission. E.g., the network can provide a time, or a window when the UE may expect the subsequent OD-SIB1 transmission.

Proposal 7:	The scheduling of OD-SIB1 transmission is indicated part of the RAR received after WUS transmission.

# SIB1 Validity

One signaling aspect for UL WUS transmission is related to the target cell recipient of the UL WUS signal. If the UL WUS configuration is available, the NES cell should be the target receiver for the UL WUS signal. If the UL WUS is transmitted directly to the NES cell, the NES cell may transmit SIB1 after detecting the PRACH signal. For the situations when the anchor cell is not good enough for the UEs, and these UEs are located much closer to the NES cell than an anchor cell, having the possibility of transmitting the UL WUS to the NES cell can be more beneficial.

In RAN2#117, it was discussed to have a OD-SIB1 configuration validity criterion, such that if a UE has a valid version of SIB1 stored, UE does not initiate the on-demand SIB1 request. When the UE camps on a NES cell, the network may not know when the UE acquired SIB1. For the cells not transmitting SIB1, it may be important to maintain a valid version of SIB1 at the UEs. Following the validity functionality for other SIBs, time-based validity for SIB1 can be introduced.

Proposal 8:	Time based validity can be introduced for OD-SIB1 maintenance.

In the scenario where the UE has a valid UL WUS configuration and is camped on the NES cell, if the UE needs to acquire SIB1 from the NES cell, e.g., after a certain validity period, the UE may transmit UL WUS to the NES cell.

Proposal 9:	Upon expiry of a validly time associated with OD-SIB1, if the UE is camped on the NES cell and an UL WUS configuration is available, the UE transmits UL WUS towards the NES cell once triggered.

# UL-WUS configuration validity

One design aspect with OD-SIB1 through the transmission of UL-WUS is to maintain the validity of the UL-WUS configurations. As UL-WUS configuration will be provided to the UE through SIB (e.g., new or existing SIBs), it would make sense to reuse the legacy validity behavior. Following the validity functionality for other SIBs, time-based validity for UL-WUS can be introduced.

Proposal 10:	Time based validity is introduced for UL WUS configuration maintenance.

When a UE performs cell (re-)selection to a NES cell, it may transmit UL-WUS based only upon a valid UL-WUS configuration.

Once a UE camps on a OD-SIB1 NES cell, the NES cell may still perform some level of energy savings by not transmitting SIB1 according to legacy periodicities. For example, the NES cell can transmit SIB1 with longer periodicity (sparse SIB1) or not transmit SIB1 at all. This means, the UE may still need to maintain UL-WUS configuration for the NES cell to be able to request SIB1 when needed. In an alternate scenario, the network may change/update the UL WUS configuration for the NES cell that the UE is camped on. To acquire the NES cell configuration from a different anchor cell may be challenging and even not feasible in some situations and deployments.

Furthermore, RAN2#127 made the following agreement:

WUS configuration of NES cell from NES cell:

4.	Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

Thus, the UE should be able to acquire the UL-WUS configuration from the NES cell directly when it is already camped on the NES cell. This can be achieved by the legacy SI modification procedure.

If the network modifies the UL-WUS configuration for the NES cell, the NES cell can indicate this modification through system information update message.

Proposal 11:	The network can indicate the modification of UL-WUS configuration for a NES cell through the NES cell itself by SI modification indication.

The network can also change the status of NES cell to a normal or anchor cell, e.g., based upon the channel and load conditions, then the (former) NES cell can provide the UL-WUS configurations for its neighboring NES cells, effectively becoming a cell A.

Proposal 12:	A NES cell can provide UL-WUS configuration for its neighboring NES cells, e.g., when the network starts operating NES cell in normal/anchor mode, effectively becoming a cell A. No special handling is needed for such case.

The provisioning of UL-WUS configuration for neighboring NES cells through a former NES cell can be easily implemented through the framework of UL-WUS transmission in a SIB and following the SIB modification procedure.

If a UE camped on a NES cell has its UL-WUS configuration about to expire, the UE can transmit a request on the NES cell to acquire its UL-WUS configuration through on-demand SIB acquisition request.

Proposal 13:	The UEs attempt to acquire a valid UL-WUS configuration by transmitting a request to the NES cell to receive its UL-WUS configuration if it is about to expire.

In the scenario where a UE does not have a valid UL-WUS configuration and is camped on the NES cell, the UE may attempt to re-acquire a valid UL-WUS configuration for the NES cell. If the UE has neither valid SIB1 nor valid UL-WUS configuration for the NES cell, the UE will perform cell reselection.

Proposal 14:	If a UE cannot acquire a valid UL-WUS configuration for the camped NES cell, and UE’s available SIB1 for the NES cell expires, the UE performs cell reselection.

# Handling for Legacy UEs in On-demand SIB1 based Cells

When a network operator is employing energy savings by cells where only on-demand SIB1 is being transmitted, the operator may like to block the legacy UEs accessing these cells. The legacy UEs may for example receive and decode an on-demand SIB1 transmission and try to camp on this NES cell, which is desirable to avoid. The following agreement was reached in RAN2#126:

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Further discussions in RAN2#127 led to the following agreement:

Access restriction for legacy UEs:

Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

A solution based on Option 1 will indicate on-demand SIB1 based cells as barred cells through the MIB indication. This will stop the legacy UEs from accessing the cells whether or not they receive SIB1. This will then require some understanding/indication for Rel-19 NES capable UEs to still camp on such a cell despite detecting MIB based cell barred indication.

In Option 2, the legacy UEs will be barred by the NES cell through no SIB1 indication transmitted through ssb-SubcarrierOffset in MIB. Rel-19 NES UEs will consider these cells as not barred but may then need to acquire ssb-SubcarrierOffset through the anchor cell or the NES cell itself.

In Option 3, the network may bar the legacy UEs from camping on NES cells by including such cells to excluded cells list. This may be bit of a problem if legacy UEs were not able to acquire excluded cell list and may try to acquire SIB1 of the NES cell based upon SSB and may make unintended measurements. This can get complicated if these UEs are even able to detect SIB1 of NES cell (e.g., UL-WUS transmission of a different UE).

Based upon the discussion, a MIB based cell barring for legacy UEs is preferable. The use of cell bar bit in the MIB can be the simplest option. If NES cell transmits cell barred indication in the MIB, the Rel-19 UEs may need to identify the cell as “not barred”. One possible method could be based on implicit indication, e.g., a Rel-19 UE to have a valid UL-WUS configuration for a NES cell, and the UE detecting SSB from the cell with cell barred indication. Under such conditions, the Rel-19 UE may treat the NES cell as not barred.

Another solution can be to add an explicit indication in the (on-demand) SIB1 cell being not barred for Rel-19 NES capable UEs.

Proposal 15:	Legacy UEs can be barred through legacy MIB “cell barred” indication. For Rel-19 NES capable UEs, RAN2 to consider the following options for allowing R19 NES UEs to camp on the NES cell:

  - if the UE doesn’t detect SIB1 on the NES cell and determines that the cell is on-demand SIB1 based cell, e.g. based upon a valid UL WUS configuration received from cell A for the NES cell, the UE can camp on the cell.

  - Receiving a separate explicit indication in MIB to enable camping on the NES cell from the NES cell or Cell A;

# Conclusions

The following proposals are made in this document for on demand SIB1 acquisition in IDLE/Inactive states:

Proposal 1: 	Support defining a new SIB carrying UL-WUS configuration for NES cell(s), e.g., neighbor NES cells.

Proposal 2: 	RAN2 to discuss whether the UE acquires CORESET 0 configuration (i) through SSB of the NES cell, or (ii) as part of UL-WUS configuration for the NES cell.

Proposal 3: 	UL-WUS configuration comprises of at least: applicable NES cell(s), associated RACH configuration (including resource partition, power control, SSB to RO association)

Proposal 4:	Support a dedicated RACH configuration for requesting OD-SIB1.

Proposal 5:	Similar to legacy RA procedure initiated for SI request, an RA initiated for OD-SIB1 request is initiated with type 4 step RA and is considered successful upon reception of RAR.

Proposal 6:	RAN2 to discuss whether to configure si-RequestPeriod in the UL WUS configuration for OD-SIB1.

Proposal 7:	The scheduling of OD-SIB1 transmission is indicated part of the RAR received after WUS transmission.

Proposal 8:	Time based validity can be introduced for OD-SIB1 maintenance.

Proposal 9:	Upon expiry of a validly time associated with OD-SIB1, if the UE is camped on the NES cell and an UL WUS configuration is available, the UE transmits UL WUS towards the NES cell once triggered.

Proposal 10:	Time based validity is introduced for UL WUS configuration maintenance.

Proposal 11:	The network can indicate the modification of UL-WUS configuration for a NES cell through the NES cell itself by SI modification indication.

Proposal 12:	A NES cell can provide UL-WUS configuration for its neighboring NES cells, e.g., when the network starts operating NES cell in normal/anchor mode, effectively becoming a cell A. No special handling is needed for such case.

Proposal 13:	The UEs attempt to acquire a valid UL-WUS configuration by transmitting a request to the NES cell to receive its UL-WUS configuration if it is about to expire.

Proposal 14:	If a UE cannot acquire a valid UL-WUS configuration for the camped NES cell, and UE’s available SIB1 for the NES cell expires, the UE performs cell reselection.

Proposal 15:	Legacy UEs can be barred through legacy MIB “cell barred” indication. For Rel-19 NES capable UEs, RAN2 to consider the following options for allowing R19 NES UEs to camp on the NES cell:

  - if the UE doesn’t detect SIB1 on the NES cell and determines that the cell is on-demand SIB1 based cell, e.g. based upon a valid UL WUS configuration received from cell A for the NES cell, the UE can camp on the cell.

  - Receiving a separate explicit indication in MIB to enable camping on the NES cell from the NES cell or Cell A;

# References

RP-240170, Revised WID: Enhancements of network energy savings for NR, Mar 2024

RP-242353, Revised WID: Enhancements of network energy savings for NR, September 2024

RAN2 Chairman’s note, RAN2 #127, August 2024.

RAN2 Chairman’s note, RAN2 #126, May 2024.

RAN2 Chairman’s note, RAN2 #125Bis, April 2024.

RAN1 Chairman’s note, RAN1 #116Bis, April 2024.

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023