3GPP TSG-RAN WG2 Meeting #127bis					R2-2408906

Hefei, China, Oct 14th – 18th, 2024

Source:	Continental Automotive

Title:                       Discussion on Paging aspects for Ambient-IoT

Agenda Item:         8.2.3

Document for:	Discussion and Decision

# Introduction

In the RAN2#125bis meeting, the following agreements were made for the paging [1] for ambient IoT.

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

In the RAN2#126 meeting, the following agreements were made for the paging [2] for ambient IoT.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In this document, we provide more details about paging aspects for Ambient IoT

# Discussion

Explicit information for contention-based and contention-free resources

In certain scenarios, the A-IoT device could implicitly determine whether to use contention-based resources or contention-free resources for the response. For example, an A-IoT device could use contention free resources if paging message contains information regarding a specific device ID. Thus, it would beneficial if the paging message contains explicit indication regarding whether to use contention-based or contention-free resources, especially if the paging message carries more than one device ID information and/or group ID information.

Proposal 1: Depending on the information included in the paging message, the network includes explicit information on whether the device must use contention-based or contention-free resources for the response.

Proposal 2: The device implicitly chooses contention-free resources its response when paging message carries the specific device ID information.

Paging monitoring:

An A-IoT device has low complexity and low power consumption. With this consideration and based on the agreement made in RAN2#125-bis meeting, an A-IoT device does not support the legacy paging procedure, where the device only wakes up during DRX on period to monitor paging message. Duty cycle option also increases device complexity considering A-IoT is an asynchronous system. Therefore, we propose that the device should always monitor the paging message if it has sufficient energy. If the device does not have sufficient energy to provide the response to the triggering message, it will skip the triggering response.

Proposal 3: An A-IoT device always monitors the paging message from the reader and skips the response if it doesn’t not have sufficient energy.

# Conclusions

In this contribution, we have shared our views on the paging aspects for A-IoT. Additionally, we ask RAN2 to discuss the following proposals:

Proposal 1: Depending on the information included in the paging message, the network includes explicit information on whether the device must use contention-based or contention-free resources for the response.

Proposal 2: The device implicitly chooses contention-free resources its response when paging message carries the specific device ID information.

Proposal 3: An A-IoT device always monitors the paging message from the reader and skips the response if it doesn’t not have sufficient energy.

# References

[1] RAN2#125bis meeting chair note

- [2] RAN2#126 meeting chair note