3GPP TSG-RAN WG2 Meeting #127bis					R2-2408908

Hefei, China, Oct 14th – 18th, 2024

Source:	Continental Automotive

Title:                       Discussion on RACH aspects for Ambient-IoT

Agenda Item:         8.2.4

Document for:	Discussion and Decision

# Introduction

In the RAN2#127 meeting, the following agreements were made for the RACH [1] for ambient IoT.

Agreements

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1,and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

In this document, we will discuss what type of ID should be used for scheduling purpose.

# Discussion

Currently, in 5G NR RACH procedure the gNB includes Scheduling ID (for example ASID) for scheduling purpose. However, A-IoT random access procedure is different from what we have in 5G NR RACH procedure. The following agreements were made related to the 4-step RACH procedure for A-IoT in RAN2#126 meeting [2].

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

In A-IoT, the device generates a 16-bit random ID and sends it to the reader. The reader then echoes the ID received in Msg1 back to the device in Msg2. As the random ID is already included in Msg2, it was discussed in the last meeting whether an additional scheduling ID (such as short AS ID or an upper-layer ID) is needed in Msg2. In this regard, there are two possibilities moving forward:

Option 1: The random ID itself is used for scheduling.

In this case, the random ID generated by the device in msg1 is used as an identifier for scheduling. However, there is a possibility that multiple devices generate the same random ID. For example, if both device 1 and device 2 use the same random ID X as shown in Figure 1, when the reader sends a paging message to e.g., device 1, device 2 would also respond along with device 1, leading to a collision between the two devices.

Figure 1: Collision between two devices

To prevent such collisions, additional information from the reader is required to indicate whether the device should maintain or overwrite the random ID after sending it in Msg1. If the reader knows that the random ID is already in use by another A-IoT device, the reader instructs the device to overwrite the random ID after Msg3. On the other hand, if the random ID is not in use by any other A-IoT device, the reader indicates that the device should maintain the same random ID.

As shown in Figure 2, device 2 has generated the same random ID as device 1. To resolve this, the reader assigns a new random ID to device 2 after Msg3."

Figure 2: RACH procedure with random ID is used for scheduling.

Observation 1: If the random ID generated by the device is also used as an identifier for scheduling without a separate scheduling ID, it is possible that the random ID generated by the device is already being used by another device. In this case, the device should receive a new random ID from the reader.

Option 2: The reader assigns AS ID for scheduling purpose.

In this case an A-IoT device would require more memory and power to process both the random ID and the additional ID for scheduling in Msg2. Additionally, the scheduling ID is assigned by the Reader, which is unique and specific to the device. Therefore, the Reader simply indicates the scheduling ID for the device when sending the paging message as shown in Figure 3, preventing any collisions as explained in option 1. Option 2 is simple and reduces the complexity for both the device and the reader. However, the only drawback is that the device will require an additional 16 bits of memory to store the information for the additional ID.

Figure 3: RACH procedure with Scheduling ID

Proposal 1: Additional Scheduling ID would be beneficial in A-IoT RACH procedure to reduce the device and reader complexity.

# Conclusions

In this contribution, we have shared our views on the RACH aspects for A-IoT. Additionally, we request RAN2 to discuss the following observation and proposal:

Observation 1: If the random ID generated by the device is also used as an identifier for scheduling without a separate scheduling ID, it is possible that the random ID generated by the device is already being used by another device. In this case, the device should receive a new random ID from the reader.

Proposal 1: Additional Scheduling ID would be beneficial in A-IoT RACH procedure to reduce the device and reader complexity.

# References

[1] RAN2#127 meeting chair note

- [2] RAN2#126 meeting chair note