3GPP TSG-RAN WG2 Meeting #127bis	R2-2408917

# Hefei, China, 14th – 18th October 2024

Agenda Item:	7.5.3

Source:	Ericsson

Title:	DRX SFN counter initialization

Document for:	Discussion, Decision

# 1	Introduction

RAN2#127 it was agreed:

Agreements

-	We leave this up to network implementation, unless a solution that works in all scenarios (CHO/LTM/Normal HO/Reconfiguration etc) and does not require network to send an extra RRC reconfiguration message can be agreed at next meeting.

In this contribution we present analysis of why other proposed solutions doesn’t work and therefore why it should be left to network implementation.

# 2	DRX Correction

There have been suggestions to initialize the DRX_SFN_COUNTER at various time instances to combat the problem with the unsynchronized between network and UE in different HO scenarios. However, all of the proposed solutions have shortcomings as was discussed during the offline in RAN2 #127. Simply put there will always be different issues with the timing in the various handover procedures, despite which time instance that is selected to do the initialization.

Changing the point of initalization doesnt solve the issues with potential mismatch in HO.

Furthermore, it has been proposed that the DRX SFN Counter could be added to the MAC reset to combat this issue. In this way the network could know what the DRX SFN Counter value is and could potentially adjust its own counter value. However, this then creates an issue with the DRX formula not being synchronized with the traffic arrival, which is the whole point of this feature. The DRX formula need to have both the DRX_SFN_COUNTER and the drx-StartOffset set correctly for the DRX periodicity to match the traffic arrival. If any of these values are changed on its own then the other value is not valid anymore. Thus after a reset of the counter variable the parameter values anyway need to be updated by the network through a new reconfiguration.

Despite adding DRX_SFN_COUNTER to the MAC reset a new reconfiguration is needed by the network.

There has so far been no proposed method that work in all situations and thus the only option left is to leave the DRX_SFN_COUNTER initialization to network implementation.

Leave the DRX_SFN_COUNTER initialization for network implementation.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Changing the point of initalization doesnt solve the issues with potential mismatch in HO.

Observation 2	Despite adding DRX_SFN_COUNTER to the MAC reset a new reconfiguration is needed by the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Leave the DRX_SFN_COUNTER initialization for network implementation.